3GPP TSG-RAN4 Meeting #87 	R4-1806335

# Busan, Korea, 21 – 25 May, 2018

Agenda item:	7.10.3.1

Source:	Intel Corporation

Title:	On short gap for LTE measurement

Document for:	Discussion

# Introduction

In the current gap pattern tables in TS36.133 and TS38.133 some part of the applicability didn’t take into account the UE capability of supporting short measurement gap of 3ms for LTE measurement. In this contribution we discuss how to correct the note in the current gap pattern table to clarify the UE capability on short measurement gap for LTE measurement.

# TS36.133 gap pattern table

The current gap pattern table in TS36.133 is as below,

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| <Skip pattern 4-23> |  |  |  |  |
| NOTE 1: When determing UE requirements using Tinter1 for GP2 and GP3, Tinter1 = [60] for GP2 and Tinter1 = [30] for GP3 shall be used. NOTE 2: This gap pattern is supported by UEs which support shortMeasurementGap-r14 NOTE 3: Editor’s note:NR measurement requirement may not be scaled by Tinter NOTE 4: This gap pattern is applicable when per FR measurement gap is configured and one of ther serving cells in on FR2 and one of the to-be-measured cells is on FR2. |  |  |  |  |

There is a note 2 to apply pattern 2 and 3, this gap pattern is supported by UEs which support shortMeasurementGap-r14. But this note 2 doesn’t include the case that UE does’t support short gap but has been configured NR measurement.

If UE does not support shortMeasurementGap-r14 but support NR measurement, UE may still receive Gap-Pattern-Id 2 or 3 for NR inter-RAT measurements which is configured by the network.

If UE does not support shortMeasurementGap-r14 and no NR MO was configured, only Gap-Pattern-Id 0 or 1 can be configured for LTE inter-frequency or UTRAN, GERAN Inter-RAT measurements.

So the current note 2 in the above table shall be revised to:

NOTE 2:	This gap pattern is supported by UEs which support shortMeasurementGap-r14 or by UEs which are configured to perform inter-RAT NR measurement only.

Proposal 1: the note 2 in TS36.133 table 8.1.2.1-1 shall be revised to “This gap pattern is supported by UEs which support shortMeasurementGap-r14 or by UEs which are configured to perform inter-RAT NR measurement only.”

# TS38.133 gap pattern table

The current gap applicability tables in TS38.133 didn’t take into account the UE capability of supporting short gap for LTE measurement. In LTE TS36.133, if UE does not support shortMeasurementGap-r14 and if UE is not confiugred with NR MO only, only Gap-Pattern-Id 0 or 1 can be used for LTE inter-frequency or UTRAN, GERAN Inter-RAT measurements. So in EN-DC case the correspodning clarification shall be also captured in the TS38.133 gap applicability table 9.1.2-2 as well. Since when the UE access the LTE PCell it will exchange the capability issue to the LTE PCell and NR serving cell will not configure LTE measurement after UE going to EN-DC status, the UE capability of short gap shall be taken into account for all the LTE related  measuerment in EN-DC mode; that is, as long as UE is not capable to support short gap for LTE measurement, the gap pattern 2 and 3 shall not be configured to the UE for any LTE MO even though in some cases UE may have mixed MOs of LTE and NR RATs.

Proposal 2: in EN-DC case the corresponding clarification shall be also captured in the TS38.133 gap applicability table 9.1.2-2,

NOTE 2:  The gap pattern 2 and 3 are supported by UEs which support shortMeasurementGap-r14.

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE Measurement gap pattern configuration; Serving cell; Measurement Purpose; Applicable Gap Pattern Id. Per-UE measurement gap; E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2; non-NR RAT Note1,2; 0,1,2,3. FR1 and/or FR2; 0-11. non-NR RATNote1,2 and FR1 and/or FR2; 0,1,2,3. Per FR measurement gap; E-UTRA and, FR1 if configured; non-NR RAT Note1,2; 0,1,2,3. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 only; 0-11. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR2 only; No gap. FR2 if configured; 12-23. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR1; 0,1,2,3. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 and FR2; 0-11. FR2 if configured; 12-23. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR2; 0,1,2,3. FR2 if configured; 12-23. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR1 and FR2; 0,1,2,3. FR2 if configured; 12-23. Note: if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitered, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: Non-NR RAT includes E-UTRA, UTRA and/or GSM. NOTE 2: The gap pattern 2 and 3 are supported by UEs which support shortMeasurementGap-r14..

In SA case, currently RAN2 has no definition for UE capability indication short gap for inter-RAT LTE measurement, in order to keep the consistence of the LTE measurement capability between NR and LTE, the gap pattern 2 and 3 shall be removed from the applicability table Table 9.1.2-3.

Proposal 3: in SA case gap pattern 2 and 3 shall be removed for LTE related measurement from the measurement gap applicability table 9.1.2-3.

Table 9.1.2-3: Applicability for Gap Pattern Configurations supported by the UE with NR standalone operation Measurement gap pattern configuration; Serving cell; Measurement Purpose; Applicable Gap Pattern Id. Per-UE measurement gap; FR1, or FR1 + FR2; E-UTRA only; 0,1. FR1 and/or FR2; 0-11. E-UTRAN and FR1 and/or FR2; 0,1. FR2; E-UTRA only; 0,1. FR1 only; 0-11. FR1 and FR2; 0-11. E-UTRAN and FR1 and/or FR2; 0,1. FR2 only; 12-23. Per FR measurement gap; FR1 if configured; E-UTRA only; 0,1. FR2 if configured; No gap. FR1 if configured; FR1 only; 0-11. FR2 if configured; No gap. FR1 if configured; FR2 only; No gap. FR2 if configured; 12-23. FR1 if configured; E-UTRA and FR1; 0,1. FR2 if configured; No gap. FR1 if configured; FR1 and FR2; 0-11. FR2 if configured; 12-23. FR1 if configured; E-UTRA and FR2; 0,1. FR2 if configured; 12-23. FR1 if configured; E-UTRA and FR1 and FR2; 0,1. FR2 if configured; 12-23.

# Conclusion

In this contribution we discuss how to correct the note in the current gap pattern table to clarify the UE capability on short measurement gap for LTE measurement.

Proposal 1: the note 2 in TS36.133 table 8.1.2.1-1 shall be revised to “This gap pattern is supported by UEs which support shortMeasurementGap-r14 or by UEs which are configured to perform inter-RAT NR measurement only.”

Proposal 2: in EN-DC case the corresponding clarification shall be also captured in the TS38.133 gap applicability table 9.1.2-2,

NOTE 2:  The gap pattern 2 and 3 are supported by UEs which support shortMeasurementGap-r14.

Proposal 3: in SA case gap pattern 2 and 3 shall be removed for LTE related measurement from the measurement gap applicability table 9.1.2-3.

# References