3GPP TSG-RAN WG4 Meeting #87	R4-1806352
Busan, Korea May 21-25, 2018

Agenda item:	7.5.8.2

Source:	Apple Inc., Intel Corporation

Title:	Network Performance Analysis for Spherical Coverage

Document for:	Discussion

# 1	Introduction

During the RAN #73 meeting, the study item on New Radio access technology [1] was finalized with its outcome captured in TR 38.803 [2]. During RAN #75 the New Radio Work Item was initiated [3]. With an agreement to define and test RF requirements for NR mm-wave over the air (OTA), the discussion on spherical coverage has evolved over a number of agreements.

In RAN4#85, a WF on spherical coverage in FR2 was agreed in [4] and suggests a work plan for forthcoming meetings as follows:

Work Plan:

- RAN4 NR AH #4 (January ’18)

  - Initial discussion of simulation results (Both EIRP CDF and Network) based on the harmonized assumptions in this way forward.

  - Propose harmonized NW model assumptions and update based on preliminary analysis.

- RAN4 #86 (February ’18)

  - Deadline to submit the EIRP CDF simulation results based on the harmonized assumptions. Target preliminary EIRP CDF spherical requirement, based on the simulation outcomes.

  - Continue to improve the NW simulation accuracy reflecting initial EIRP CDF requirement (from AH #4)

  - Initial discussion of measurement results for prototype devices.

- RAN4 #86bis (April ’18)

  - Continue to improve the NW simulation accuracy reflecting preliminary EIRP CDF requirement (#86)

  - Continue to improve the prototype measurement effort and compare to preliminary EIRP CDF simulation

- RAN4 #87 (May ’18)

  - Finalize the spherical coverage requirement for handheld UEs based on the contributions

In RAN4#AH-1801, a WF on network performance analysis for spherical coverage in FR2 was agreed in [6] and provides harmonized simulation assumptions to be used in the performance analysis of spherical coverage.

Agreements:

- NW simulation scenarios

- At least indoor hotspots scenario as described in TR 38.803 should be simulated.

- For dense urban scenario, use the scenario as described in TR 38.803.

- For urban macro scenario, use the scenario as described in TR 38.803 (ISD=200m, 400m) with 0% indoor as the baseline.

- UE elevation distribution is modified to be uniform from 0 to 180 degrees.

- For UE resource allocation (UL), assume:

  - 20MHz for outage evaluation

    - Option 1: It is assumed that all UEs in the network are allocated fully overlapping resources

    - Option 2: Random resource allocation

  - 200MHz for mean throughput evaluation

    - Zero throughput UEs are excluded from the mean throughput calculation

- For indoor/outdoor UE ratios, assume:

  - For urban macro: 0% is the baseline, 20%, 100% indoor

  - For dense urban: 0% is the baseline, 20%, 100% indoor

  - Note 1: this is only for initial simulation comparison and NOT for performance requirement conclusion. Further down selection on the ratio to be done for performance requirement conclusion

  - Note 2: Results for 20 and 100% are informative.

- On blockage modeling:

  - No body blockage and hand grip modelling as baseline.

  - Companies are encouraged to provide results with blockage.

    - Study of blockage model is out of scope from the work plan. Companies can still bring results with any model of choice.

- On target UL SNR:

  - Target UL SNR = 22dB.

  - Note that the UL PC formula in TR38.803 should be modified to adjust for the conducted/TRP level assumed (see R4-167751).

- On UE antenna pattern modeling:

  - For UE antenna pattern modeling (that parameterizes (percentile, EIRP) the spherical coverage requirements), consider the following options:

    - Option 1: based on mathematical modelling. A detailed explanation of the model including each step in the simulation process and the corresponding simulated CDF should be provided

      - The proposed mathematical model should be able to achieve large variety of the spherical coverage CDFs by parameterization

    - Option 2: based on measured or simulated pattern. The assumption(s) used from the next slide and the corresponding EIRP CDF should be provided

RAN4#AH-1801, another WF on EIRP CDF for spherical coverage in FR2 was agreed in [7] with following agreements:

Agreements:

- It should be considered that the restrictions of real product UE to decide the spherical coverage requirement

- In order to give a clear information to RAN4#86, companies are encouraged to submit their contribution focusing on following results to meet the workplan

  - Information of display and cover material near antennas should be included

  - EIRP CDF Curve or EIRP loss at each %-tile point, [20-50%], from the peak in dB

In RAN4#86, a WF on spherical coverage in FR2 was agreed in [9] with following agreements:

Agreements:

- To evaluate performance sensitivity to CDF, use the CDF provided in TR38.803 as the reference with finite beam pointing directions per panel: [-60° -30° 0° 30° 60°] X [-60° -30° 0° 30° 60°]. [0°, 0°] is the boresight per panel based on local coordinates. Boresight shift of the two panels is 180° in azimuth

  - Note: Target EIRP CDF should also assume finite beams (beam pointing directions up to implementation for the target EIRP CDF)

- Absolute performance values of the reference CDF will be provided

In RAN4#86bis, following agreements were captured in the chairman’s notes:

Agreements:

- For Rel-15

  - For 28GHz

    - Min peak EIRP is 22.4 dBm

    - 50%-tile requirement for EIRP CDF is FFS

  - For 39GHz

    - Min peak EIRP is 20.6 dBm

    - 50%-tile requirement for EIRP CDF is FFS

- Note: for 50%-tile, the values are FFS.

During the last several RAN4 meetings, we have provided several simulation results evaluating the impact of EIRP CDF on the network performance.  A brief summary of our previous contributions is provided below:

In RAN4#85, we shared our views on the EIRP CDF for a modern form factor [5] and compared it to a form factor in which the radiating elements see largely plastic. Our results showed that when the phone model employs features which are common to the modern cellular phone, including a full display OLED/LCD and a metal bezel, spherical CDF performance is significantly compromised relative to that of a generally plastic and benign form factor.  We presented our view that setting spherical radiation requirements based on a benign form-factor design, will only impede adoption of the technology.

In RAN4#AH-1801, we provided our views on the simulation assumptions for spherical coverage performance evaluation in [8] and showed some simulation results on the outage probability of the urban macro scenario using the EIRP CDF in [5]. We showed that the outage probability of the urban macro scenario stays under 5% when the ISD is smaller than 200 m and peak EIRP is within the 21.2-25.2 dBm range.

In RAN4#86, we provided simulation results on throughput and outage performance for the indoor, dense urban, and urban macro deployment scenarios in [10]. We used minimum peak EIRP value of 21.2 dBm and observed that for indoor office, and dense urban scenario when all users are outdoor there is no outage in the network. There is also no outage in the urban macro deployment when all the users are outdoor and ISD is 200 m. It was however observed that urban macro scenario with ISD of 400 m is not a viable option due to high outage in both CDFs. The DL throughput loss of the masked CDF was shown to be less than 5% in indoor office scenario and less than 7% for dense urban and urban macro scenarios when all users were outdoor.

In RAN4#86bis, we provided additional simulation results on throughput and outage performance of the indoor, dense urban, and urban macro deployment scenarios in [11]. We used minimum peak EIRP of 22 dBm and simulated the updated assumptions provided in the WF [6] and finite beam directions for the 3GPP antenna pattern in TR 38.803 per [9]. We showed that the performance of the realistic double panel CDF (Model-2 (A+B)) [5] is comparable with that of 3GPP CDF from TR 38.803 in agreed scenarios. We also evaluated the sensitivity of the network performance to variations of the CDF at 50%-tile and 20%-tile and we observed that the network performance is less sensitive to the variations of peak EIRP at 20%-tile. We thus suggested to specify the spherical coverage requirement at not smaller than 50%-tile value.

In this paper, we evaluate the performance of the network when single panel UEs with realistic CDF (Model-2 (A))  as explained in [5], and [14] is being used. We study the outage and throughput performance of such UEs and compare them with those of double panel UEs. Our goal is to compare the performance of single and double antenna panels with CDF presented in [5] and [14] with that of the 3GPP antenna pattern in TR 38.803. We also intend to verify that the EIRP CDF of single antenna panel [5] provides reasonable performance in all three deployment scenarios. We also follow up on our analysis of the network performance sensitivity to variation of EIRP CDF values between 20%-tile and 50%-tile.

# 2	Simulation Results

In last few meetings, companies have provided views on the relevant parameters and scenarios in TR 38.803 and necessary modifications to address the spherical coverage performance as summarized in the WF [6] and [9]. In this paper, we follow up our previous contribution [11] in which we provided

simulation results for all three network deployment scenarios (indoor office, dense urban, and urban macro) at 30 GHz. For each scenario, we evaluated the mean throughput and outage probability when all UEs in the network are allocated fully overlapping resources and minimum peak EIRP was 22.4 dBm in the uplink. In [11], we used the spatially masked antenna pattern that we introduced in [8]; this allows us to simulate the same EIRP CDF we introduced as Model-2 (A+B) in [5], [14] derived from two antenna panels on the back of the UE.

In RAN4#86bis, there were discussions on using a single panel UE as baseline for the spherical coverage requirement [12]. It was also discussed that due to handgrip, body blockage, and MPE impact, it is reasonable to assume that only one of the antenna panels of the UE is active at a time. We concur with this observation.

Observation 1: Due to handgrip and body blockage, it is reasonable to assume that only one of the antenna panels of the UE is active at a time.

In this paper, we present the performance of a network comprising single antenna panel UEs and compare it with that of the two panel UEs as defined in TR 38.803. We use the CDF of a single panel UE (Model-2 (A)) that we introduced in [5], and [14]. We use finite beam directions for the 3GPP antenna pattern from TR 38.803 per [9]. Figure 1 demonstrates our CDF of a single panel UE (Model-2 (A)), two panel UE (Model-2 (A+B)), and the CDF of a two panel UE from TR 38.803.

Figure 1: EIRP CDF.

We note that simulation results of our two panel CDF were presented in [11]. In this paper, we present the performance of the single panel (Model-2 (A)) UE and compare it with that of the two panel CDF (Model-2 (A+B)) as well as the two panel CDF defined in TR 38.803. As we demonstrated in [11], the UMa scenario is the most challenging from a spherical coverage requirement perspective and it is only feasible when the ISD is 200 m and all users are outdoor.

## 2.1	Outage Performance

We simulated the coverage performance for the urban macro scenario using the minimum peak EIRP value of 22.4 dBm on the uplink. We defined the outage as a SNR below -10 dB and compared the outage probability of the single panel CDF (Model-2 (A)) and two panel CDF (Model-2 (A+B)) [5], [14] with that of the CDF defined in TR 38.803 with finite beam directions [9].  Table 1 shows the outage performance in the UMa scenario when ISD is 200 m and all users are outdoor.

|  | Single Panel (Model-2 A) | Two Panel (Model-2 A+B) | TR 38.803 |
| --- | --- | --- | --- |
| Outage (%) | 0.14 | 0.07 | 0.03 |

Table 1: Outage performance in UMa scenario.

We observe that the outage performance of one and two panel UEs is comparable.

Observation 2: In the UMa scenario when ISD is 200 m and all users are outdoor, the outage performance of one and two panel UEs is comparable.

## 2.2	Throughput Performance

We simulated the throughput performance for the urban macro and dense urban scenario. We compared the mean throughput performance of the single panel CDF (Model-2 (A)) and two panel CDF (Model-2 (A+B)) [5], [14] with that of the CDF defined in TR 38.803 with finite beam directions [9]. The simulated BW for the mean throughput analysis was 200 MHz. Table 2 shows the throughput loss of the UMa scenario with respect to the throughput of the TR 38.803 assumptions when ISD is 200 m and all users are outdoor.

|  | Average Throughput Loss (%) |  |
| --- | --- | --- |
|  | Single Panel (Model-2 A) | Two Panel (Model-2 A+B) |
| UL | 19 | 12 |
| DL | 0.24 | 0.09 |

Table 2: Throughput Loss in UMa scenario.

We observe that the difference in the mean UL throughput loss when UEs have realistic single and two panels is about 7%. Furthermore, the mean DL throughput performance is comparable.

Observation 3: In the UMa scenario when ISD is 200 m, the difference in the mean UL throughput loss when UEs have realistic single and two panels is about 7%. Furthermore, the mean DL throughput performance is comparable.

Table 3 shows the throughput loss of the dense urban scenario with respect to the throughput of the TR 38.803 assumptions when all users are outdoor.

|  | Average Throughput Loss (%) |  |
| --- | --- | --- |
|  | Single Panel (Model-2 A) | Two Panel (Model-2 A+B) |
| UL | 9.3 | 5.3 |
| DL | 1.04 | 0.64 |

Table 3: Throughput Loss in Dense Urban scenario.

We observe that the throughput performance of the dense urban scenario when UEs have realistic single and two panels is quite comparable.  We can therefore conclude that a single panel UE is a viable option and can be considered as the baseline for the spherical coverage requirement.

Observation 4: In the dense urban scenario, the throughput performance when UEs have realistic single and two panels is quite comparable.

Proposal 1: A single panel UE is a viable option and can be considered as the baseline for the spherical coverage requirement.

# 3	Network Performance Sensitivity to EIRP CDF

Over the past several meetings, companies have discussed the suitable percentile value of the EIRP CDF to be used for the spherical coverage requirement. In [11], we studied the sensitivity of the network performance to the EIRP percentile at 20%-tile and 50%-tile. We used the CDF from TR 38.803 where two panels are used for our study. To understand the sensitivity of the network performance to the 50%-tile value, we degraded the EIRP by 0-to-8 dB for all UEs whose EIRP percentile is lower than 50%. In another sets of simulations, we reduced the EIRP by 0-to-8 dB for all UEs whose EIRP percentile is lower than 20%. By comparing the outage and throughput loss performances, we concluded that the network performance is less sensitive to the variations of EIRP at 20%-tile. We therefore suggested that the spherical coverage requirement should be specified at not smaller than 50%-tile value [11].

In RAN4#86bis, one company also confirmed our conclusion [12] but another company argued that our method of CDF degradation is unrealistic and suggested a continuous CDF degradation [13]. We note that in a continuous CDF degradation at 20%-tile as suggested in [13], the CDF is also degraded at higher percentile values and the impact on the performance is thus from a mix of CDF degradations at the various values within the range of 50%-tile to 20%-tile. It is therefore unclear to what extent the network performance is sensitive to 20%-tile variations. In order to further evaluate the impact of CDF degradation on the network performance, we simulated 6 CDFs as shown in Figure 2. The CDFs are comprised of two sets:

1)

- TR 38.803 reference CDF,

- Discontinuous 20% degraded by 8 dB,

- Discontinuous 50% degraded by 8 dB,

These have been showed in [11].

2)

- Continuous 20% degraded by 8 dB, and 20%-50% CDF is degraded from 0 to 8 dB linearly,

- Continuous 20% degraded by 8 dB, and 20%-50% CDF is degraded from 0 to 8 dB in a 2nd order polynomial,

- Continuous 20% degraded by 8 dB, and 20%-50% CDF is degraded from 0 to 8 dB in a 3rd order polynomial.

Figure 2: CDF Degradation

There are countless methods of continuous CDF degradation but we only picked a few representative samples and note that in all continuous CDF degradation methods, the performance loss is  also function of CDF degradation in the 20%-tile to 50%-tile range.

Figure 3 shows the UL performance of all CDFs compared to the 3GPP reference for the UMa scenario with an ISD of 200 m and peak EIRP of 22.4 dBm. We observe that different “continuous” CDF degradation methods provide varied results. Furthermore, even the linear method (arguably the median CDF between the 20%-tile and 50%-tile degradation points) increases the mean and 5%-tile throughput loss by less than 5% compared to the discontinuous approach in [11].

Figure 3: Sensitivity of Network Performance to EIRP Degradation in UMa Scenario

Observation 5: Different “continuous” CDF degradation methods provide varied results but even the linear method (arguably the median CDF between the 20%-tile and 50%-tile degradation points) increases the mean and 5%-tile throughput loss by less than 5% compared to the discontinuous approach in [11].

In addition to the network performance evaluation, we also studied the UL UE throughput performance comparing the same set of CDFs explained above with 8dB degradation. The UE throughput is defined as the TR38.803 throughput for a fixed UE/BS link averaged over all UE orientations (i.e., EIRP CDF), as shown below.

Tput=EEIRP[αlog2(1+SNR)]

The receive SNR at the BS is SNR=EIRP+∆, where  captures all link characteristics including pathloss and the receiver characteristics (beamforming and antenna gain, noise/interference and noise figures) which is a constant for a fixed UE/BS pair. Maximum receive SNR is 22 dB in accordance with TR38.803..

We measure UL throughput loss at a fixed location in all possible UE orientations and also record the minimum receive SNR at that location. We vary the position of the UE across the cell to obtain a range of minimum receive SNR. Figure 4 shows the average throughput loss for a degraded CDF compared to the reference CDF  with the x-axis showing minimum Rx SNR logged at the location for the reference CDF.

Figure 4: Sensitivity of UE Throughput to EIRP Degradation in UMa Scenario.

Figure 4 shows that the linear CDF degradation at 20%-tile increases the average UE throughput loss by less than 8% compared to the 20%-tile discontinuous degradation while the discontinuous degradation of CDF at 50%-tile results in the largest throughput loss.

Observation 6: The linear CDF degradation at 20%-tile increases the average UE throughput loss by less than 8% compared to the 20%-tile discontinuous degradation while the discontinuous degradation of CDF at 50%-tile results in largest throughput loss.

Following the above observations, we understand that even though a continuous CDF degradation results in a  performance loss from a CDF degradation at the various values within the range of 50%-tile to 20%-tile, the network and UE performance is much less sensitive to the value of the EIRP CDF at 20%-tile for all CDF degradation methods. We therefore reiterate that defining the spherical coverage requirement at 20%-tile value is impractical.

Observation 7: The network performance is less sensitive to the variations of EIRP CDF at 20%-tile value.

Proposal 2: The spherical coverage requirement should be specified at not smaller than 50%-tile value.

# 4	Conclusions

In this contribution, we provided simulation results for throughput and outage performance of a UE with single and double antenna panels using the updated assumptions provided in the WF [6]. We compared the performance of our realistic antenna patterns with CDFs presented in [5], [14] with that of the antenna pattern in TR 38.803. We also evaluated the sensitivity of the network and UE performance to variations of EIRP CDF at 50%-tile and 20%-tile.

Based on the simulation results, the following observations and proposal are made:

Observation 1: Due to handgrip and body blockage, it is reasonable to assume that only one of the antenna panels of the UE is active at a time.

Observation 2: In the UMa scenario when ISD is 200 m and all users are outdoor, the outage performance of one and two panel UEs is comparable.

Observation 3: In the UMa scenario when ISD is 200 m, the difference in the mean UL throughput loss when UEs have realistic single and two panels is about 7%. Furthermore, the mean DL throughput performance is comparable.

Observation 4: In the dense urban scenario, the throughput performance when UEs have realistic single and two panels is quite comparable.

Observation 5: Different “continuous” CDF degradation methods provide varied results but even the linear method (arguably the median CDF between the 20%-tile and 50%-tile degradation points) increases the mean and 5%-tile throughput loss by less than 5% compared to the discontinuous approach.

Observation 6: The linear CDF degradation at 20%-tile increases the average UE throughput loss by less than 8% compared to the 20%-tile discontinuous degradation while the discontinuous degradation of CDF at 50%-tile results in largest throughput loss.

Observation 7: The network performance is less sensitive to the variations of EIRP CDF at 20%-tile value.

Based on the above observations, we would like to propose:

Proposal 1: A single panel UE is a viable option and can be considered as the baseline for the spherical coverage requirement.

Proposal 2: The spherical coverage requirement should be specified at not smaller than 50%-tile value.

# 5	References

- RP-161596, “Study on New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #73, September 2016.

- TR 38.803, “Study o New Radio Access Technology RF and co-existence aspects,” Version 2.0.0, 3GPP RAN #75, March 2017.

- RP-170855, “Work Item on New Radio (NR) Access Technology,” NTT DOCOMO, INC., 3GPP RAN #75, March 2017.

- R4-1714455, “WF on spherical coverage in FR2”, Samsung, Apple Inc., LGE, Intel, Oppo, Xiaomi, Vivo, MediaTek, 3GPP RAN4 #85, November 2017.

- R4-1713850, “Consideration of EIRP spherical coverage requirement”, Apple Inc., Intel Corporation, 3GPP RAN4 #85, November 2017.

- R4-1801198, “WF on network performance analysis for spherical coverage”, Samsung, RAN4#AH-1801, January 2018.

- R4-1801202, “WF on EIRP CDF for spherical coverage”, Samsung, MediaTek, RAN4#AH-1801, January 2018.

- R4-1800285, “Assumptions for Simulating Spherical Coverage Performance”, Apple Inc., Intel Corporation, RAN4#AH-1801, January 2018.

- R4-1803275, “WF on network performance analysis for spherical coverage”, Samsung, RAN4#86, February 2018.

- R4-1802220, “Network Performance Analysis for Spherical Coverage”, Apple Inc., Intel Corporation, 3GPP RAN4 #86, February 2018.

- R4-1805668, “Network Performance Analysis for Spherical Coverage”, Apple Inc., Intel Corporation, 3GPP RAN4 #86bis, April 2018.

- R4-1803820, “Network performance analysis for spherical coverage”, Samsung, 3GPP RAN4 #86bis, April 2018.

- R4-1805669, “Impact of spherical coverage requirement to NR network performance”, Qualcomm, 3GPP RAN4 #86bis, April 2018.

- R4-1806185, “EIRP CDF results for FR2 handheld UEs,” Apple Inc., 3GPP RAN4 #87, May 2018