3GPP TSG-RAN4 Meeting #87	R4-1806354

# Busan, Korea (Republic Of), 21st May 2018 - 25th May 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0006 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for RF requirements for Coherent UL MIMO for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of a new feature to 38.101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RF requirements for Coherent UL MIMO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requriements for Coherent UL MIMO will not be in 38.101 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of changes>

6.4D.4		RF requirements for Coherent UL MIMO

For coherent UL MIMO, relative difference in power and phase between the coherent antennas affects its performance. Since gNB estimates the phase and amplitude difference between the antennas when SRS is transmitted, the relative error from that at the last SRS transmitted is of importance.

The following table lists the maximum allowable delta of the relative power and phase errors measured at a given time from those measured at the last transmitted SRS. The requirements apply to the UEs and only to those bands that signal “Coherent UL MIMO” capability. Further, they only apply to the antennas in coherence at any given time. The requirements shall be met at any time within the time window specified from the last transmitted SRS.

Table 6.4D.4-1: Minimum requirements for delta of relative phase and power errors at a given time compared to those measured at last SRS transmitted

| Delta of relative phase error | Delta of relative power error | Time window |
| --- | --- | --- |
| ¦ error ¦ ≤ 40 degrees | ¦ error ¦ ≤ 4 dB | 20 msec |

The above requirements do not apply when the UE is signaled to disable UL Coherent MIMO or to disable one or more transmit chains, which are in coherence, before the above timer expires.

< End of changes>