3GPP TSG-RAN WG4 Meeting RAN4#87                                                 R4-1806444

Busan,Korean, 21 -25 May, 2018

Agenda Item:   7.12.1.5

Source:        Samsung

Title:          Overview of UE CSI requirements

Document for:  Discussion

Introduction

A dedicated specification was assigned for NR UE performance requirements as 38.101-4. The target competition time for Rel-15 NR performance part is RAN#82 in Dec 2018. In this contribution, preliminary views for UE CSI requirements were provided.

Discussion

Overall Test scope

CSI framework

NR_MIMO was designed to further improve performance and enable deployment in high frequency i.e. above 3GHz in FR1 and mmWave bands in FR2 with multi beam management. CSI framework in NR can be considered as an enhancement based on the design for Rel-14 eFD-MIMO CSI to facilitate hybrid beam operation of. Modular CSI framework was introduced in a future proof manner which consists of CSI-RS resources settings, CSI reporting settings and links configurations between them for measurement list.

CSI reporting content

In NR, CSI consists of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), layer indication (LI), rank indication (RI) and/or L1-RSRP.

CSI reporting/CSI-RS resource setting

For CSI reporting setting/CSI-RS resource setting, ‘aperiodic’, and ‘periodic’ and ‘semi-persistent’ types are supported for NR.

For CSI-RS resource setting, both CSI-IM resources and NZP CSI-RS resources can be configured for interference measurement.

NR UE feature list related to CSI

It’s unrealistic to verify all the features, functionality and different configurations specified in CSI framework considering work load and tight timeline for Rel-15 NR performance requirements. Also considering diverse capability values for different feature groups, it’s impractical to introduce test cases covering all UE capabilities.

P1: Introducing CSI performance requirements focused on the mandatory and fundamental features of NR CSI framework under Rel-15 timeframe.

Following agreed NR feature list in RAN1 (LS R1-1805728), below basic CSI feedback was mandatory without UE capability.

P2-1: CSI report types: Rel-15 focused on periodic CSI report and aperiodic CSI report.

For CSI resources, below capability agreed:

P2-2: CSI-RS resources setting: Rel-15 focused on 1 NZP CSI-RS resource and 1 CSI-IM resource configuration for CSI report. CSI-IM used for interference measurement.

For number of configured CSI report, below capability signalling agreed:

P2-3: Number of CSI report and BWP: Rel-15 focused on single BWP/CC with 1 CSI reporting configured.

L1-RSRP

L1-RSPR was introduced in NR CSI framework for beam management. Unlike RSRP reporting in RRM, the reporting mechanism and trigger mechanism are different between RRM measurement and CSI measurement. L1-RSRP should be included in the scope of CSI performance requirements.

L1-RSRP reporting is essential feature al to facilitate beam management (Tx/Rx beam switching) for data and control channel transmission during RRC connected state. Follow agreed UE feature list, periodic and aperiodic L1-RSRP report is mandatory at least for FR2.

Proposal 3: L1-RSRP belong to NR CSI framework, new requirements need to be introduced under CSI performance requirements agenda in Rel-15.

CQI Test

Generally, CQI test cases can be categorized as below test cases:

- Static CQI definition test: verify UE properly implementation of effective SNR calculation and SNR to CQI table mapping followed CQI definition

- Wideband CQI test: verify UE properly tracking channel/interference variation in time domain to avoid excessive averaging across sub-frames

- Sub-band CQI test: verify UE properly tracking channel/interference variation in frequency domain to avoid excessive averaging across frequency domain

Proposal 4: For CQI test, both static CQI definition test, wideband fading CQI test and sub-band frequency selective test need to be introduced.

For static CQI test, three types of CQI tables introduced in TS 38.214:

- Table 1 (without 256QAM), table 2 with 256QAM for target BLER 10%

- Table 3 with configurable target BLER

In general, existing test metric can be reused for static CQI test at least in FR1. For FR2, FFS pending on further conclusion for achievable SNR accuracy and SNR range of TE implementation under ‘pure baseband’ test methodology.

Proposal 5: For static CQI test, test metrics (CQI distribution and BLER requirements) as existing LTE test can be reused as starting point at least for FR1 and for FR2, FFS pending on achievable SNR accuracy and SNR range of TE implementation under ‘pure baseband’ test methodology.

| Parameter | Unit | Static CQI test (FR1) | Static CQI test (FR2) |
| --- | --- | --- | --- |
| System parameter |  |  |  |
| Bandwidth/SCS | MHz/kHz | 20MHz/30KHz | 100MHz/60kHz |
| Transmission Scheme |  | 1 | 1 |
| Duplex Mode |  | FDD &TDD | TDD |
| DLUL configuration |  | FFS | FFS |
| Propagation channel |  | AWGN | AWGN |
| Correlation and antenna configuration |  | 2Rx 2*2 with 4Rx: 2*4 with | 2Rx 2*2 with |
| CSI-RS resource setting (periodic) |  |  |  |
| resourceConfigType |  | FFS | FFS |
| CSI-RS/CSI-IM periodicity and slot offset |  | FFS | FFS |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | FD-CDM2/density 1 | FD-CDM2/density 1 |
| CSI-IM configuration |  | FFS | FFS |
| CSI report setting (periodic) |  |  |  |
| reportConfigType |  | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS |
| Codebook Restriction |  | 010000 | 010000 |
| Reporting granularity |  | Single CQI reporting | Single CQI reporting |
| Physical channel for reporting |  | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 1 without 256QAM |

PMI Test

For PMI reporting, 4 types of codebook structure were introduced for different applicable scenarios (SU-MIMO/MU-MIMO, single panel/multi-panels):

- Type I–Single panel codebook: similar as LTE eFD-MIMO Class A codebook (single Beam and co-phasing selection)

- Type I–Multi panel codebook: further extended with inter-panel co-phasing selection for multi panels Ng = (2 or 4)

- Type II codebook: similar as LTE eFD-MIMO Advanced codebook with multi-beam linear combinations

- Type II Port Selection Codebook

Different codebook structures correspond to different deployment scenario and beamforing scheduled schemes. Proposal 6: New PMI test cases required to verify PMI reporting accuracy with different codebook configurations (Type I–Single panel codebook, Type I–Multi panel codebook, Type II codebook and Type II Port Selection Codebook).

For Type I–Single panel codebook, test metric and beamforming mode for LTE Rel13 FDMIMO Class A PMI test cases can be reused as a starting point.

In order to construct Type I–Single panel codebook, below RRC parameters need to be configured:

- Number of ports in vertical/horizontal directions: (N1,N2)

- Oversampling rate: (O1,O2)

- CodebookMode: 1/2

Proposal 7: Introduce below PMI test cases for Type I–Single panel codebook

- Single PMI test case: 8 ports, aperiodic CSI report, (N1,N2) = (2,2) and (O1,O2) = (4,4), CodebookMode 1/2 (Rank1)

- Multiple PMI test case: 4 ports, aperiodic CSI report, (N1,N2) = (2,1) and (O1,O2) = (4,1), CodebookMode 1 (Rank2)

A detailed test set-up proposed in table below for Type-Single panel codebook test

| Parameter | Unit | Single PMI Test | Multiple PMI test |
| --- | --- | --- | --- |
| System parameter |  |  |  |
| Bandwidth/SCS | MHz/kHz | 20MHz/30kHz (FR1) | 20MHz/30kHz (FR1) |
| Transmission Scheme |  | 1 | 1 |
| Duplex Mode |  | FDD &TDD | FDD &TDD |
| DLUL configuration (TDD) |  | FFS | FFS |
| Propagation channel |  | TDL A (30 ns), 5Hz | TDL B (100ns), 5Hz |
| Correlation and antenna configuration |  | (N1,N2) = (2,2) 8*2 XP High 8*4 XP High | (N1, N2) = (2,1) 4*2 XP High 4*4 XP High |
| CSI-RS resource setting (periodic) |  |  |  |
| ResourceConfigType |  | periodic | periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | FFS | FFS |
| NZP CSI-RS ports |  | 8 port {3000,….,3007} | 4 port {3000,…,3003} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | CDM4,density 1 (row 9) | FD-CDM2 density 1 (row 4) |
| CSI-IM configuration |  | FFS | FFS |
| CSI report setting (aperiodic) |  |  |  |
| reportConfigType |  | aperiodic | aperiodic |
| (N1,N2) |  | (2,2) | (2,1) |
| (O1,O2) |  | (4,4) | (4,1) |
| CodebookMode |  | 1/2 | 1 |
| Reporting interval |  | FFS | FFS |
| Codebook Restriction |  | FFS | FFS |
| Reporting granularity |  | Single PMI reporting | Mutiple PMI reporting |
| Physical channel for reporting |  | PUSCH | PUSCH |
| Rank &MSCS |  | Rank1 | Rank2 |

Conclusion

In this contribution, preliminary views for UE CSI requirements were provided.

Test scope

P1: Introducing CSI performance requirements focused on the mandatory and fundamental features of NR CSI framework under Rel-15 timeframe.

P2: In Rel-15, NR CSI test cases focused on below configurations:

- CSI report types: periodic CSI report and aperiodic CSI report.

- CSI-RS resources setting: 1 NZP CSI-RS resource and 1 CSI-IM resource configuration for CSI report. CSI-IM used for interference measurement.

- Number of CSI report and BWP: single BWP/CC with 1 CSI reporting configured.

P3: L1-RSRP belong to NR CSI framework, new requirements need to be introduced under CSI performance requirements agenda in Rel-15.

CQI Test

Proposal 4: For CQI test, both static CQI definition test, wideband fading CQI test and sub-band frequency selective test need to be introduced.

Proposal 5: For static CQI test, test metrics (CQI distribution and BLER requirements) as existing LTE test can be reused as starting point at least for FR1 and for FR2, FFS pending on achievable SNR accuracy and SNR range of TE implementation under ‘pure baseband’ test methodology.

PMI Test

Proposal 6: New PMI test cases required to verify PMI reporting accuracy with different codebook configurations (Type I–Single panel codebook, Type I–Multi panel codebook, Type II codebook and Type II Port Selection Codebook).

Proposal 7: Introduce PMI test cases for Type I–Single panel codebook

- Single PMI test case: 8 ports, aperiodic CSI report, (N1,N2) = (2,2) and (O1,O2) = (4,4), CodebookMode 1/2

- Multiple PMI test case: 4 ports, aperiodic CSI report, (N1,N2) = (2,1) and (O1,O2) = (4,1), CodebookMode 1

Reference

[1] R4-1805549, “Way Forward on NR UE performance requirements”, Intel

[2] R1-1805752, “LS on NR UE feature list”