3GPP TSG-RAN WG4 Meeting #87                     	R4-1806540

Busan, Korea, May 21st –25th, 2018

Agenda item:            7.1.4

Source:	MediaTek Inc.

Title:	Features regarding 1 symbol GP for 60KHz in FR1 and 120KHz in FR2

Document for:          Discussion

1	Introduction

In last RAN4 meeting, there were some agreements [1][2] achieved with the impact on whether to support of 1 symbol GP for for 60KHz in FR1 and 120KHz in FR2. In this paper, we discuss their impact on the feature list in Rel-15.

2	Discussion

In last RAN4 meeting, it was agreed in RRM session that

Proposal 1: TA_offset is not tightened in Rel-15.

The agreement was captured in the meeting report. Later, a WF [1] was agreed as captured below

- It is not feasible to support the features of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2 in Rel-15 - LS to RAN2 to remove the features from the feature list in Rel-15 - FFS on how to handle later releases - LS to RAN1 on - The infeasibility of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2 in Rel-15 - The feasible GP length, at least given by GP≥TAmax+TA offset+TUE RX-TX - TAmax is max timing advance value based on expected cell size - TUE RX-TX is addressed in another separate LS to RAN1 (R4-1805766) - TA offset can be referred to TS38.133 Table 7.1.2-2

In the parallel UE RF session, an LS [2] was agreed on UE’s max allowed transition times from Rx to Tx and from Tx to Rx. The times are captured in below table.

| Transition/FR | FR1 | FR2 |
| --- | --- | --- |
| TRX2TX | 13s | 7s |
| TTX2RX | 13s | 7s |

If we jointly consider above 2 agreed document together, the minimum GP length is calculated in Table 1 which assumes zero TA. It can be seen that 1 symbol GP for 60KHz in FR1 and 120KHz in FR2 are not enough. This observation coincides the 1st agreement in [1].

Table 1 minimum GP length

|  | SCS | 1 OFDM symbol duration (us) | TTA, offset (us) | TRX2TX (us) | Minimum GP length, assuming zero TA: TTA, offset + TRX2TX (us) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 15KHz | 71.35 | 13 | 13 | 26 |
|  | 30KHz | 35.68 | 13 | 13 | 26 |
|  | 60KHz | 17.84 | 13 | 13 | 26 |
| FR2 | 60KHz | 17.84 | 7 | 7 | 14 |
|  | 120 KHz | 8.92 | 7 | 7 | 14 |

Based on above analysis and the agreement in [1], it is suggest to remove the features of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2.

Proposal 1: Remove the features of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2

3	Conclusions

In the contribution, we discuss the features of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2 based the progress in the last RAN4 meeting. The proposal is

Proposal 1: Remove the features of 1 symbol GP for 60KHz in FR1 and 120KHz in FR2

4	Reference

[1] R4-1805534, “Wayforward on 1 symbol GP”, Mediatek inc

[2] R4-1805766, “LS on UE Rx/Tx Transition Times”, RAN4