3GPP TSG-RAN WG4 Meeting #87                     	R4-1806547

Busan, Korea, May 21st – 25th, 2018

Agenda item:            7.1.4

Source:	MediaTek Inc.

Title:	Consideration on TDM and FDM based ULSUP with SUL bands

Document for:          Discussion

# 1	Introduction

In this paper, we discuss the feature “Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective”

# 2	Discussion

It is agreed in RP#79 [1] that for UpLink sharing from UE perspective (ULSUP)

- In Q2 2018, RAN4 will setup the agenda item for ULSUP with SUL band with following sub agenda items.

- TDM based ULSUP with SUL bands

- FDM based ULSUP with SUL bands

In RAN4 #86bis, the following is agreed as in the meeting report

Proposal 1: UE requirements for FDM based ULSUP are developing assuming simultaneous LTE and NR UL transmissions on the shared UL carrier used in EN-DC.

Proposal 3: No UE switching time is allowed for FDM based ULSUP as UE is required to support simultaneous LTE and NR UL transmissions on the shared UL carrier.

Based on these agreement, in FDM mode, power sharing and simultaneous LTE and NR UL transmission must be supported in the UE, which is different from TDM mode

From RAN4 perspective, therefore, a new feature needs to be introduced in the System parameter group (1-10) for the UE to signal which multiplexing scheme(s) it is capable of supporting.

Proposal: Introduce a new feature for the signaling of supported multiplexing scheme(s) for UpLink sharing from UE perspective as shown in Table 1

Table 1 Feature for Supplemental uplink in uplink sharing from UE perspective

| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB t | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System parameter | 1-10a | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) TDM based 2) FDM based |  |  |  | Type 3 (per band combination) | No Need | Applicable only to FR1 |  |  |  |  |  |

# 3	Conclusions

In the contribution, we provide our view on the Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective. We have the following proposals:

Proposal: Introduce a new feature for the signaling of supported multiplexing scheme(s) for UpLink sharing from UE perspective

# 4	Reference

# [1] RP-180560, “WF on RAN4 work plan for uplink sharing from UE respective (ULSUP)”

# [2] R4-1803564, “Updated UE feature list”