3GPP TSG- RAN WG4 Meeting #87 	R4-1806589

# Busan, Korea, 21 – 25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: AAS OTA sensitivity conformance testing with CATR (10.3.2.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The details of Rel-13 AAS BS OTA sensitivity conformance testing with CATR have been recorded in TR 37.842, and they can be reused in TR 37.843 for Rel-15 eAAS BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Record the details of Rel-15 eAAS BS OTA sensitivity conformance testing with CATR in TR 37.843, according to those in TR 37.842. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details of Rel-15 eAAS BS OTA sensitivity conformance testing with CATR in TR 37.843 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 10.3.2.3	CATR

##### 10.3.2.3.1	General

CATR uses the feed antenna which radiates a spherical wavefront to a range reflector antenna which will then collimate the radiated spherical wavefront to the DUT. There is sufficient separation between the DUT and the transmitter (feed antenna shown in figure 10.3.2.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The feed antenna transmits a wavefront that will illuminate the range antenna reflector, which will then reflect the transmitted energy towards the DUT.

# Figure 10.3.2.3.1-1: CATR measurement system setup for EIS

##### 10.3.2.3.2 	Calibration

The calibration measurement is done by using a reference antenna (SGH used in figure 10.3.2.3.2-1) with known efficiency or gain values. In the calibration measurement the reference antenna is measured in the same place as the DUT, and the attenuation of the complete transmission path (A↔B) from the DUT to the measurement receiver is calibrated out. The following figure is used as a setup of a typical compact antenna test range.

# Figure 10.3.2.3.2-1 CATR calibration system setup for EIS

1)	Path loss calibration C→A:

a)	Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓSGH ΓSGH. .

b)	Measure cable loss from point C to input of SGH, call this LC↔SGH LC↔SGH  which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from C→A by using the following expression LSGHcal=Lc,SGH+10log1-ΓSGH2-GSGH LSGHcal = LC,SGH + 10log(1 - |ΓSGH|2) - GSGHLSGHcal=Lc,SGH+10log1-ΓSGH2-GSGH,, where10log1-ΓSGH2 10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss,GSGH GSGH GSGH  is the known gain of the reference SGH.

2)	Connect SGH and C↔A cable.

3)	To remove polarization(s) mismatch between range antenna (labelled as feeder antenna in diagram) and SGH use positions to position the SGH in the beam peak direction of range antenna.

4)	Measure path loss C→B with network analyzer LC→B = 20log|S21|.

5)	Calculate the test path loss compensation factor. This is the total path loss between A↔B using the results from step 1c and 4. L = LSGHcal LSGHcal  - LC→B.

Where ΓSGH is the reflection coefficient (or mismatch) seen at the SGH connector (S11 with a network analyzer).

##### 10.3.2.3.3 	Procedure

1)	Set up AAS BS in place of SGH from calibration stage. Align AAS BS with beam peak direction of range antenna.

2)	Configure signal generator, one port (polarization) and one carrier at a time according to maximum power requirement.

3)	Start with signal level at sensitivity level using FRC A1-3 in 3GPP TS 36.104 [8] configuration.

4)	Calculate EIS per port (polarization) at this point with EIS = PBER - LA→B.

5)	Repeat steps 1-4 for all conformance test directions as described in 3GPP TS 37.145-2 [24], subclause 7.2.

##### 10.3.2.3.4 	MU assessment

###### 10.3.2.3.4.1 	MU Budget

# Table 10.3.2.3.4.1-1: CATR uncertainty budget format for EIS measurement

| UID | Description of uncertainty contribution | Details in anex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | C2-1 |
| 2 | Standing wave between DUT and test range antenna | C2-2 |
| 3 | Signal Generator uncertainty | E |
| 4 | RF leakage & dynamic range, a) feeder package terminated b) DUT terminated | C2-3 |
| 5 | QZ ripple DUT | C2-4 |
| 6 | Miscellaneous uncertainty | C2-13 |
| Stage 1: Calibration measurement |  |  |
| 7 | Uncertainty of network analyser and/or signal generator | E |
| 8 | Mismatch of transmitter chain | C2-6 |
| 9 | Insertion loss of transmitter chain | C2-7 |
| 10 | RF leakage & dynamic range | C2-8 |
| 11 | Influence of the calibration antenna feed cable a) Flexing cables, adapters, attenuators & connector repeatability | C2-9 |
| 12 | Uncertainty of the absolute gain of calibration antenna | E |
| 13 | Uncertainty due to antenna mounting apparatus or rotary joints | C2-11 |
| 14 | Quality of quiet zone | C2-4 |
| 15 | Rotary Joints | C2-14 |
| 16 | Misalignment positioning system | C2-15 |
| 17 | Misalignment calibration antenna | C2-1 |
| 18 | Standing wave between DUT and test range antenna | C2-16 |
| 19 | Switching uncertainty | C2-17 |

###### 10.3.2.3.4.2 	MU Value

# Table 10.3.2.3.4.2-1: CATR uncertainty assessment for EIS measurement

| EIS uncertainty budget |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz |  | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |  |
| 2 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |  |
| 3 | RF signal generator | 0.46 | 0.46 | Gaussian | 1 | 1 | 0.46 | 0.46 |  |
| 4 | RF leakage & dynamic range, test range antenna cable connector terminated. | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 |  |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 |  |
| 6 | Miscellaneous uncertainty | 0 | 0 | Normal | 1 | 1 | 0 | 0 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |
| 7 | Network Analyzer | 0.13 | 0.20 | Normal | 1 | 1 | 0.13 | 0.20 |  |
| 8 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 |  |
| 9 | Insertion loss of transmitter chain | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.10 | 0.10 |  |
| 10 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | 0.00156 | 0.00156 | Normal | 1 | 1 | 0.00156 | 0.00156 |  |
| 11 | Flexing cables & connector repeatability | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 |  |
| 12 | SGH Calibration uncertainty | 0.502 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 |  |
| 13 | Misalignment positioning system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |  |
| 14 | QZ ripple with SGH | 0.09 | 0.09 | Normal | 1 | 1 | 0.09 | 0.09 |  |
| 15 | Rotary joints | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 |  |
| 16 | Misalignment SGH and pointing error | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 |  |
| 17 | Misalignment calibration system | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 |  |
| 18 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 |  |
| 19 | Switching uncertainty | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.68 | 0.71 |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.33 | 1.40 |  |

<End of change>