3GPP TSG- RAN WG4 Meeting #87 	R4-1806593

# Busan, Korea, 21 – 25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: AAS OTA REFSENS conformance testing (10.3.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The draft CR to TS 37.145-2 for Rel-15 AAS BS OTA REFSENS conformance testing have been endorsed in RAN4#86Bis, while the corresponding details in TR 37.843 have yet to be filled in. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fill in the details for Rel-15 eAAS BS OTA REFSENS conformance testing in TR 37.843, based on the details for Rel-15 eAAS BS OTA sensitivity conformance testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details for Rel-15 eAAS BS OTA REFSENS conformance testing in TR 37.843 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 10.3.3 	OTA reference sensitivity

#### 10.3.3.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.3.3.2	In-door anechoic chamber

##### 10.3.3.2.1	General

This method measures the OTA reference sensitivity in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the transmitting antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.2.2.1-1.

##### 10.3.3.2.2	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.2.2.

##### 10.3.3.2.3 	Procedure

Refer to procedure in subclause 10.3.2.2.3 where in	 step 5) the required level is EISREFSENS.

##### 10.3.3.2.4 	MU assessment

###### 10.3.3.2.4.1 	MU Budget

Same budget as shown in table 10.3.2.2.4.1-1.

###### 10.3.3.2.4.2 	MU Value

Same value as shown in table 10.3.2.2.4.2-1.

#### 10.3.3.3	CATR

##### 10.3.3.3.1	General

CATR uses the feed antenna which radiates a spherical wavefront to a range reflector antenna which will then collimate the radiated spherical wavefront to the DUT. There is sufficient separation between the DUT and the transmitter (feed antenna shown in figure 10.3.2.3.1-1) so that the emanating spherical wave reaches nearly plane phase fronts from transmitter to receiver. The feed antenna transmits a wavefront that will illuminate the range antenna reflector, which will then reflect the transmitted energy towards the DUT.

##### 10.3.3.3.2 	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.3.2.

##### 10.3.3.3.3 	Procedure

Refer to procedure in subclause 10.3.2.3.3 where in step 4) the required level is EISREFSENS.

##### 10.3.3.3.4 	MU assessment

###### 10.3.3.3.4.1 	MU Budget

Same budget as shown in table 10.3.2.3.4.1-1.

###### 10.3.3.3.4.2 	MU Value

Same value as shown in table 10.3.2.3.4.2-1.

#### 10.3.3.4	One Dimensional Compact Range Chamber

##### 10.3.3.4.1	General

The principle of the measurement set up for OTA sensitivity is shown in figure 10.3.2.4.1-1. It closely resembles the standard compact range chamber. But for the generation of the plane wave in the quiet zone the proposed method uses a special one dimensional probe instead of the standard set up comprising a concave mirror and a feed horn. The main advantage is a reduced chamber size and a simplified probe system replacing the standard mirror feed horn set up.

##### 10.3.3.4.2	Calibration and Procedure

Calibration shall be done with the procedure shown in subclause 10.3.2.4.2.

Refer to procedure in subclause 10.3.2.4.2 where in step 6) the required level is EISREFSENS.

##### 10.3.3.4.3 	MU assessment

###### 10.3.3.4.3.1 	MU Budget

Same budget as shown in table 10.3.2.4.3.1-1.

###### 10.3.3.4.3.2 	MU Value

Same value as shown in table 10.3.2.4.3.2-1.

#### 10.3.3.5	Near Field Test range

##### 10.3.3.5.1	General

The near field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS BS under test, on some specific surfaces such as planar, cylindrical, and spherical. Each of the near field test methods can be implemented by one or more mechanical rotations of the probe and/or AAS BS under test as shown in figure 10.3.2.5.1-1. One or more mechanical movement can be substitute by a probe array. All the scanning methods will need an RF transmit and receive system equipped with an automated scanning, a data collection and control system, and computerized analysis ability.

##### 10.3.3.5.2	Calibration

Calibration shall be done with the procedure shown in subclause 10.3.2.5.2.

##### 10.3.3.5.3	Procedure

Refer to procedure in subclause 10.3.2.5.3 where in step 2) the required level is EISREFSENS.

##### 10.3.3.5.4 	MU assessment

###### 10.3.3.5.4.1 	MU Budget

Same budget as shown in table 10.3.2.5.4.1-1.

###### 10.3.3.5.4.2 	MU Value

Same value as shown in table 10.3.2.5.4.2-1.

#### 10.3.3.6	Summary

Same common maximum accepted test system uncertainty values as shown in table 10.3.2.6-1.

<End of change>