3GPP TSG-RAN WG4 Meeting #87	R4-1806760

Busan, Korea (Republic of), 22 – 25 May 2018

Agenda Item:	6.20.7

Source:	Ericsson

Title:	Discussion on BS demodulation requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

RAN4#86bis agreed with the way forward on BS demodulation requirements for eFeMTC [1] as follows.

RAN4 will introduce new PUSCH demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK demodulation requirements. - For detailed parameters, RAN4 will wait for RAN1 conclusion. - FFS: 3/6 subcarriers QPSK demodulation requirements RAN4 will introduce new PUSCH demodulation requirements for CEModeA with higher Doppler condition. - Doppler frequency - Option 1: 200 Hz - Other options (e.g., lower than 200Hz) are not precluded

In this contribution, we discuss the open issues on BS demodulation requirements and propose simulation assumptions.

# 2	Discussion

## 2.1	PUSCH sub-PRB allocation

Table 1 summarizes the RAN1 agreement so far. As table shows, RAN1 has not agreed with the applicable MCS/TBS tables.

Table 1	Summary of PUSCH sub-PRB allocation.

| Sub-PRB allocation type | RU size | Modulation | Number of RUs | MCS/TBS |
| --- | --- | --- | --- | --- |
| 2 of 3 subcarriers | 8ms | Pi/2 BPSK | 1, 2, and 4 for CE Mode A 2 and 4 for CE Mode B | Need to wait for RAN1 decision |
| 3 subcarriers | 4ms | QPSK |  |  |
| 6 subcarriers | 2ms | QPSK |  |  |

Since RAN4 needs TBS for simulation assumptions, RAN4 should wait for RAN1 decision on MCS/TBS for PUSCH with subPRB resource allocation.

Proposal 1: RAN4 should wait for RAN1 decision on MCS/TBS regarding sub-PRB resource allocation.

## 2.2	PUSCH CE Mode A with high Doppler

As Rel-15 eMTC WID mentions, one of the objectives is to specify new requirements for high UE velocity in CE Mode A [2]. TS36.104 specifies one set of BS demodulation requirements for PUSCH with CE Mode A as shown in Table 2 and Table 3. We propose to specify the BS demodulation requirement by reusing the same configuration, but propagation condition is set to EPA200.

Proposal 2: RAN4 will specify the PUSCH demodulation requirements with the following setting: 1x2 EPA200, 8 PUSCH repetitions, FRC A3-2 (QPSK 1/3, TBS=600bit, 6PRB).

Table 2	Test parameter for PUSCH CE mode A

| Parameter | unit | Mode A |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions |  | 8 |
| Frequency hopping |  | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD |

Table 3	Test case for PUSCH CE Mode A

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Mode A | EPA 5Hz Low => EPA 200MHz Low | A3-2 | 70% |

# 3	Conclusion

Proposal 1: RAN4 should wait for RAN1 decision on MCS/TBS regarding sub-PRB resource allocation.

Proposal 2: RAN4 will specify the PUSCH demodulation requirements with the following setting: 1x2 EPA200, 8 PUSCH repetitions, FRC A3-2 (QPSK 1/3, TBS=600bit, 6PRB).

# 4	References

R4-1805477, “Way forward on BS demodulation requirements for eFeMTC”, Ericsson, Samsung, Huawei.

RP-172811, “Revised WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm.