3GPP TSG RAN WG4 Meeting #87	                                          	R4-1807418

Busan, Korea, May 21st – 25th, 2018

Agenda item:	7.12.2.3

Source: 	AT&T

Title: 	PUCCH demodulation requirements

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  In the previous RAN4 meeting (RAN4#86bis), it was decided to define BS demodulation requirements for PUSCH, PUCCH, PRACH [1] in FR1 and reuse the same for FR2 unless no technical issues are identified.

In this contribution, we provide our views on PUCCH demodulation requirements for NR.

# PUCCH Demodulation Requirements

In this section, we discuss the key aspects of PUCCH demodulation requirements.

- PUCCH Formats: NR defines 5 PUCCH formats for reporting HARQ-ACK, SR and CSI. Table 1 summarizes the characteristics of each PUCCH format.  For improving coverage, we prefer to have performance requirements for PUCCH Format 1 as the first priority.  Since it is already decided that RAN4 will set performance requirements for single user case, we can skip Format 4.  In addition, we prefer PUCCH Format 3 for CSI feedback and Format 0 for fast HARQ-ACK responses. Hence, we would like to have performance requirements defined for Formats 1, 3 and 0.

Proposal 1: RAN4 should define performance requirements for PUCCH Formats 1, 3 and 0

Table 1 PUCCH Formats in NR

| Format Name | Alternative name | Symbol length | Waveform | Information | Single UE/Multiple UE |
| --- | --- | --- | --- | --- | --- |
| Format 0 | Short PUCCH <= 2 bits | 1 -2 | CP-OFDM | HARQ-ACK, SR | Single UE |
| Format 1 | Long PUCCH <=2 bits | 4-14 | CP-OFDM | HARQ-ACK, SR | Single UE |
| Format 2 | Short PUCCH>2 bits | 1-2 | CP-OFDM | CSI | Single UE |
| Format 3 | Long PUCCH >2 bits | 4-14 | DFT-s-OFDM | CSI | Single UE |
| Format 4 | Long PUCCH >2 bits | 4-14 | DFT-s-OFDM | CSI | Multiple UEs |

- Frequency Hopping:   As the bandwidth allocated in NR carriers can be very large, we expect significant channel variation. Hence frequency hopping is attractive to obtain frequency diversity gain. Hence we would like to define performance requirements for frequency hopping enabled for long PUCCH.

Proposal 2: Due to the possibility of large channel bandwidths in NR, we would like to specify performance requirements with frequency hopping enabled

- Multi Slot PUCCH:  Since NR PUCCH would be deployed in frequencies much higher than LTE PUCCH, its coverage would be limited due to higher propagation losses. NR needs new techniques to mitigate these losses.  In our evaluations, we observed that receive diversity can help. Also, if PUCCH is repeated over multiple slots, we can get additional diversity as the receiver aggregates symbols in multiple slots and uses, for example, maximum ratio combining for decoding the long PUCCH. The additional diversity can be obtained if we use multi slot based transmission for long PUCCH.  Hence we recommend to define performance requirements for multi slot based transmission for long PUCCH.

As an example, Figure 1 shows BLER for PUCCH Format 1 with 2 bits of HARQ-ACK when LTE-PUCCH is deployed at 700 MHz band and NR-PUCCH is deployed at 2 GHz band.  Simulation assumptions are outlined in the Appendix. It can be observed that with 4 slots of PUCCH, NR can match LTE coverage.

Figure 1 BLER comparison for PUCCH Format 1 with multi slot transmission

Similarly, Figure 2 shows the performance of PUCCH Format 3 with Pi/2 BPSK modulation and CSI length equal to 4 bits. In this case too we observe that NR can match the coverage of LTE at low bands by using 2 slot operation.

Figure 2  BLER comparison for PUCCH Format 3 with multi slot transmission

Proposal 3: To match LTE coverage at low frequency bands, performance requirements should be defined for multi slot operation of long PUCCH with number slots repeated equal to 2 and 4

- Additional DMRS:  In addition to the conventional DMRS of one symbol per frequency hop, an additional DMRS can be configured. It was observed that additional DMRS improves the performance especially with higher number of receiving antennas which is a typical case of NR base station. Hence we would like to have performance requirements defined with additional DMRS for PUCCH.

Proposal 4:  RAN4 should define performance requirements for PUCCH with additional DMRS per hop for long PUCCH

- Modulation and Coding Schemes:  We prefer to define the performance requirement with QPSK modulation and the coding scheme as RM code and polar code as defined in TS 38.212.

- Performance Metric:  BLER should be used as the performance metric for PUCCH format 3. For format 0 and 1, we prefer to probability of misdetection of ACK (NAK/DTX to ACK) and probability should be used.

# Conclusions

In this contribution we outlined our views on PUCCH demodulation requirements. Based on our observations we recommend

Proposal 1: RAN4 should define the performance requirements for PUCCH Formats 1, 3 and 0

Proposal 2: Due to the possibility of large channel bandwidths in NR, we would like to have performance requirements defined with frequency hopping enabled

Proposal 3: To match the coverage of LTE at low frequency bands, performance requirements should be defined for multi slot operation of long PUCCH with number slots repeated equal to 2 and 4

Proposal 4:  RAN4 should define performance requirements for PUCCH with additional DMRS per hop for long PUCCH

# References

[1]. R4-1805549, “Way forward on NR BS demodulation performance”, Huawei, Nokia, Nokia Shanghai Bell, Ericsson, Samsung

# Appendix

In this section we list the simulation assumptions for Figures 1-2.

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 700 MHZ for LTE and 1800 MHz for NR |
| Duplex | FDD |
| System Bandwidth | 10 MHz |
| Slot length | 1 ms |
| Subcarrier spacing | Single numerology case: 15KHz |
| FFT size | 1024 for 15KHz subcarrier spacing |
| Channel model | EPA, 3 Kmph |
| Spectral shaping filter | Not modeled |
| Antenna configuration | 1T NR, where N is 32 for NR and is equal to 2 for LTE |
| Rank per UE | Fixed rank |
| MCS | QPSK, and PI/2 BPSK |
| DM-RS | According to the specification – QPSK modulated |
| Channel estimation | MMSE |