3GPP TSG-RAN WG4 #87	R4- 1807466

Busan, Korea 21st April –25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.843: Section 10 OTA Timing Alignment Error |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding CATR test method procedure and calibration steps for OTA Timing Alignment Error |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA Occupied bandwidth requirement TX directional requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure is needed for OTA frequency error requirement with respect to the CATR test method. If not included, then CATR test method is excluded from this OTA requirement for conformance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | For a specific set of signals/transmitter configuration/transmission mode, the OTA Time Alignment Error (OTA TAE) is defined as the largest timing difference between any two different E-UTRA signals or any two different UTRA signals belonging to different reference symbols (e.g. CRS0 or CRS1) in the radiated domain. The OTA time alignment error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range. DUT is configured to generate the beams carrying reference signals (e.g. CRS0 and CRS1 of TX diversity or MIMO for E-UTRA etc). The reference signals are transmitted in different transmitter groups associated to logical antenna ports at physical layer. |  |  |  |  |  |  |  |  |

#### 10.2.6.3	CATR

##### 10.2.6.3.1	General

Figure 10.2.6.3.1-1: CATR measurement system setup for Timing Alignment Error (SA – Spectrum Analyzer or Signal Analyzer)

##### 10.2.6.3.2 	Calibration

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

- Path loss calibration C→A:

- Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓSGH.

- Measure cable loss from point C to input of SGH, call this LC↔SGH which is the equivalent of 20log|S21| from the use of a network analyser.

- Calculate the combined total path loss from C→A by using the following expression:

-	LSGHcal = LC,SGH + 10log(1 - |ΓSGH|2) - GSGH;

-	where10log1-ΓSGH2  10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss, GSGH  is the known gain of the reference SGH.

- Connect SGH and C↔A cable.

- To remove polarization(s) mismatch between range antenna (labelled as feeder antenna in diagram) and SGH use positions to position the SGH in the beam peak direction of range antenna.

- Measure path loss C→B with network analyzer LC→B = 20log|S21|.

- Calculate the test path loss compensation factor.  This is the total path loss between A↔B using the results from step 1c and 4. L = LSGHcal LSGHcal  - LC→B.

Where ΓSGH is the reflection coefficient (or mismatch) seen at the SGH connector (S11 with a network analyzer).

##### 10.2.6.3.3 	Procedure

Figure 10.2.6.3.1-1: CATR measurement system setup for Timing Alignment Error

- Align DUT with beam peak direction of range antenna.

- Configure DUT to transmit signals carrying reference signals. The beams containing reference signals must be pointed at the same direction. The narrowest beam shall be pointed to the mirror during the measurement.

Note: Transmitted signals are the set of signals/transmitter configurations/transmission modes or their combinations.

- Set the transmitted signal according to E-TM1.1 at middle channel.

Note: For TX diversity and MIMO transmission, different signal may be configured in E-TM (using p = 0 and 1), then different reference signals may be CRS0 and CRS1.

For an AAS BS declared to be capable of single carrier operation only, set DUT to transmit according to rated beam EIRP level .

If the AAS BS supports intra band contiguous or non-contiguous Carrier Aggregation, set DUT to transmit using the applicable test configuration and corresponding power setting specified in TS 37.145-2 subclauses 4.10 and 4.11.

If the AAS BS supports inter band carrier aggregation, set DUT to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in TS 37.145-2 subclauses 4.10 and 4.11.

- Measure the time alignment error between the reference signals with signal/spectrum analyser or equivalent equipment.