3GPP TSG-RAN WG4 Meeting #87			R4-1807490

Busan, South Korea, 21 - 25 May 2018

Source:	Sony

Title:	UE Spherical coverage measurements at mmWave 28GHz

Agenda item:	7.5.8.2

Document for:	Discussion

# Background

In this paper we continue presenting measurement result. In [1] Measurement on a prototype with front side + backside antenna panels were shown. This document is updated with measurement on front side only including glass cover and backside only, including plastic cover. For simulated results according to the assumptions in [3] please refer to [1].

# Measurement set-up

The measurement is done on an evaluation prototype in a smartphone UE size (slightly larger than 3GPP simulation assumption) with partial display (i.e. front side antenna is behind glass cover) and plastic back cover. A schematic outline is shown in Figure 1. Measurement is done with a CW signal and therefore only relative gain is reported. Antenna modules consists of patch elements in 2x2 matrices. A far field test method is used.  Measurements are reported for front side antenna only (glass cover), backside antenna only (plastic cover) and combined performance (best beam chosen). Each antenna panel has 5 discrete beams.

Figure 1. Schematic outline of measurement prototype.

# Result

Measured CDF EIRP spherical performance is shown in Figure 2 and result is listed in Table 1. From the data we make the following observations:

Observation 1: 	The gap between a single and two modules is bigger at the lower CDF points.

Observation 2: 	Implementation loss due to high-loss glass (Front-side) is 1 – 2 dB higher compared to plastic (Back-side) cover material which is consistent with simulations.

Figure 2. Measurements result (front glass, plastic back cover and combined).

| Assumption | - | 2-modules | Back-side only | Front-side only |
| --- | --- | --- | --- | --- |
| Display | Partial / Full | Partial |  |  |
| No antenna arrays | Back side | 1 | 1 | N/A |
|  | Front side | 1 | N/A | 1 |
| Side cover (cover material near antennas) | Metal / Plastic | Plastic | Plastic | Plastic |
| Back cover | Glass / Plastic | Plastic | Plastic | N/A |
| Front cover | Glass / Plastic | Glass | N/A | Glass |
| EIRP @ 100%%-tile point | dBm | 22.4dBm (agreement in #86bis [4]) |  |  |
| EIRP @ 50% | dB | 9.1 | 12.5 | 13.1 |
| EIRP @ 40% | dB | 10.1 | 14.2 | 14.7 |
| EIRP @ 30% | dB | 11.1 | 16.2 | 17.9 |
| EIRP @ 20% | dB | 12 | 19.1 | 21 |

Table 1. Summary of measured CDF spherical performance

# Conclusion

The spherical coverage of a handheld UE was measured on a smartphone evaluation prototype. The following observations were made:

Observation 1: 	The gap between a single and two modules is bigger at the lower CDF points.

Observation 2: 	Implementation loss due to high-loss glass (Front-side) is 1 – 2 dB higher compared to plastic (Back-side) cover material which is consistent with simulations.

# References

- R4-1805321	“UE Spherical coverage at mmWave 28GHz”, Sony

- R4-1802868	“UE Spherical coverage at mmWave 28GHz”, Sony

- R4-1801202	“WF on EIRP CDF for spherical coverage”, Samsung MediaTek

- RAN4#86bis Meeting Report, Agenda 7.4.8.2, 3GPP TSG-RAN WG4 Meeting #86 bis  (Melbourne Australia 16 – 20 April 2018)