3GPP TSG RAN WG 4 Meeting #87	R4-1807662

Busan, South Korea, 21 – 25 May 2018

Source:		Rohde & Schwarz

Title:	Estimation of DL SNR and AWGN levels for demodulation and CSI requirements in FR2

Agenda Item:		7.13.4.2

Document for:	Discussion

# Introduction

At the last meeting the link budget for FR2 demodulation test setups has been estimated in [1]. In this paper we present our analysis of the maximum SNR and AWGN level for FR2. It is necessary to understand the limitation due to real life test system implementation when defining the NR FR2 demodulation and CSI requirements. Otherwise requirements might be defined that end up being not testable.

# Discussion

## General

Based on the methodology presented in [2], we have analysed the SNR range for demodulation requirements.

As a starting point for the used test system, the permitted IFF and DFF RF test systems in [3] have been taken as a reference, since those in fulfill the requirements for the demodulation test system in [3] as well. While testing in the near field is a possibility for demodulation and CSI testing as well, reusing already existing RF test system should also be feasible.

Additionally the results in this paper take into account channel bandwidths of 100, 200 and 400 MHz in comparison to the 1 GHz in [2].

## Assumptions

Several assumptions need to be made regarding the test setup and the used signals so that the feasible SNR can be estimated correctly. An overview of the assumptions used in this paper can be seen in table 1 below.

| Type | Value |
| --- | --- |
| Crest factor of signal | 13.5 dB |
| Cable loss | @ 24GHz 1.6 dB/m |
|  | @ 43GHz 3.6 dB/m |
| Cable length | DFF: 4 m |
|  | IFF: 2 m |
| Test antenna gain | 12 dB |
| UE Noise figure | 10 dB |
| UE antenna gain | 6 dBi |

Table 1 Assumptions for SNR estimation

To achieve comparable results to those presented in [2], the assumptions on the test antenna gain, UE Noise figure and UE antenna gain have been reused.

The crest factor of the signal is estimated at 13.5 dB, based on measurements of a 200 MHz 5G signal with 120 kHz SCS. This is in contrast to the 10 dB used in [2], which in our view underestimates the crest factor.

## Results

Using the above assumptions and the methodology from [2], the following results can be calculated for the IFF and DFF systems at 24 and 43.5GHz.

Figure 1 SNR estimation for DFF system at 24 GHz

Figure 2 SNR estimation for IFF system at 24 GHz

Figure 3 SNR estimation for DFF system at 43.5 GHz

Figure 4 SNR estimation for IFF system at 43.5 GHz

From the graphs in figures 1 to 4 the feasible SNR ranges have been summarized in table 2 below, taking into account various levels of allowed SNR error.

| SNR error [dB] | Maximum SNR (400 MHz bandwidth) |  |  |  |
| --- | --- | --- | --- | --- |
|  | DFF system at 24 GHz | IFF system at 24 GHz | DFF system at 43.5 GHz | IFF system at 43.5 GHz |
| 0.1 dB | 29 dB | 32 dB | 16 dB | 23 dB |
| 0.3 dB | 34 dB | 37 dB | 20 dB | 28 dB |
| 0.5 dB | 36 dB | 39 dB | 23 dB | 30 dB |
| 1.0 dB | 39 dB | 42 dB | 26 dB | 33 dB |
| 2.0 dB | 43 dB | 46 dB | 30 dB | 37 dB |
| 3.0 dB | 45 dB | 48 dB | 32 dB | 39 dB |

Table 2 Maximum SNR for 400 MHz bandwidth

The table shows the maximum SNR values for 400 MHz channel bandwidth. The values for 100 and 200 MHz can be calculated from the values in the table by adding 6 dB for 100 MHz channel bandwidth and 3 dB for 200 MHz.

For LTE the AWGN has mostly been fixed at -98 dBm/15kHz, so it is not only needed to investigate what could be seen feasible as with regards to the SNR but also with regards to the AWGN level. The results are shown below in table 3, with the same assumptions as for the other results.

| AWGN error [dB] | Feasible AWGN levels (400 MHz bandwidth) |
| --- | --- |
| 0.5 dB | -68 dBm |
| 1.0 dB | -71 dBm |
| 2.0 dB | -74 dBm |
| 3.0 dB | -75 dBm |

Table 3 AWGN levels for 400 MHz bandwidth

Similarly as for table 2, the values for 100 and 200 MHz can be calculated by subtracting 6 dB for 100 MHz channel bandwidth and 3 dB for 200 MHz.

Further investigations on this topic will be necessary in the future to refined the values for SNR and AWGN, since factors like the noise figure of the test system amplifiers were not taken into account when deriving the above values. This might further limit the possible SNR and AWGN values to tighter numbers.

Observation: Further evaluations may be needed, further limiting the SNR and AWGN values.

However the above values show the absolute limits and those values need to be taken into account when discussing NR FR2 demodulation and CSI requirements.

Proposal: RAN4 takes the Maximum SNR and feasible AWGN levels into account when discussing NR FR2 demodulation and CSI requirements.

# Proposals

# In this paper we have analysed the SNR range and AWGN noise floor for FR2 demodulation and CSI tests, taking into account the implementation of these tests on the permitted IFF and DFF RF test systems.

The observations made in this paper and in [2] should be captured in TR 38.810.

Observation: Further evaluations may be needed, further limiting the SNR and AWGN values.

Proposal: RAN4 takes the Maximum SNR and feasible AWGN levels into account when discussing NR FR2 demodulation and CSI requirements.

# References

- R4-1805899: Way Forward on Proposals on Concluding the SI; Intel, CATR; RAN4#86bis Melbourne; April 2018

- R4-1804667: Link budget considerations for NR FR2 demodulation test setup; Intel, CATR; RAN4#86bis Melbourne; April 2018

- TR 38.810 v.2.1.0