3GPP TSG-RAN WG4 Meeting #87	R4-1807748

## Busan, Korea, 21 - 25 May, 2018

Source: 	Huawei

Title:	Discussion on necessity of the MSR EMC specification

Agenda Item:	7.9 (NR BS EMC)

Document for:	Discussion

# Introduction

During RAN4#86bis meeting, discussion was held on the NR inclusion into the MSR EMC and AAS EMC specifications, as this is required by the NR WID [1]. During offline discussion, some companies shared the view on the necessity of the MSR EMC specification.

In this contribution we are briefly evaluating the need for the MSR EMC specification TS 37.113, based on the comparison of its content with the single RAT E-UTRA EMC specification in TS 36.113. We also refer to the structure of the AAS EMC and provide the suggested way forward.

# Discussion

The MSR EMC specification in TS 37.113 [2] covers the assessment of E-UTRA, UTRA, GSM/EDGE and NB-IoT Multi-Standard Radio (MSR) Base Stations and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC). TS 37.113 specifies the applicable test conditions, performance assessment and performance criteria for E-UTRA, UTRA, GSM/EDGE and NB-IoT Base Stations and associated ancillary equipment in one of the following categories:

-	Multi-Standard Radio (MSR) Base Stations for E-UTRA, UTRA and GSM/EDGE meeting the requirements of TS 37.104 [6], with conformance demonstrated by compliance to TS 37.141 [11].

-	Base Stations for E-UTRA meeting the requirements of TS 36.104 [4], with conformance demonstrated by compliance to TS 36.141 [9].

-	Base Stations for UTRA FDD meeting the requirements of TS 25.104 [2], with conformance demonstrated by compliance to TS 25.141 [7].

-	Base Stations for UTRA TDD meeting the requirements of TS 25.105 [3], with conformance demonstrated by compliance to TS 25.142 [8].

-	Base Stations for GSM/EDGE meeting the requirements of TS 45.005 [5], with conformance demonstrated by compliance to TS 51.021 [10].

- 	Base Stations for NB-IoT meeting the requirements of TS 36.104 [4], with conformance demonstrated by compliance to TS 36.141 [9].

In addition to MSR base stations, the present document covers other BS supporting more than one RAT.

## Comparison of the MSR EMC and E-UTRA BS EMC specifications

Based on the analysis of the MSR EMC TS 37.113 and E-UTRA BS EMC TS 36.113 [3] specifications, the following observations were made:

- Clause 4 (Test conditions): The MSR specification provides additional (over the SRAT EMC specifications) information on test coverage, e.g.

  - “Tests shall be performed relating to each type of port, and need not be repeated for each RAT if operating RATs are assessed simultaneously during the test.” This information saves testing effort. This type of information can be only kept in the MSR EMC specification and does not belong to the SRAT EMC specifications.

- Subclause 4.2 (Arrangements for establishing a communication link): even though RAT-specific aspects were captured in this subclause, which can be seen as redundant from SRAT EMC point of view, it is observed that information on GSM/EDGE is not provided elsewhere,

- Subclause 4.5 (BS test configurations): defines the BS test configurations that shall be used for demonstrating conformance depending on the MSR capability sets declared for the BS. This information is specific to the MSR specifications and not available in SRAT EMC specs.

- Clause 5 (Performance assessment): despite consideration of multiple RATs and their specific Performance criteria, a lot of text of this clause is shared with the legacy specifications.

- Clause 7 (Applicability): those clauses in MSR EMC and EUTRA EMC specs are almost the same.

- Clause 8 (Emission requirements): great parts of those sections in MSR EMC and EUTRA EMC are the same.

- Clause 9 (Immunity requirements): great parts of those sections in MSR EMC and EUTRA EMC are the same.

Considering the above list, it is seen that EMR EMC specification shall not be considered as the redundant specification, comparing to the single RAT EMC specifications.

Observation 1: there were multiple items identified in the TS 37.113, which provide valid and important information for the MSR product conformance testing, e.g. the BS test configurations selection depending on the MSR Capability Set.

Proposal 1: keep maintaining the MSR EMC specification in RAN4, due to value added information available in the general sections, e.g. BS test conditions.

Still, referring to the above bullets, multiple clauses share great amount of the repeated text, e.g. clauses 7, 8, 9. From the maintenance point of view, there is room for improvements.

Proposal 2: in order to reduce the work overlaps, refer from the MSR EMC specification to the SRAT EMC specifications, where possible.

Details of potential inter-spec optimisations are out of scope of this specification and are FFS.

## AAS EMC specification

The AAS BS EMC specification covers the UTRA, EUTRA and MSR.

Referring to the AAS BS EMC specification in TS 37.114, one can notice that there are multiple references to the MSR EMC specification TS 37.113, in order to reuse its content. Please note that that first release f the AAS EMC specification in TS 37.114 was basically purely based on cross-references to other RAN4 EMC specifications in TS25.113, TS36.113 and TS37.113.

Potential withdrawal of TS 37.113 would require to first fix the other RAN4 EMC specifications and check for potential cross-referencing.

Observation 2: Potential withdrawal of TS 37.113 would require to first fix the other RAN4 EMC specifications and check for potential cross-referencing.

## EMC for GSM

TS 37.113 MSR EMC specification covers E-UTRA, UTRA, GSM/EDGE and NB-IoT Multi-Standard Radio (MSR) BS. In case of the potential withdrawal of the TS 37.113 MSR EMC specification, it would be required to rely on the single RAT BS EMC specifications:

- For the E-UTRA the TS 36.113 is available,

- For the UTRA the TS 25.113 is available (covering both FDD and TDD),

- For the GSM/EDGE there is no EMC specification defined in the 45.xxx series (GSM only (Rel-4 and later)), neither in 05 series (GSM only (before Rel-4)),

- For the NB-IoT, it is covered by the TS 36.113 specification (which covers E-UTRA, E-UTRA with NB-IoT or NB-IoT base stations).

Observation 3: removal/withdrawal of the TS 37.113 would create inconsistency when comparing with the RF specification, i.e. the RF specification would cover the GSM/EDGE, while it would not be addressed by the EMC specifications in RAN4.

# Conclusion

Based on brief analysis of the validity of the MSR EMC specification in RAN4, the following was observed:

Observation 1: there were multiple items identified in the TS 37.113, which provide valid and important information for the MSR product conformance testing, e.g. the BS test configurations selection depending on the MSR Capability Set.

Observation 2: Potential withdrawal of TS 37.113 would require to first fix the other RAN4 EMC specifications and check for potential cross-referencing.

Observation 3: removal/withdrawal of the TS 37.113 would create inconsistency when comparing with the RF specification, i.e. the RF specification would cover the GSM/EDGE, while it would not be addressed by the EMC specifications in RAN4.

It was found that the TS 37.113 shall not be removed from RAN4, as it would invalidate certain amount of MSR specific information captured in this specification:

Proposal 1: keep maintaining the MSR EMC specification in RAN4, due to value added information available in the general sections, e.g. BS test conditions.

On the other hand, there are multiple clauses which are repeated among the EMC specifications in RAN4 and its proposed to look into the text redundancy limitation among the EMC specs in RAN4:

Proposal 2: in order to reduce the work overlaps, refer from the MSR EMC specification to the SRAT EMC specifications, where possible.

Interested companies are encouraged to provide feedback on the above proposed approach.

# References

[1]	RP-180536	Work Item on New Radio (NR) Access Technology, RAN#79

[2] 3GPP TS 37.113, v 15.2.0

[3] 3GPP TS 36.113, v 15.2.0