3GPP TSG-RAN WG4 Meeting #87	R4-1807804

Busan, Korea, 21st – 25th May 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On spherical Fibonacci sampling grid for numerical TRP approximations

Agenda item:	6.28.3.1.4

Document for:	Discussion

# 1	Introduction

During the RAN4#86 meeting, the WF on OTA TRP measurement grids [1] was approved, which outlined a list of open issues to be addressed. One of the open issues is numerical integration methods for TRP estimation. The WF on TRP measurement grids [2] was approved, which captured agreements that were made during the last RAN4#86bis meeting.

This document investigates a spherical Fibonacci sampling grid which generates uniform point distribution on the spherical surface for numerical integration of an AAS BS TRP. Refer to [3][4] for discussions on numerical TRP approximation that is based on the spherical equal angle and equal area sampling grids, respectively.

# 2	Discussion

From [2], the following terms and definitions are defined

| Term | Definition |
| --- | --- |
| TRPReference | theoretical TRP |
| TRPEstimate | numerically approximated TRP |
| TRP systematic error | The difference between the theoretical TRP and numerically estimated TRP |

In the far field region, TRPReference can be expressed as

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ															(1)

Spherical equal angle sampling grid:

With the spherical equal angle sampling grid, the elevation angle interval [0, π] and azimuth angle interval 0,2π are discretized into equally spaced angular subintervals. Figure 1 shows the distribution of points on the spherical surface. The numerical approximation of Equation (1) is

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn																(2)

Spherical equal area sampling grid:

With the spherical equal angle sampling grid, the spherical surface is partitioned into N equal area regions. Let n be the index for the nth region and there is one point (θn,ϕn) located in the centre of each region. Figure 2 shows the uniform distribution of points on the spherical surface. The TRP integral in Equation (1) can be approximated as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)  																				(3)

Spherical Fibonacci sampling grid:

With the spherical Fibonacci sampling grid, the points on the spherical surface are arranged along a generative spiral as shown in Figure 3. Similar to the equal area sampling grid, the Fibonacci grid generates points that are uniformly spaced in an isotropic way on the spherical surface. Let us assume there are N points in the Fibonacci sampling grid, the spherical coordinates of the nth point are [5]

θn=cos-11-2i+1N, where		n ∈[0, N-1]

ϕn=2πiΨ,	where 	Ψ=1+52

The numerical approximation of Equation (1) is

TRPEstimate=1Nn=1NEIRP(θn,ϕn)  																				(4)

Figure 1: Spherical equal angle sampling grid

Figure 2: Spherical equal area sampling grid

Figure 3: Spherical Fibonacci sampling grid

# 3	Conclusions

We have discussed the approximation of TRP based the spherical Fibonacci sampling grid. The numerical TRP approximation is

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

# References

- R4-1803392, WF on OTA TRP conformance requirements, Ericsson

- R4-1805865, WF on TRP measurement grids, Nokia, Nokia Shanghai Bell

- R4-1805391, On spherical equal angle sampling grid for numerical TRP, Nokia, Nokia Shanghai Bell

- R4-1805392, On spherical equal area sampling grid for TRP estimation, Nokia, Nokia Shanghai Bell

- B. Keinert, et al., Spherical Fibonacci Mapping, ACM Trans. On Graphics, 2015