3GPP TSG-RAN WG4 Meeting #87                	R4-1807895

Busan, Korea (republic of), May 21st – 25th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurements of intra-frequency and inter-frequency NR cells in NR idle mode

Agenda item:	7.10.6.2

Document for:	Discussion

Introduction

In the previous meetings, RAN4 has discussed the measurement requirements for intra-frequency and inter-frequency NR cells in NR idle mode. Latest updates were agreed in the Melbourne meeting in [1] and [2]. However, the detection time (Tdetect,NR_Intra and Tdetect_NR_Inter) is still open. In this contribution we discuss the detection time requirements in NR idle mode.

Discussion

In the Melbourne meeting, RAN4 has introduced the measurement requirements of intra-frequency and inter-frequency NR cells in NR idle mode as below:

Intra-frequency: Table 4.2.2.3-1 : Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_intra [s] (number of DRX cycles). 0.32; [TBD]; 1.28 (4); 5.12(16). 0.64; [TBD]; 1.28 (2); 5.12 (8). 1.28; [TBD]; 1.28 (1); 6.4 (5). 2.56; [TBD]; 2.56 (1); 7.68 (3). Inter-frequency: Table 4.2.2.4-1 : Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_inter [s] (number of DRX cycles). 0.32; [TBD]; 1.28 (4); 3.84(16). 0.64; [TBD]; 1.28 (2); 3.84 (8). 1.28; [TBD]; 1.28 (1); 3.84 (5). 2.56; [TBD]; 2.56 (1); 7.68 (3).

The time TDetect,NR_Intra and TDetect,NR_Inter for NR intra-frequency and inter-frequency respectively, within which the UE shall be able to evaluate whether a newly detectable cell meets the reselection criteria.

As behavior in NR FR1 is similar to E-UTRAN, we can propose to reuse E-UTRAN cell detection latencies also in NR FR1, the intra-frequency and inter-frequency requirements for NR idle mode in FR1 are specified in Table 1 and Table 2.

Table1 : Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra (FR1)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12(16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Table 2 : Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter (FR1)

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 3.84(16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 3.84 (8) |
| 1.28 | 32(25) | 1.28 (1) | 3.84 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

- Reuse E-UTRAN definition of detection time for NR intra-frequency and inter-frequency cells in FR1 in NR idle mode.

Proposals 1 is included in our CR [3].

Conclusion

In this contribution we have discussed the detection time requirements of intra-frequency and inter-frequency NR cells in NR idle mode. We have made the following proposals:

- Reuse E-UTRAN definition of detection time for NR intra-frequency and inter-frequency NR cells in NR idle mode.

Proposals 1 is included in our CR [3].

References

- R4-1805542, CR on intra-frequency measurement requirements in idle mode, Huawei, HiSilicon

- R4-1805543, CR on inter-frequency measurement requirements in idle mode, Huawei, HiSilicon

- R4-1807896, CR for 38.133 measurement in intra-F and inter-F in idle mode, Nokia, Nokia Shanghai Bell