3GPP TSG-RAN WG4 Meeting #87	R4-1807977

Busan, Korea (Republic of), 21 – 25 May 2018

Agenda Item:	6 (6.20.6, 6.20.7, 5.3.4, 6.19.6, 6.19.7, 5.6.3, 6.22.6, 6.22.7)

Source:	Ericsson

Title:	Ad hoc meeting minutes for LTE UE/BS demodulation for multiple features

Document for:	Approval

# 1	BS demodulation requirements

## 1.1	eFeMTC BS demod (Rel-15)

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.20.7 | R4-1806423 | discussion | Discussion and simulation results for eFeMTC | Samsung |
| 6.20.7 | R4-1806424 | discussion | Discussion on BS demodulation requirements for eFeMTC | Samsung |
| 6.20.7 | R4-1806760 | other | Discussion on BS demodulation requirements for eFeMTC | Ericsson |
| 6.20.7 | R4-1807468 | discussion | On BS demodulation requirements for eFeMTC | Huawei, CATR |

Summary of proposals:

R4-1806423 (Samsung):

- Proposal 1: In demodulation tests for PUSCH with CE Mode A, ETU70 can be regarded as the option the Doppler option for propagation condition

R4-1806424 (Samsung):

- Proposal 1: In demodulation tests for PUSCH with sub-PRB allocation 2/3 Pi/2 BPSK for FDD, the number of repetition with different RU allocation number is

  - CE Mode A , 1 or 2 repetition number for 1RU allocation, 1 repetition for 2 RU allocation

  - CE Mode B, 16 repetition for 2 RU allocation,  8 repetition for 4RU allocation

- Proposal 2: Reuse the legacy frequency hopping pattern in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

- Proposal 3: Reuse the legacy number of antennas in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

- Proposal 4: Reuse the legacy system bandwidth 3MHz,5MHz,10MHz,15MHz,20MHz in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

- Proposal 5: Reuse the legacy values for maximum number of HARQ transmissions (4), and RV sequence (0, 2, 3, 1) in PUSCH with sub-PRB allocation 2/3 Pi/2 BPSK scheme in CE Mode A for FDD

- Proposal 6: Define new FRC for different RU size with sub-PRB allocation

- Proposal 7: In demodulation tests for PUSCH with sub-PRB allocation, we propose to use the parameters as shown in Table 3.

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Fraction of maximum throughput | 70 % |
| RV sequences | 0,2,3,1,0,2,3,1 |
| Maximum number of HARQ transmissions | 4 |
| Reference receiver | MRC |
| Channel bandwidth | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |
| RU size | CE Mode A: 1RU, 2RU, [4RU] CE Mode B: 2RU, 4RU |
| Repetition Number | CE Mode A: 1,2 for 1RU, 1 for 2RU, [1 for 4RU] CE Mode B: 16 for 2RU, 8 for 4RU |
| DMRS | Fixed one |
| Frequency hopping | On |
| * Reduce the number of test cases by testing all BWs, but not testing all combinations of these parameters. |  |

R4-1806760 (Ericsson):

- Proposal 1: RAN4 should wait for RAN1 decision on MCS/TBS regarding sub-PRB resource allocation.

- Proposal 2: RAN4 will specify the PUSCH demodulation requirements with the following setting: 1x2 EPA200, 8 PUSCH repetitions, FRC A3-2 (QPSK 1/3, TBS=600bit, 6PRB).

R4-1807468 (Huawei):

- Proposal 1: For Rel-15 eFeMTC, introduce the new CE Mode B PUSCH demodulation performance requirement with 2-of-3 subcarrier pi/2 BPSK under ETU1 for bandwidth 3~20MHz, which are generic for FDD/HD-FDD and TDD.

- Proposal 2: For Rel-15 eFeMTC, introduce the new CE Mode B PUSCH demodulation performance requirement with 6 subcarriers QPSK.

- Proposal 3: For Rel-15 eFeMTC, introduce the new CE Mode A PUSCH demodulation performance requirements under EPA200 for bandwidth 3~20MHz, which are generic for FDD/HD-FDD and TDD.

Issues for the discussion:

PUSCH demodulation with high speed condition

Test setting:

- PUSCH CEModeA

- FDD/HD-FDD and TDD

- 8 PUSCH repetitions

- FRC A3-2 (QPSK 1/3, TBS=600bit, 6PRB).

- Bandwidth 3, 5, 10, 15, and 20MHz

- Propagation channel:

  - Option 1: EPA200

  - Option 2: ETU70

Comments:

Samsung: Existing BS demodulation requirements are ETU. We want to follow it. Prefer to lower Doppler shift, 300 or 600Hz is too high. For 900MHz bands, 120km/h corresponding to 100Hz.

Huawei: Don’t strong view on ETU or EPA. But EPA is preferable.

Samsung: Following the existing LTE condition, prefer ETU.

Huawei: How about ETU200 as compromise?

Samsung: ETU200 is OK.

Agreement: ETU200

PUSCH with SubPRB:

- 2 of 3 subcarriers

  - CE Mode A, 1 or 2 repetition number for 1RU allocation, 1 repetition for 2 RU allocation

  - CE Mode B, 16 repetitions for 2 RU allocation, 8 repetitions for 4RU allocation

  - CE Mode B PUSCH demodulation performance requirement with 2-of-3 subcarrier pi/2 BPSK under ETU1 for bandwidth 3~20MHz, which are generic for FDD/HD-FDD and TDD.

- 6 carriers

  - CE Mode B PUSCH demodulation performance requirement with 6 subcarriers QPSK

Note: RAN1 is discussing MCS/TBS for subPRB PUSCH

Comments:

Ericsson: We propose to CEModeA for 2 of 3 subcarriers. Open for 6 subcarriers.

Huawei: Need to declare the applicability.

Samsung: Propose to focus on FDD case. Keep open for 3 subcarrier cases.

## 1.2	FeNB-IoT BS demod (Rel-15)

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.19.7 | R4-1807901 | discussion | Discussion on BS further NB-IoT enhancements demodulation requirements | Huawei |
| 6.19.7 | R4-1807902 | other | Way forward for FeNB-IOT BS demodulation performance requirements | Huawei |

Summary of proposals:

R4-1807901 (Huawei):

- Proposal 1: Use the similar simulation assumptions as preamble 0 and 1 defined for NB-IoT Release 13 for preamble format 2 as shown below:

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -2.1 | -2.1 | TBD |
|  |  |  | EPA1 Low | 200 Hz | 6.1 | 6.1 | TBD |
|  |  | 32 | AWGN | 0 | -6.8 | -6.8 | TBD |
|  |  |  | EPA1 Low | 200 Hz | 0.5 | 0.5 | TBD |

- Proposal 2: Use the similar simulation assumptions as NPRACH FDD for NPRACH TDD for different preamble formats as shown in Table 2.2-2.

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix | Frequency offset | Number of Repetitions |  | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | AWGN | 0 | 8 | TBD | TBD | TBD | TBD | TBD- |
|  |  |  |  | 32 | TBD | TBD | TBD | TBD | TBD |
|  |  | EPA1 Low | 220 Hz | 8 | TBD | TBD | TBD | TBD | TBD |
|  |  |  |  | 32 | TBD | TBD | TBD | TBD | TBD |

Issues for the discussion:

Comments:

Huawei: Want to put Frequency offset in [], like [220].

Samsung: How to consider frequency hopping for FDD?

Huawei: Reuse the existing FDD NPRACH frequency hopping pattern.

Samsung: RAN1 is discussing a new hopping pattern for FDD long range and TDD. We should wait for RAN1 decision. Want to reduce number of test cases.

Agreements:

Huawei to ask revision of WF R4-1807902.

## 1.3	sTTI BS demod (Rel-15)

### 1.3.1	Summary

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.22.6 | R4-1806164 | discussion | results summary for sPUCCH and sPUSCH | Ericsson |

Comments:

Huawei: Not sure results for PUSCH are based on which DMRS pattern.

### 1.3.2	SPUCCH

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.22.6.2 | R4-1806161 | discussion | Simulation results for sPUCCH | Ericsson |
| 6.22.6.2 | R4-1806162 | CR | Performance requirements for SPUCCH | Ericsson |
| 6.22.6.2 | R4-1806422 | discussion | Simulation results for SPUCCH | Samsung |
| 6.22.6.2 | R4-1807757 | other | Simulation results for SPUCCH | Nokia, Nokia Shanghai Bell |
| 6.22.6.2 | R4-1807904 | discussion | Discussion and simulation results on sTTI BS SPUCCH demodulation requirements | Huawei |

Summary of proposals:

Simulation results only

Issues for the discussion:

Ericsson: We have 3dB difference. Encourage companies to identify the reason. Need more alignment work.

Samsung: Suggest to align 2OS and 3OS separately first.

Ericsson: Good approach. But we need first see the results from all the companies.

### 1.3.3	PUSCH

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.22.6.1 | R4-1806160 | discussion | Simulation results for sPUSCH | Ericsson |
| 6.22.6.1 | R4-1806163 | CR | Performance requirements for SPUSCH | Ericsson |
| 6.22.6.1 | R4-1807756 | discussion | Simulation results and remaining parameters for SPUSCH | Nokia, Nokia Shanghai Bell |
| 6.22.6.1 | R4-1807903 | discussion | Discussion and simulation results on sTTI BS SPUSCH demodulation requirements | Huawei |

Summary of proposals:

R4-1806160 (Ericsson):

- For option 1, the DMRS overhead is smaller compared with option 2. However, option 2 is a good balance between overhead and performance. Further, since option 2 is the DMRS pattern used for SPS and SPS is quite useful for the sTTI usage scenarios. Thus, we slightly prefer option 2.

R4-1807756 (Nokia):

- Proposal 1: Use Option 1 RDD DD DD RD DD RDD as the DMRS pattern for SPUSCH demodulation tests.

R4-1807903 (Huawei):

- Proposal1: Use DMRS sharing pattern “RDD DD DD RD DD RDD” for subslot PUSCH performance requirements.

Issues for the discussion:

DMRS pattern:

- Option 1 (RDD DD DD RD DD RDD)

- Option 2 (RDD DD | R RD DD | R RD RDD)

Comments:

Ericsson: we are ok to option 1.

Samsung: For Huawei’s results, found 2dB performance difference from the last meeting.

Huawei: Need double check

Comments for CR R4-1806163

Huawei: Need check.

Agreements:

DMRS pattern: Option 1 (RDD DD DD RD DD RDD)

# 2	UE demodulation and CSI requirements

## 2.1	eFeMTC UE demod (Rel-15)

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.20.6 | R4-1806277 | discussion | Discussion on eFeMTC UE demodulation performance requirements | Intel Corporation |
| 6.20.6 | R4-1806758 | other | Discussion on UE demodulation requirements for eFeMTC | Ericsson |
| 6.20.6 | R4-1806759 | other | Simulation assumption of UE demodulation requirements for eFeMTC | Ericsson |
| 6.20.6 | R4-1807467 | discussion | On UE demodulation requirements for eFeMTC | Huawei, CATR, HiSilicon |

Summary of proposals:

R4-1806277 (Intel):

- Proposal 1: In RAN4 eFeMTC WI, new demodulation/CSI feedback requirements will be introduced to BL/CE UE only.

- Proposal 2: No new demodulation/CSI feedback requirements will be introduced to non-BL/CE UE in RAN4 eFeMTC WI.

R4-1806758 (Ericsson):

- Proposal 1: No demodulation requirement due to the flexible PDSCH resource allocation.

- Proposal 2: RAN4 introduces new UE demodulation and CQI reporting requirements for non-BL UEs. FFS which test cases are applicable for non-BL UEs.

- Proposal 3: RAN4 revises the PBCH demodulation requirements for multiple TTI scenario based on the enhanced MIB decoder.

- Proposal 4: Introduce CQI reporting requirements with Table 2. Test point(s) are chosen to that UE report CQI indices 11 or more. Applicability is only for Cat-M1/M2 CE Mode A.

- Proposal 5: Introduce CQI reporting requirements with Table 1 (option B CQI table). Test point(s) to be chosen so that UE reports 1) CQI indices 3 or 4, and 2) 11 or 12. Applicability for both Cat-M1/M2 and higher category UE with/without 64QAM.

R4-1806759 (Ericsson):

- We propose the simulation assumption for eFeMTC:

  - DL 64QAM

    - TM6, 64QAM 1/2 (TBS=776bits with 2PRB), EPA5 2x1 Low

    - Applicability: UE category M1 and M2 capable of 64QAM

  - High velocity UE

    - TM2, QPSK 1/3, (TBS=504bits with 6PRB), EPA200, 2x1 Low

    - CE Mode A, 8 repetitions without frequency hopping

    - Applicability: UE category M1, M2, non-BL UEs

  - CRS muting

    - Applicable test cases (FDD):

    - 8.11.1.1.1.1 Test 1 (TM6, 16QAM 1/2, 3PRB, CE Mode A, No repetitions)

    - 8.11.1.1.2.1 Test 1 (TM9, QPSK 1/3, 6PRB, CE Mode A, 8 repetitions)

    - 8.11.1.1.3.1 Tests 1 (TM2, QPSK 1/10, 6PRB, CE Mode B, 64 repetitions)

    - 8.11.1.1.3.1 Tests 2 (TM2, 16QAM 1/2, 3PRB, CE Mode A, No repetitions)

    - 8.11.1.1.3.2 Tests 1 (TM2, QPSK 1/3, 18PRB, CE Mode A, 8 repetitions)

    - 8.11.1.1.3.2 Tests 2 (TM2, QPSK 1/10, 18PRB, CE Mode B, 32 repetitions)

    - No new simulation results are needed.

R4-1807467 (Huawei):

- Proposal 1: For Rel-15 eFeMTC, introduce TM6 64QAM test for CE Mode A with 3PRB allocation, TBS=968 and under EPA5 2x1 low.

- Proposal 2: For Rel-15 eFeMTC, introduce CQI reporting test point at high SNR level to incorporate 64QAM CQI index.

- Proposal 3: For Rel-15 eFeMTC, introduce CRS muting test with only center 6PRB CRS transmitted, using QPSK modulation in EPA5 channel of 2x1 low.

- Proposal 4: For Rel-15 eFeMTC, introduce PDSCH transmit diversity performance requirements in CE Mode A with TM2 QPSK 1/2 under EPA200.

- Proposal 5: Do not introduce new PBCH demodulation requirement.

Issues for the discussion:

DL 64QAM

- Option 1: TM6 64QAM test for CE Mode A with 3PRB allocation, TBS=968 and under EPA5 2x1 low.

- Option 2: TM6, 64QAM 1/2 (TBS=776bits with 2PRB), EPA5 2x1 Low

Comments:

Huawei: Prefer to use larger TBS (maximum size) considering the 64QAM.

Qualcomm: Want to TM open:

- Option 1: TM6

- Option 2: TM2

Agreement: 64QAM test for CE Mode A with 3PRB allocation, TBS=968 and under EPA5 2x1 low.

High velocity UE

- Option1: TM2, QPSK 1/3, (TBS=504bits with 6PRB), EPA200, 2x1 Low, CE Mode A, 8 repetitions without frequency hopping

- Option 2: CE Mode A with TM2 QPSK 1/2 under EPA200.

Comments:

Huawei: QPSK 1/3 is also fine for higher Doppler.

Qualcomm: Prefer to no repetition case (TM2).

Repetition:

- Option 1: No repetition

- Option 2: 8 repetitions

CRS muting

- Option 1:

  - Cat-M1

    - 8.11.1.1.1.1 Test 1 (TM6, 16QAM 1/2, 3PRB, CE Mode A, No repetitions)

    - 8.11.1.1.2.1 Test 1 (TM9, QPSK 1/3, 6PRB, CE Mode A, 8 repetitions)

    - 8.11.1.1.3.1 Tests 1 (TM2, QPSK 1/10, 6PRB, CE Mode B, 64 repetitions)

    - 8.11.1.1.3.1 Tests 2 (TM2, 16QAM 1/2, 3PRB, CE Mode A, No repetitions)

  - Cat-M2

    - 8.11.1.1.3.2 Tests 1 (TM2, QPSK 1/3, 18PRB, CE Mode A, 8 repetitions)

    - 8.11.1.1.3.2 Tests 2 (TM2, QPSK 1/10, 18PRB, CE Mode B, 32 repetitions)

- Option 2: only center 6PRB CRS transmitted, using QPSK modulation in EPA5 channel of 2x1 low.

Comments:

Huawei/Qualcomm: Do you want to introduce new 6 test cases? Prefer to reduce the number of test cases.

Ericsson: OK to reduce the number of test cases.

PBCH demodulation assuming cross-TTI decoding

- Option 1: Introduce new requirements

- Option 2: Do not introduce new requirements

Applicability of Rel-15 requirements:

- Option 1: Only applicable to Cat-M1 and Cat-M2

- Option 2: Some requirements are also applicable to higher category UE (like Cat-1) with coverage enhancement.

CQI reporting test:

- CQI reporting requirements with the CQ table below. Test point(s) are chosen to that UE report CQI indices 11 or more.

Comments:

Agreement: CQI definition with the CQ table below. Test point(s) are chosen so that UE report CQI indices 11 or more.

| CQI index | modulation | code rate x 1024 x | efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | 64QAM | 466 | 2.7305 |
| 12 | 64QAM | 567 | 3.3223 |
| 13 | 64QAM | 666 | 3.9023 |
| 14 | 64QAM | 772 | 4.5234 |
| 15 | 64QAM | 873 | 5.1152 |

- Whether or not to introduce CQI reporting requirements with the Option B CQI table (See below). Test point(s) to be chosen so that UE reports 1) CQI indices 3 or 4, and 2) 11 or 12.

Comments:

Qualcomm: One way is to test the SNR point for QPSK or 16QAM.

Agreement: CQI definition with the CQ table below. Test point(s) are chosen so that UE report CQI indices for QPSK or 16QAM

| CQI index | modulation | code rate x 1024 | Repetition |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 56 | 32 |
| 2 | QPSK | 207 | 16 |
| 3 | QPSK | 266 | 4 |
| 4 | QPSK | 195 | 2 |
| 5 | QPSK | 142 | 1 |
| 6 | QPSK | 266 | 1 |
| 7 | QPSK | 453 | 1 |
| 8 | QPSK | 637 | 1 |
| 9 | 16QAM | 423 | 1 |
| 10 | 16QAM | 557 | 1 |
| 11 | 16QAM | 696 | 1 |
| 12 | 16QAM | 845 | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |

Agreements:

Huawei to draft way forward on eFeMTC UE demod/CSI.

## 2.2	eMTC demod (Rel-13)

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 5.3.4 | R4-1806873 | CR | Correction to eMTC subband CQI test R13 | Qualcomm Incorporated |
| 5.3.4 | R4-1806874 | CR | Correction to eMTC subband CQI test R14 | Qualcomm Incorporated |
| 5.3.4 | R4-1806875 | CR | Correction to eMTC subband CQI test R15 | Qualcomm Incorporated |

Summary of proposals:

There is a mismatch in the test parameter in subband CQI test for eMTC UE. R^CSI is set to 4 while PDSCH repetition is set to 1. This means that a spec-compliant UE that reports the correct subband CQI may fail the test since the CQI computed based on repetition level of 4 being too agresstive when transmitted without any repetition.

R^CSI is changed to 1.

Agreements:

CR is agreeable.

## 2.3	FeNB-IoT demod (Rel-15)

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.19.6 | R4-1807899 | discussion | Discussion on UE further NB-IoT enhancements demodulation requirements | Huawei, HiSilicon |
| 6.19.6 | R4-1807900 | other | Way forward for FeNB-IOT UE demodulation performance requirements | Huawei, HiSilicon |

Summary of proposals:

R4-1807899 (Huawei):

- Proposal 1: Reuse the NPBCH FDD performance requirements for NPBCH TDD, but with RMC and test parameters updates for TDD.

- Proposal 2: Not define performance requirements for the scenarios with the special subframe configuration 0 and 5 that the number of OFDM symbols in DwPTS equals to 3;

- Proposal 3: Select special subframe configuration #4 for the scenario that the number of OFDM symbols in DwPTS is larger than 3 for the related NPDSCH TDD demodulation performance evaluation:

  - Consider to add a certain margin [0.5]dB on top of the existing NPDCH FDD performance requirements, or

  - Wait for RAN1 agreements about how to handle transmission in special subframe for further evaluation.

- Proposal 4: Adopt the similar approach as NPDSCH TDD for NPDCCH TDD demodulation performance requirements evaluation.

Way forward (R4-1807900):

- NPBCH - Reuse the NPBCH FDD performance requirements for NPBCH TDD, but with RMC and test parameters updates for TDD - NPDSCH - Not define performance requirements for the scenarios with the special subframe configuration 0 and 5 that the number of OFDM symbols in DwPTS equals to 3 - Select special subframe configuration #4 for the scenario that the number of OFDM symbols in DwPTS is larger than 3 for the related NPDSCH TDD demodulation performance evaluation. - Consider to add a certain margin [0.5]dB on top of the existing NPDCH FDD performance requirements - NPDCCH - Adopt the similar approach as NPDSCH TDD for NPDCCH TDD demodulation performance requirements evaluation.

Comments:

Qualcomm: For proposal 1, OK to reuse the configuration. But not sure the required SNR for NPBCH for TDD is same as HD-FDD. We need to check by running simulation.

Huawei: Not sure channel estimation is different between TDD and HD-FDD.

Qualcomm: NPBCH FDD is transmitted in SF#0 and NSSS is available in SF#9, but NPBCH TDD is transmitted in SF#9 and there is no guarantee NRS is transmitted in SF#8.

Qualcomm: For proposal 3, prefer to wait for RAN1 agreements.

Huawei: Agree to wait for RAN1 agreement.

Qualcomm: For proposal 4, can you elaborate?

Huawei: Same as proposal 3.

Agreements:

Huawei to revise the WF.

## 2.4	eNB-IoT demod (Rel-14)

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 5.6.3 | R4-1806752 | CR | Removal of square brackets from eNB-IoT UE demodulation requirements | Ericsson |
| 5.6.3 | R4-1806753 | CR | Removal of square brackets from eNB-IoT UE demodulation requirements | Ericsson |

Summary of proposals:

Removal of square bracket.

Agreements:

CR is agreeable

## 2.5	sTTI UE demod and CSI (Rel-15)

### 2.5.1	SPDCCH

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.22.7.1.2 | R4-1806764 | other | Simulation results of SPDCCH for sTTI UE demodulation requirements | Ericsson |
| 6.22.7.1.2 | R4-1807906 | discussion | Discussion and simulation results on sTTI UE SPDCCH demodulation requirements | Huawei, HiSilicon |

Summary of proposals:

R4-1806764 (Ericsson):

- Proposal 1: RAN4 sets EVA30 for CRS-based SPDCCH demodulation requirements.

R4-1807906 (Huawei):

- Proposal 1: Use EPA5 for CRS-based sPDCCH performance requirements

Issues for the discussion:

Fading channel condition:

- Option 1: EVA30

- Option 2: EPA5

### 2.5.2	PDSCH

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.22.7.1 | R4-1806627 | discussion | Simulation results for CRS-based FDD sPDSCH demodulation | Qualcomm Incorporated |
| 6.22.7.1.1 | R4-1806762 | other | TBS tables used for sTTI UE demodulation requirements | Ericsson |
| 6.22.7.1.1 | R4-1806763 | other | Simulation results of PDSCH for sTTI UE demodulation requirements | Ericsson |
| 6.22.7.1.1 | R4-1807905 | discussion | Discussion on FRC definition for sTTI PDSCH demodulation requirements | Huawei, HiSilicon |

Summary of proposals:

R4-1806627 (Qualcomm):

- Observation 2. It is very difficult to find same modulation with same target coding rate across all the subslots for subslot-based PDSCH due to the significant difference in the coding rate with the same MCS.

- Observation 3. In case there are PDSCH allocation on both 2-symbol sTTIs and 3-symbol sTTIs in subslot-based PDSCH test, we need to agree on an eNB retransmission scheme to be able to align the simulation results.

- Observation 4. Note that there is an ongoing discussion in RAN1 to potentially change the scaling factor for TBS computation for 3 symbol subslot TTI from α= 1/6 to α=1/4, which can impact the FRC table for subslot-based PDSCH in case it gets agreed in RAN1.

- Proposal 1. We propose to use Table 1 as FRC table for CRS-based slot-based PDSCH, which is defined considering R.11 as reference, as agreed in RAN4#86bis meeting [1].

- Proposal 2. We propose the following options to define FRC table for CRS-based subslot-based PDSCH:

  - Option 1: Consider 16QAM target coding rate 1/3 with PDSCH allocation on all subslots except subslot index 3 (since we could not find 16QAM MCS with coding rate 1/3 for subslot index 3) with Table 2 as FRC table.

  - Option 2: Consider two different test cases

    - One test with PDSCH allocation on subslot index 2 and subslot index 4 with 16QAM MCS target coding rate ½ with Table 3 as FRC table.

    - One test with PDSCH allocation on subslot index 1 and subslot index 5 with 64QAM MCS target coding rate ½ with Table 4 as FRC table.

R4-1806762 (Ericsson):

- Proposal: RAN4 confirms the method to derive TBS and the TBS tables used for slot/subslot-PDSCH demodulation requirements as shown in this paper.

R4-1806763 (Ericsson):

- Proposal 1: For Subslot-PDSCH, subslot#3 should be DTX’ed to avoid very high coding rate at retransmission.

- Proposal 2: RAN4 sets EVA30 for TM3 PDSCH demodulation requirements with sTTI.

- Proposal 3: RAN4 specifies TM9 single layer PDSCH demodulation requirements for sTTI.

R4-1807905 (Huawei):

- Proposal 1: Use MCS 16 for subslot-TTI#1 and 5 to keep the same modulation order and closet code rate as much as possible as other subslot-TTI.

- Proposal 2: Dual layer transmission shall be configured for DMRS-based TM9 sPDSCH cases.

- Proposal 3: Agreed to use the FRC defined in section 2 for the related simulation assumptions.

Issues for the discussion:

Fading channel for TM3 dual layer with 16QAM 1/2:

- Option 1: EVA30

- Option 2: EPA5?

Number of layers in TM9 with QSPK 1/3:

- Option 1: Single layer

- Option 2: Dual layer

MCS/TBS for Slot-PDSCH (CRS-based):

- Need alignment of MCS/TBS values

| SF | Slot | Huawei | Qualcomm | Ericsson |
| --- | --- | --- | --- | --- |
| TBS |  |  |  |  |
| 0 | 0 | 4776 | 9912 | 4968 (9912 for 2layer) |
|  | 1 | 5992 | 12576 | 6456 (12576 for 2 layer) |
| 1,2,3,4,6,7,8,9 | 0 | 5736 | 11448 | 5736 (11448 for 2 layer) |
|  | 1 | 6968 | 14112 | 6968 (14112 for 2 layer) |
| Channel bits |  |  |  |  |
| 0 | 0 | 9184 | 10264 | 10264 |
|  | 1 | 11552 | 13184 | 13184 |
| 1,2,3,4,6,7,8,9 | 0 | 11200 | 11200 | 11200 |
|  | 1 | 14144 | 14114 | 14114 |

MCS/TBS for Subslot-PDSCH (CRS-based):

- Option 1: 16QAM target coding rate 1/3 with PDSCH allocation on all subslots except subslot #3

- Option 2: Specify two test cases: 1) PDSCH allocation on subslots #2 and #4 with 16QAM 1/2 and 2) PDSCH allocation on subslots #1 and #5 with 64QAM 1/2.

- Option 3: DTX Subslot #3

- Option 4: Use MCS16 (16QAM 0.58) for subslot #1 and #5

Comments:

Qualcomm: RAN1 is discussing the different scaling factor for TBS selection according to 2OS or 3OS. Prefer to keep the same coding rate over transmission.

Huawei: Different coding rate has been exiting in LTE, but not so serious as sTTI subslot-PDSCH.

Ah-hoc chair: Companies to think about how to make sure the TE’s behavior in the case of retransmission to minimize the huge coding rate difference between the first transmission and retransmission.

### 2.5.3	CQI reporting

| Agenda | Tdoc | Type | Title | Source |
| --- | --- | --- | --- | --- |
| 6.22.7.2 | R4-1806628 | discussion | Discussion on CSI reporting for sTTI simulation assumptions | Qualcomm Incorporated |
| 6.22.7.2.1 | R4-1806765 | other | CQI reporting tests for sTTI | Ericsson |
| 6.22.7.2 | R4-1807907 | discussion | Discuss on sTTI UE CSI test | Huawei, HiSilicon |

Summary of proposals:

R4-1806628 (Qualcomm):

- Observation 1. Note that there is an ongoing discussion in RAN1 to potentially change the scaling factor for TBS computation for 3 symbol subslot TTI from α= 1/6 to α=1/4, which can impact the CQI2MCS mapping for subslot-based PDSCH in case it gets agreed in RAN1.

- Proposal 1. Wideband CQI reporting with PUSCH 1-1 with EPA5 channel can be selected for both CRS-based TM4 and DMRS-based TM9 sTTI CSI reporting test.

- Proposal 2. We propose to use Table 1 as CQI2MCS table for CRS-based slot-based PDSCH based on the agreed simulation assumptions in RAN4#86bis meeting [1-2].

- Proposal 3. We propose to use Table 2 as CQI2MCS table for CRS-based subslot-based PDSCH based on the agreed simulation assumptions in RAN4#86bis meeting [1-2].

R4-1806765 (Ericsson):

- Proposal 1: For sTTI CQI test, RAN4 adopts the wideband CQI test with PUSCH 1-1.

- Proposal 2: RAN4 discusses further the SPDCCH configuration for CQI test.

- Proposal 3: RAN4 discusses further whether the CQI to MCS table is derived based on the averaged OFDM symbols over one subframe or other methods.

R4-1807907 (Huawei):

- Proposal1: Suband CQI reporting for PUSCH 3-1 under frequency selective fading conditions can be selected for both CRS-based TM4 and DMRS-based TM9 transmission mode.

- Proposal2: Agreed to use the above FRC table for CQI to MCS mapping for CRS-based and DMRS-based PUSCH 3-1 subband CQI reporting for subslot/slot TTI.

Issues for the discussion:

Test method:

- Option 1: Wideband CQI reporting test with PUSCH 1-1 with EPA5 for both TM4 and TM9.

- Option 2: Subband CQI reporting test with PUSCH 3-1 with frequency selective fading condition for both TM4 and TM9.

Comments:

Huawei: We are fine with option 1.

CQI-to-MCS table:

- CQI-to-MCS table definition

  - Option 1: Based on subframe (the averaged OFDM symbols over one subframe)

  - Option 2: Based on slot/subslot index

Comments:

Huawei: We are fine with option 2.

Agreements:

Qualcomm to draft WF on sTTI.