3GPP TSG-RAN WG4 Meeting #87													R4-1808120

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Nokia, LGE, Skyworks

Title: 						TP for 37.863-01-01: DC_(n)71B

Agenda Item: 			9.2.2.5

Document for:	Approval

# 1	Introduction

This contribution completes the information of DC_(n)71B in technical report.

*************************Start of the TP*******************************************

## 6.69	DC_(n)71B

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_(n)71B | 71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
|  | n71 |  |  |  |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| EN-DC configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configuration | Component carriers in order of increasing carrier frequency |  |  |  |  |  | Maximum aggregated bandwidth [MHz] |  | Bandwidth combination set |  |  |
|  |  | Channel bandwidths for LTE carrier [MHz] |  | Channel bandwidths for NR carrier [MHz] |  | Channel bandwidths for LTE carrier [MHz] |  |  |  |  |  |  |
| DC_(n)71B | DC_(n)71B | 15 |  | 5 |  |  |  | 20 |  | 0 |  |  |
|  |  | 10 |  | 5, 10 |  |  |  |  |  |  |  |  |
|  |  | 5 |  | 5, 10, 15 |  |  |  |  |  |  |  |  |
|  |  |  |  | 5 |  | 15 |  |  |  |  |  |  |
|  |  |  |  | 5, 10 |  | 10 |  |  |  |  |  |  |
|  |  |  |  | 5, 10, 15 |  | 5 |  |  |  |  |  |  |

### 6.69.3	Co-existence studies

DC_(n)71B is a contiguous allocation thus only uplink harmonic analysis is needed.

Table 6.69.3-1: DC_(n)71B UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

From Table 6.69.3-1 it can be seen that uplink harmonics 3rd harmonic overlaps with bands 3, 25, 34 and 70.

### 6.69.4	∆TIB and ∆RIB values

No dTib or dRib is required for DC_(n)71B as it is intra-band DC.

### 6.69.5	MSD

Table 6.69.5-1: MSD for the DC_(n)71B configuration

| MSD / DC bandwidth class B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation [LCRB] | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
|  | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
|  | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
|  | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |

### 6.69.6	SEM for DC_(n)71B

FCC has defined spectrum emission mask that is relevant for operation on band 71 frequency range. SEM is presented in Table 6.69.6-1.

6.69.6-1: SEM for band 71 frequency range

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

| - |  |  |  |
| --- | --- | --- | --- |
| - |  |  |  |

### 6.69.7	A-MPR

In this session, we analyze the A-MPR requirements for EN-DC_(n)71B UE based on single Tx antenna and single PA RF architecture.

To derive A-MPR level, following simulation parameters and assumptions were used

- Consider CP-OFDM waveform and DFT-S-OFDM for NR

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0.5 dB MPR

- LO leakage and IQ-Image

  - LTE UE =  - 25 dBc

  - NR UE = -28dBc

- ALCR from 38.101-3

- LTE SEM from 36.101, Table 6.6.2.1.1-1 and NR SEM from 38.101-1, Table 6.5.2.2-1

- Additional SEM from 38.101-3, Table 6.69.6-1

- NR general spurious limit from 38.101, Table 6.5.3.1-2

- EVM not included

- Equal PSD for LTE and NR : Same backoff reported for LTE and NR

- UE-to-UE coexistence assumption to protect Band 12, 29 and own Bands

  - UE-to-UE requirement is -50 dBm/1 MHz for bands 12 and -38 dBm/1MHz for band 29.

  - filter rejection to band 12 is at least 25 dB

  - filter rejection to band 29 is at least 10 dB

Based on simulations the required A-MPR levels are defined as below

For DC_(n)71B with configured with network signaling values as per Table 6.2B.3.1.1-1 the allowed A-MPR is defined by

- for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE.

- AMPRDC is total output power back off i.e. MPR+A-MPR

AMPRDC=CEILMA,DC(A), 0.5

-	for OFDM:

MA,DC(A) =	10.00 - 11.67*A;		0.00 < A ≤ 0.30

7.10 -  2.00*A;					0.30 < A ≤ 0.80

5.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC(A) =	10.00 - 13.33*A;  	0.00 < A ≤ 0.30

7.00 - 3.33*A;   					0.30 < A ≤ 0.60

5.00;								0.60 < A ≤ 1.00

where

A=LCRB,LTE+LCRB,NRNRB,LTE+NRB,NR

- for UE not indicating support of dynamicPowerSharing

The A-MPR is defined carrier wise and are total output power back off i.e. MPR+A-MPR per carrier as follows

AMPRLTE=CEIL{MA,LTE, 0.5}

AMPRNR=CEIL{MA,NR, 0.5}

where

MA,LTE=MA,DC(ALTE,wc)-LTE

MA,NR=MA,DC(ANR,wc)-NR

Above, the worst-case allocation ratios for each carrier are as follows where LTE modems assumes that NR allocation is 1 as it will lead to maximum needed A-MPR and vice versa.

ALTE,wc=LCRB,LTE+1NRB,LTE+NRB,NR

ANR,wc=1+LCRB,NRNRB,LTE+NRB,NR

and the worst-case division of power between carriers is represented by

∆LTE=10 log10LCRB,LTELCRB,LTE+NRB,NR

and

∆NR=10 log10LCRB,NRNRB,LTE+LCRB,NR

Table 6.69.7-1. Symbols for DC_(n)71B A-MPR

| LCRB,LTE | Allocation size (in RB) in LTE channel |
| --- | --- |
| LCRB,NR | Allocation size (in RB) in NR channel |
| NRB,LTE | Transmission bandwidth configuration in LTE channel |
| NRB,NR | Transmission bandwidth configuration in NR channel for its configured SCS |
| NRB,NR | Transmission bandwidth configuration in NR channel for 15 kHz SCS |