3GPP TSG-RAN WG4 Meeting #87	R4-1808327

## Busan, Korea, 21 - 25 May, 2018

Source: 	Huawei

Title:	TP to TS 38.141-2: multi-band operation

Agenda Item:	7.8.4 (Radiated conformance testing (38.141-2))

Document for:	Approval

# Introduction

During RAN4#86bis meeting the multi-band text was Endorsed for Draft CRs to the TS 38.104 in [2 - 4].

Based on 38.104 text in subclause 4.8, in this TP to TS 38.141-2 (latest version shared on the RAN4 reflector before the meeting), introduction of the multi-band operation for conducted tests of single RAT NR is proposed.

The following modifications were introduced in the attached TP:

- 3.1: alignment of the “multi-band connector” definition with the RAN4#86bis correction, as well as with RAN#87 correction in [5],

- 4.5: Editor’s note as a placeholder for potential future text in the multi-band operation in the BS configurations subclause (it was addressed in the conducted testing part, and in the legacy specifications),

- 4.11: text for multi-band RIB operation, based on TS 38.104 text.

  - New subclause for “Tests for BS capable of multi-band operation with three or more bands” is kept empty and in “[]” as it requires discussion and it FFS.

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1:  approve the attached TP to TS 38.141-2 on the multi-band operation for single RAT NR.

# References

[1] 3GPP TS 38.141-2, shared on RAN4 reflector after RAN4#86bis

[2] R4-1804939	Draft CR to TS 38.104: correction of the multi-band operation text (4.8), RAN4#86bis

[3] R4-1805792	Draft CR to TS 38.104: Introduction of CA/Multi-carrier operation related terminology, RAN4#86bis

[4] 	R4-1805793	DraftCR 38104 - multi-band clean up, RAN4#86bis

[5] R4-1807736	Draft CR to TS 38.104: multi-band RIB correction (4.8)

# TP to TS 38.141-2

------------------------------ Modified section ------------------------------

## Definitions

------------------------------ Unchanged text omitted ------------------------------

------------------------------ Unchanged text omitted ------------------------------

------------------------------ Next modified section ------------------------------

## 4.5	BS configurations

### 4.5.1	Transmit configurations

Unless otherwise stated, the radiated transmitter characteristics in clause 6 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor’s note: to be aligned with the figures for the RIB interfaces and co-location concept.

### 4.5.2	Receive configurations

Unless otherwise stated, the radiated receiver characteristics in clause 7 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor’s note: to be aligned with the figures for the RIB interfaces and co-location concept.

### 4.5.3	Power supply options

If the BS is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

------------------------------ Next modified section ------------------------------

## 4.11	Requirements for BS capable of multi-band operation

For multi-band RIB, the radiated test requirements in clause 6 and 7 apply separately to each supported operating band, unless otherwise stated. For some radiated test requirements, it is explicitly stated that specific additions or exclusions to the test requirement apply at multi-band RIB(s) as detailed in the requirement subclause.

BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

For multi-band RIBs supporting the bands for TDD, the radiated test requirements in the present specification assume no simultaneous uplink and downlink occur between the bands.

The radiated test requirements for multi-band RIBs supporting bands for both FDD and TDD are FFS and are not covered by the present release of this specification.

## 4.11A		Tests for BS capable of multi-band operation with three or more bands

Detailed content of this subclause is FFS.

------------------------------ End of modified section ------------------------------