3GPP TSG RAN WG4 Meeting #87	R4-1808440

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.10.4.3

Source: 	Ericsson et al.

Title:	Summary of SFTD discussions

Document for:	Information

# Introduction

The present document summarizes agreements related to inter-RAT SFTD measurements.

# Summary

| Contributions: |  |  |  |
| --- | --- | --- | --- |
| R4-1806311 | Discussion | On SFTD measurement | Intel Corporation |
| R4-1806538 | Discussion | Discussion on SFTD when PSCell is not configured | MediaTek Inc. |
| R4-1807263 | Discussion | Further discussion on inter-RAT SFTD measurement requirements | ZTE |
| R4-1807393 | Other | On interruptions for inter-RAT SFTD | Ericsson |
| R4-1807394 | Other | On remaining requirements for inter-RAT SFTD measurements | Ericsson |
| R4-1807395 | CR (revision) | Introduction of inter-RAT SFTD measurement requirement | Ericsson |
| R4-1807260 | CR | CR to 36.133 on FDD inter-RAT SFTD measurement requirements | ZTE |
| R4-1807261 | CR | CR to 36.133 on TDD inter-RAT SFTD measurement requirements | ZTE |
| R4-1807262 | CR | CR to 36.133 on inter-RAT SFTD interruption requirements | ZTE |

Agreed assumptions

- RSRP measurement period

  - RSRP measurement time is assuming measurements over 5 SSB

  - Side condition is -3dB

- PCell and SCell(s) Interruption length for inter-band interruption-based SFTD

  - 1 subframe

- PCell and SCell(s) Interruption length for intra-band interruption-based SFTD

  - 5 subframes [conditioned on two interruptions in total]
Justification: Colocation is assumed for both contiguous and non-contiguous intra-band EN-DC. This implies that there will be a limited power imbalance between the strongest received LTE cell and the strongest received NR neighbour cell. The interruption of 5 subframes is for allowing the UE to find an optimum gain setting after having widened the received bandwidth – similar to how it has been specified for LTE intra-band CA and LTE DC. [The condition is justified by that if more interruptions are allowed, gain states can be reused and the additional interruptions shall therefore be shorter.]

- Gap-based SFTD and other measurements in gaps:

##### 8.1.2.1.1	Monitoring of multiple layers using gaps

When monitoring of multiple inter-frequency E-UTRAN and inter-RAT (UTRAN, GSM, NR) using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (RSRP, RSRQ, RSTD, UTRAN TDD P-CCPCH RSCP, UTRAN FDD CPICH measurements, GSM carrier RSSI, NR SS-RSRP, NR SS-RSRQ, NR SS-SINR, etc.) of detected cells on all the layers

The effective total number of frequencies excluding the frequencies of the PCell, SCells, and PSCell being monitored is Nfreq, which is defined as:

Nfreq = Nfreq, E-UTRA + Nfreq, UTRA + Mgsm + Nfreq, cdma2000 + Nfreq, HRPD + Nfreq, NR

where

Nfreq, E-UTRA is the number of E-UTRA carriers being monitored (FDD and TDD)

Nfreq, UTRA is the number of UTRA carriers being monitored (FDD and TDD)

MGSM is an integer which is a function of the number of GSM carriers on which measurements are being performed. MGSM is equal to 0 if no GSM carrier is being monitored. For a MGRP of 40 ms, MGSM is equal to 1 if cells on up to 32 GSM carriers are being measured. For a MGRP of 80 ms, MGSM is equal to ceil(Ncarriers,GSM /20) where Ncarriers,GSM is the number of GSM carriers on which cells are being measured.

Nfreq, cdma2000 is the number of cdma2000 1x carriers being monitored.

Nfreq, HRPD is the number of HRPD carriers being monitored.

Nfreq, NR is the number of NR inter-RAT carriers being monitored.

Note: Nfreq, NR is applicable for a E-UTRA-NR dual connectivity capable UE not configured with NR PSCell.

Inter-RAT SFTD reporting delay requirement is scaled by Nfreq similar to how it is handled for other RRM measurements. 
Justification can be found in 36.133, 8.1.2.1.1 “Monitoring of multiple layers using gaps” (see below). Existing RRM requirements already take this scaling into account.

- Interruption-based SFTD and measurements in gaps:

  - The interruption-based SFTD reporting delay requirement is for Rel-15 derived under the assumption that there are no measurements carried out in measurement gaps while the UE is carrying out the SFTD measurement. 
Justification: to reduce the specification work. Measurement configuration and connected mode UE mobility are both under network control, and hence the network node can and should avoid requesting SFTD measurements to be carried out by UEs who are in urgent need of handover to inter-frequency or inter-RAT neighbor cell.

- Abortion of SFTD measurement when UE receives PSCell addition RRC signalling

  - The UE aborts interruption-based inter-RAT SFTD reporting when the UE receives a PSCell addition RRC message.
Justification: the scenario, and hence the UE solution, changes. PSCell addition procedure may in this case require hardware resources (e.g. cell searcher).

- Number of SFTD reports

reportAmount

Number of measurement reports applicable for triggerType event as well as for triggerType periodical. In case purpose is set to reportCGI or reportStrongestCellsForSON only value 1 applies. In case reportSFTD-Meas is configured, only value 1 applies.

For inter-RAT SFTD, only one measurement report is to be provided by the UE, i.e., no periodic reporting is requested.
Justification: This is already captured in 36.331 V15.1.0 clause 6.3.5 “ReportConfigInterRAT” (see below).

- Applicability of interruption-based inter-RAT SFTD with respect to band combinations

  - Interruption-based SFTD measurement is supported only for LTE – NR band combinations that are supported by the UE. Capture in Section 7 together with interruption requirements.
Justification: Band combinations that are not supported may for instance cause interference between RF receiver chains or result in RF hardware conflicts, and hence would have to be measured using autonomous gaps, i.e., where the UE tunes away from the LTE PCell during the time it receives NR radio samples. For the early roll-out it is reasonable to assume that the network acquires the timing information using UEs that also can benefit from the same.

- Number of cells to report

  - The UE may report up to 3 cells. Inter-RAT SFTD measurement delay requirements are derived based on reporting of a single cell.
Justification: When we decided to ask RAN2 for signaling that would allow reporting for up to 3 cells, the justification was that some UE implementations may find more than one cell, and reporting of more than one cell would then give added value without requiring additional efforts. Allowing reporting for up to 3 cells was not intended to put requirement on the UE to detect more than one cell. Additionally, the functionality is intended to provide the initial timing between the LTE and NR systems so the LTE PCell can configure regular measurements – by which additional NR neighbor cells can be identified.

- When interruptions are allowed for interruption-based SFTD

  - Inter-band: interruptions before and after each chunk of radio samples. Captured in Section 7 as an allowed percentage of interruptions for the whole SFTD measurement procedure.
Intra-band: One before and one after the whole SFTD measurement procedure. Captured in Section 7 as an allowed number of interrupted subframes for the whole SFTD measurement procedure.
Justification: Compromise solution balancing UE power consumption and system throughput.

- UE cell detection processing capabilities in FR2

  - The UE is assumed to be capable of processing 5ms of radio samples every 20ms when carrying out cell detection on a carrier in FR2 (similar to the assumption for FR1 agreed at RAN4#AH1801)
Justification: The number of samples as well as the number of SSBs in a half-frame are in general larger in the numerologies used for FR2 than for FR1. However, other requirements in NR are dimensioning, hence also for FR2 the UE needs to be capable of carrying out cell detection every 20ms. Therefore we can use the assumption on 5ms radio samples being processed every 20ms also for inter-RAT SFTD for carrier in FR2.

Open issues, Delay requirements

- Measurement period (excluding RSRP)
Companies to provide numbers under the agreed conditions above before end of 3rd round

  - Proposals for measurement period for FR1, gap-based SFTD

  - Proposals for measurement period for FR2, interruption-based SFTD

  - Proposals for measurement period for FR2, gap-based SFTD

- Measurement period (including RSRP)
Companies to provide numbers under the agreed conditions above before end of 3rd round

  - Proposals for measurement period for FR1, interruption-based SFTD

  - Proposals for measurement period for FR1, gap-based SFTD

  - Proposals for measurement period for FR2, interruption-based SFTD

  - Proposals for measurement period for FR2, gap-based SFTD