3GPP TSG-RAN WG4 Meeting #88	R4-1809702

Gothenburg, Sweden, 20 – 24 August 2018

Agenda Item:	7.9.4.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for NR FR1 OTA receiver in-band general blocking requirement

Document for:	Approval

# 1.	Introduction

The MU for eAAS and NR FR1 OTA receiver in-band directional requirements were discussed in RAN4 AH-1807 and the way forward were agreed [1], where it was agreed that ‘If no further analysis on NR FR1 according to the agreed timeline, the same MU budget as eAAS (3GHz < f ≦ 4.2GHz) can be reused for NR FR1 (3GHz < f ≦ 6GHz).’

This contribution provides our proposal on the MU for NR FR1 OTA receiver in-band general blocking requirement, according to the agreed way forward in [1] and our proposals in [2].

# 2.	Discussion

It was agreed in [1] that the MU for eAAS and NR FR1 OTA receiver in-band directional requirements should be calculated as:

MU=MUconductedwanted2+MUconductedint2+MUEIS2-MUTestEquipment2-2*MUmatching2+ACLR effect

For operating bands above 4.2GHz, we propose in [2] that:

1.	For the MU variables independent of the radiated measurement methods, use the values specified in TS 36.141 [3] for 4.2GHz < f ≦ 6GHz.

2.	For the MU variables dependent of the radiated measurement methods, use the values included in [4] for 4.2GHz < f ≦ 6GHz, and reuse the 3GHz < f ≦ 4.2GHz values in TR 37.842 [5] for 4.2GHz < f ≦ 6GHz for those not included in [4], unless other values are proposed in this meeting.

Using the above proposals, the MU for NR FR1 OTA receiver in-band general blocking requirement can be calculated as shown in Tables 1 and 2 below for Indoor Anechoic Chamber and Compact Antenna Test Range, respectively.

Table 1: MU for Indoor Anechoic Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2GHz | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz |
| --- | --- | --- | --- |
| 1.96σ MUconductedwanted (Wanted signal level error in [3]) | 0.7 | 1 | 1.5 |
| 1.96σ MUconductedint (Interferer signal level error in [3]) | 1 | 1.2 | 1.8 |
| MUEIS (Combined standard uncertainty in [4, 5]) | 0.62 | 0.64 | 0.73 |
| MUTestEquipment (Uncertainty of the RF signal generator in [4, 5]) | 0.46 | 0.46 | 0.58 |
| MUmatching (Impedance mismatch in the transmitting chain in [4, 5]) | 0.1 | 0.16 | 0.16 |
| 1.96σ ACLReffect (Impact of interferer leakage in [3]) | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 0.94 | 1.09 | 1.46 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 1.84 | 2.13 | 2.86 |

Table 2: MU for Compact Antenna Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2GHz | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz |
| --- | --- | --- | --- |
| 1.96σ MUconductedwanted (Wanted signal level error in [3]) | 0.7 | 1 | 1.5 |
| 1.96σ MUconductedint (Interferer signal level error in [3]) | 1 | 1.2 | 1.8 |
| MUEIS (Combined standard uncertainty in [4, 5]) | 0.68 | 0.71 | 0.79 |
| MUTestEquipment (Uncertainty of the RF signal generator in [4, 5]) | 0.46 | 0.46 | 0.58 |
| MUmatching (Standing wave between DUT and test range antenna in [4, 5]) | 0.15 | 0.15 | 0.15 |
| 1.96σ ACLReffect (Impact of interferer leakage in [3]) | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 0.97 | 1.14 | 1.50 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 1.91 | 2.24 | 2.94 |

Consequently, the test system specific measurement uncertainty values for NR FR1 OTA receiver in-band general blocking test can be obtained in Table 3 below.

# Table 3: Test system specific measurement uncertainty values for NR FR1 OTA receiver in-band general blocking test

|  | Expanded uncertainty ue [dB] |  |  |
| --- | --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2GHz | 4.2GHz < f ≦ 6GHz |
| Indoor Anechoic Chamber | 1.84 | 2.13 | 2.86 |
| Compact Antenna Test Range | 1.91 | 2.24 | 2.94 |
| Common maximum accepted test system uncertainty | 1.9 | 2.2 | 2.9 |

# 3.	Conclusion

This contribution has provided our proposal on the MU for NR FR1 OTA receiver in-band general blocking requirement, according to the agreed way forward in [1] and our proposals in [2].

Proposal:

Adopt the following table as the MU for NR FR1 OTA receiver in-band general blocking test.

# Table 3: Test system specific measurement uncertainty values for NR FR1 OTA receiver in-band general blocking test

|  | Expanded uncertainty ue [dB] |  |  |
| --- | --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2GHz | 4.2GHz < f ≦ 6GHz |
| Indoor Anechoic Chamber | 1.84 | 2.13 | 2.86 |
| Compact Antenna Test Range | 1.91 | 2.24 | 2.94 |
| Common maximum accepted test system uncertainty | 1.9 | 2.2 | 2.9 |

# References

[1]	R4-1809428, “WF on MU of receiver directional requirements”, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, NTT DOCOMO, INC.

[2]	R4-1809697, “Proposals on MU for NR FR1 OTA receiver requirement for operating bands above 4.2GHz”, Nokia, Nokia Shanghai Bell.

[3]	3GPP TS 36.141 v15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[4]	R4-1805879, “Test Equipment uncertainty values for FR1”, Keysight Technologies.

[5]		3GPP TR 37.842 v13.2.0, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.