3GPP TSG-RAN4 #88 	R4-1809759

# Gothenburg, SE, August 20 – 24, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | 0 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pcmax for Rel-15 intra-band contiguous NR CA within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-25 |
| Category: |  |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Pcmax definition for intra-band contiguous CA FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.2A.4.1.1 Configured transmitted power for Intra-band contiguous CA - FR1 sub-clause is added to the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 intra-band contiguous NR CA is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

##### 6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX in each slot. This requirement uses the assumption that PUSCH, PUCCH are configured with the same UL slot symbol pattern over all aggregated cells.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

For uplink intra-band contiguous carrier aggregation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.2A.2.1 and subclause 6.2A.3.1.1 respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass – ΔPPowerClass) – MAX(MPR + A-MPR + ΔTIB,c + TC + ∆TRxSRS, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

where

PEMAX,c is the value given by IE P-Max for serving cell c, defined in TS 38.331[7];

PPowerClass is the maximum UE power specified in subclause 6.2A.1.1 without taking into account the tolerance specified in the Table 6.2A.1.1-1;

ΔPPowerClass = 3 dB for a power class 2 capable UE operating in Band n41, n77, n78 and n79, when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evalutation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or if P-Max is not indicated in the cell, ΔPPowerClass = 0 dB;

MPR and A-MPR are specified in subclause 6.2A.2.1 and subclause 6.2A.3.1.1 respectively;

∆TIB,c is the additional tolerance for serving cell c as specified in subclause 6.2A.4.2.1; ∆TIB,c = 0 dB otherwise;

∆TC is TBD;

∆TRxSRS is 3 dB and is applied when UE transmits SRS to the antenna port that is designated as Rx port. For other SRS transmissions ∆TRxSRS is zero

P-MPRc = P-MPR is the allowed maximum output power reduction for

a) ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN spduiecifications;

b) ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPR only for the above cases. For UE conducted conformance testing P-MPR shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

The measured maximum output power PUMAX over all serving cells with same intra-slot symbol pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

### where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale.

### The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in 6.2A.4.1.1-1

Table 6.2A.4.1.1-1: PCMAX tolerance for uplink intra-band contiguous CA

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 < PCMAX ≤ 33 | 2.0 |  |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

### ---------------------------------------End of Change 1-------------------------------