3GPP TSG-RAN4 #88 	R4-1809763

# Gothenburg, SE, August 20 – 24, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | 0 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR applicability for NR intra-slot hopping or multiple transmissions type FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Due to multiple possible intra-slot transmissions type time multiplexed or configured frequency hopping, the A-MPR value to be aplied in the 6.2.4 subclause for Pcmax needs clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a paragraph that clarifies the A-MPR rule to be applied for the entire slot transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR value to be applied per slot is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3-1 specifies for UE Power Class 3 the additional requirements and allowed A-MPR with corresponding network signalling value and operating band. Unless otherwise stated, the allowed A-MPR is in addition to the allowed MPR specified in subclause 6.2.2.

Table 6.2.3-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) | Value of additionalSpectrumEmission |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 |  |  |  |  | N/A | 1 |
| NS_02 | 6.5.2.1.2 | n1, n2, n3, n4, n5, n8, n20, n25, n66, n80, n81, n82, n84, NOTE 1 |  |  | Table 6.2.3-2 | 1 |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70 |  |  | Table 6.2.3.7-1 | 3 |
| NS_04 | 6.5.2.3.1 | n41 | 10, 15, 20, 40, 50, 60 80, 100 |  | Subclause 6.2.3.2 | 4 |
| NS_06 | 6.5.2.3.3 | n12 | 5, 10, 15 | 5.3.5 | N/A | 2 |
| NS_10 |  | n20, n82 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 | NS_xx |
| NS_07 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.3-1 | [1]3,4 |  |
| NS_05 | 6.5.3.3.3 | n28, n83 | 5 | ≥ 2 | ≤ 24 |  |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 54 |  |
| NS_08 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 205 |  | Subclause 6.2.3.4- |  |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A | 2 |
| NS_40 | 6.5.2.3.7 | n51 | 5 |  | Table 6.2.3.5-1 | 35 |
| NS_09 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Subclause 6.2.3.6 |  |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: The total maximum output power reduction for NS_xx and NS_yy is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-1 in TS 36.101 and A-MPR specified in Table 6.2.3-1. NOTE 3: The A-MPR is 0dB for inner RB allocations for DFT-s-OFDM PI/2 BPSK and QPSK. NOTE 4: The A-MPR for CP-OFDM shall also add the corresponding MPR specified in Table 6.2.2-1. NOTE 5: No A-MPR is applied for 5MHz CBW where the lower channel edge is ≥1930MHz,10MHz CBW where the lower channel edge is ≥1950MHz and 15MHz CBW where the lower channel edge is ≥1955MHz. |  |  |  |  |  |  |

Table 6.2.3-2: A-MPR for UTRA protections

| Modulation | A-MPR |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 1.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 0.5 | 0 |
| DFT-s-OFDM 64 QAM | ≤ 0.5 | 0 |
| DFT-s-OFDM 256 QAM | 0 | 0 |
| CP-OFDM QPSK | ≤ 1 | 0 |
| CP-OFDM 16 QAM | ≤ 1 | 0 |
| CP-OFDM 64 QAM | ≤ 0.5 | 0 |
| CP-OFDM 256 QAM | 0 | 0 |
| NOTE 1: A-MPR defined in this Table is additive to MPR defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |

When multiple transmissions are scheduled and time multiplexed in a slot or intra-slot frequency hopping is configured, the A-MPR is evaluated for each transmission type and/or hopping region and the maximum value over the transmissions is then applicable for the entire slot.

### ---------------------------------------End of Change 1-------------------------------