3GPP TSG-RAN4 #88 	R4-1809768

# Gothenburg, SE, August 20 – 24, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | 0 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pcmax for Rel-15 inter-band EN-DC within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Pcmax definition for inter-band EN-DC FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.2B.4.1.3 Inter-band EN-DC within FR1 sub-clause is added to the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 inter-band EN-DC is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

### 6.2B.4.1.3	Inter-band EN-DC within FR1

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in 36.101 sub-clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN {MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-      MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PLTE, PPowerClass – ΔPPowerClass}

The configured maximum output power PCMAX_ NR,c (q) in slot q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in sub-clause 6.2.4 of 38.101-1 modified by PNR as follows:

PCMAX_L,f,c,,NR = MIN {MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PNR , PPowerClass – ΔPPowerClass }

- PLTE and PNR are the linear values for the PLTE and PNR respectively signaled by RRC defined in [7]

- ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in 36.101 applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

- TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise TC_NR,c = 0dB;

- ΔTIB,c specified in sub-clause 6.2.7 for EN-DC, the individual Power Class defined in table 6.2B.1-3 and any other additional power reductions parameters specified in sub-clauses 6.2.3 and 6.2.4 for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX_ NR,c evaluations.

If the transmissions from NR and E-UTRA do not overlap, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

If the EN-DC UE is not supporting dynamic power sharing, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications 36.101 and 38.101-1 respectively apply with the modifications specified above, and if total possible transmit power expressed by PLTE + PNR exceeds PPowerClass, EN-DC, then single UL operation is assumed by the UE. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX_ NR,c for the configured E-UTRA and NR uplink carriers, respectively, as specified above and its total configured maximum output power PCMAX.

When an UL subframe transmission p from E-UTRA overlap with a slot q from the NR, then for PCMAX (p,q) evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

The total UE configured maximum output power PCMAX (p,q) in a subframe p of CG 1 and a scheduling unit (slot) q of CG 2 that overlap in time shall be evaluated for each Teval over the TREF and respect the following upper limit for synchronous and asynchronous operation unless stated otherwise:

PCMAX_ EN-DC _L (p,q)  ≤  PCMAX (p,q)  ≤  PCMAX_ EN-DC _H (p,q)

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

when the NR UL grant is available in the same time with LTE UL grant of before;

Or

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC}

When the NR UL grant is known after LTE UL grant.

where

- pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

- pCMAX H _ NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

- pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

- pCMAX L _ NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

- PPowerClass, EN-DC is defined in sub-clause 6.2B.1.3-1 for inter-band EN-DC;

- PEMAX, EN-DC is  PMAX, EN-DC value signaled by RRC and defined in [7];

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframes and slots durations are used in aggregated carriers.

PPowerClass ,EN-DC shall not be exceeded by the UE during any period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | NR slot |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n), }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL slot overlapping with LTE subframe p.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference unit duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity LTE-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |

### ---------------------------------------End of Change 1-------------------------------