3GPP TSG-RAN WG4 Meeting #88 	R4-1809877

Gothenburg, Sweden, August 20 – 24, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption due to BWP switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for interruption requirements due to BWP switching for EN-DC and NR CA need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification is made in section 8.5.3. New section 8.5.3.1 and section 8.5.3.2 are added to specify interruption requirements due to BWP switching in EN-DC and NR CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirements due to BWP switching exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.5.3, and new section 8.5.3.1, 8.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change >>

### 8.5.3	Interruption due to Active BWP switching Requirement

When UE receives a DCI indicating UE to switch its NR active BWP, the UE is allowed to cause interruptions to other active serving cells if the UE is not capable of per-FR gap. Otherwise the UE is allowed to cause interruptions to other active serving cells in the same FR range wherein the UE is performing BWP switching.

When a BWP timer [BWPtimer (editor’s notes: to be replaced according to RAN2 specification)] expires, UE is allowed to cause interruptions to other active serving cells if the UE is not capable of per-FR gap. Otherwise the UE is allowed to cause interruptions to other active serving cells in the same FR range wherein the UE is performing BWP switching.

#### 8.5.3.1	Interruptions at active BWP switching in EN-DC

The requirements in this clause shall apply for the UE configured with only NR PSCell or with NR PSCell and one or more NR SCells.

DCI-based or timer-based downlink BWP and/or uplink BWP switching due to change in bandwidth, center frequency or SCS in NR PSCell or in any NR SCell may cause an interruption on activated SCell(s) in the SCG.

When BWP switch occurs on the NR PSCell or NR SCell in EN-DC and UE is capable of per UE measurement gap, the UE is allowed to an interruption on any other serving NR serving cell:

-	of up to X slot, if the NR serving cell on which BWP switching occurs is not in the same band as the NR seving cell being interrupted, or

-	of up to Y slot + Δ if the NR serving cell on which BWP switching occurs is in the same band as the NR seving cell being interrupted;

When BWP switch occurs on the NR PSCell or NR SCell in EN-DC and UE is capable of per FR measurement gap, the UE is allowed to an interruption on any other serving NR serving cell:

-	of up to X slot, if the NR serving cell on which BWP switching occurs is in the same frequency range but not in the same band as the NR seving cell being interrupted, or

-	of up to Y slot + Δ if the NR serving cell on which BWP switching occurs is in the same band as the NR seving cell being interrupted

The values of X and Y are defined in Table 8.5.3.1-1.

Table 8.5.3.1-1 Interruption length X and Y at BWP switching

|  | NR Slot length (ms) | Interruption length X slot | Interruption length Y slot |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
| 1 | 0.5 | 1 | 1 |
| 2 | 0.25 | 3 | 2 |
| 3 | 0.125 | 5 | 4 |

Editor’s note: The value of Δ is FFS.

#### 8.5.3.2	Interruptions at active BWP switching in NR CA

The requirements in this clause shall apply for the UE configured with only NR PCell or with NR PCell and one or more NR SCells.

DCI-based or timer-based downlink BWP and/or uplink BWP switching due to change in bandwidth, center frequency or SCS in NR PCell or in any NR SCell may cause an interruption on PCell or on activated SCell(s).

When BWP switch occurs on the NR PCell or NR SCell in NR CA and UE is capable of per UE measurement gap, the UE is allowed to an interruption on the other NR serving cell

-	of up to the duration shown in Table 8.5.3.2-1, if the NR cell on which BWP switching occurs is not in the same band as the NR serving cell being interrupted, or

-	of up to the duration shown in Table 8.5.3.2-2, if the NR cell on which BWP switching occurs is in the same band as the NR serving cell being interrupted;

When BWP switch occurs on the NR PCell or NR SCell in NR CA and UE is capable of per FR measurement gap, the UE is allowed to an interruption on the other NR serving cell

-	of up to the duration shown in Table 8.5.3.2-1, if the NR cell on which BWP switching occurs is in the same frequency range but not in the same band as the NR serving cell being interrupted, or

-	of up to the duration shown in Table 8.5.3.2-2, if the NR cell on which BWP switching occurs is in the same band as the NR serving cell being interrupted;

Table 8.5.3.2-1: Interruption duration for BWP switching in inter-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |

Table 8.5.3.2-2: Interruption duration for BWP switching in intra-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 + Δ |
| 1 | 0.5 | 2 + Δ |
| 2 | 0.25 | 4 + Δ |
| 3 | 0.125 | 8 + Δ |

Editor’s note: The value of Δ is FFS.

## << End of Change >>