3GPP TSG-RAN WG4 Meeting #88                                                                         R4-1809883

# Gothenburg, Sweden, 20th – 26th August, 2018

Agenda Item:	7.11.4.4.1

Source:	Intel Corporation

Title:	Discussion about L1-RSRP measurement accuracy for beam management

Document for:	Discussion

# Introduction

In this contribution, we discuss the L1-RSRP measurement accuracy and measurement period for beam reporting.

# Received RSRP difference for Tx beams

For beam management, UE needs to measure the L1-RSRP for different beams and choose the proper beams to indicate to gNB. One of key remaining issues for beam management is the number of samples required for L1-RSRP measurement. For LTE and NR RSRP, the number of measurement is derived such that the minimum measurement accuracy requirement can be met. For L1-RSRP, similar approach should be taken.

However, RAN4 has never discussed how accurate of L1-RSRP measurement is accurate enough. The only thing we know is L1-RSRP accuracy should be better than SSB-based RSRP accuracy. The reason is L1-RSRP beam density should be higher than SSB-based RSRP beam density. To distinguish the beam with high beam density, the measurement should be more accurate.

Observation 1: it is difficult to conclude how many samples are enough for L1-RSRP measurement without knowing the corresponding measurement accuracy requirement.

To answer the question “how accurate L1-RSRP measurement is accurate enough?”, L1-RSRP difference between different TX beams is investigated. By statistically calculating the L1-RSRP difference, it will help to understand the “enough” measurement accuracy to distinguish these Tx beams. The simulation assumption is in Table 1.

Table 1. simulation assumption

| Simulation parameters | Comments/values |
| --- | --- |
| Tx beam configuration with 32 beams | 8*4(horizontal Tx elements number, vertical Tx elements number) |
| Tx beam configuration with 16 beams | 4*4(horizontal Tx elements number, vertical Tx elements number) |
| Tx beam configuration with 8 beams | 2*4(horizontal Tx elements number, vertical Tx elements number) |
| Tx beamforming | DFT codebook |
| UE receive antennas | 4*2(horizontal Rx elements number, vertical Rx elements number) |
| Rx beamforming | DFT codebook |
| Propagation conditions | CDL-C (delay spread: 30 ns UE speed: 3 km/h) |
| SINR | No noise and interference are considered. |
| Angle between gNB and UE | Random during drops |

Firstly, different beam densities are considered and defined by the number of beams, M, covering the whole spatial domain, where M=32/16/8. UE measurements the L1-RSRP for each of M Tx beams respectively and sort the beams in decreasing order based on measured L1-RSRP, i.e. the 1st beam is the beam with the highest L1-RSRP and the M-th beam is the one with the lowest L1-RSRP.

The CDF of RSRP differences are plotted in Fig.1 to Fig.3 for M=32/16/8 respectively. RSRP delta between 1th and 2th sorted beams is RSRP difference between the largest and the second largest beams. RSRP delta between 1th and 3th sorted beams is RSRP difference between the largest and the third largest beams, and so on. It is noted that the CDF in all figures is calculated based on geni L1-RSRP estimation.

| Fig.1 geni L1-RSRP delta for 32 TX beams | Fig.2 geni L1-RSRP delta for 16 TX beams |
| --- | --- |

Fig.3 Geni L1-RSRP delta for 8 TX beams

In Fig.1, we take the CDF curve “RSRP delta between 1st and 2nd sorted beam” as an example. 10 percentile of CDF curve shows the RSRP delta is 0.2dB. That means the measurement accuracy has to be +/-0.1dB in order to successfully distinguish between the 1st and 2nd beams in 90% case.

As another example as shown in the CDF curve “RSRP delta between 1st and 4th sorted beam”, 10 percentile results in 2.5dB L1-RSRP difference. That means the measurement accuracy has to be +/-1.25dB in order to successfully distinguish between the 1st and 4th beams in 90% case.

Additionally, it has been well known the eventual measurement accuracy should depend on three factors: RF margin (~1dB), baseband margin(~1dB) and inaccuracy due to measurement. That means it is hard to achieve the measurement accuracy less than 2dB due to the implementation margin. In other words, if the RSRP delta between beams are less than 4dB, UE won’t be able to distinguish which beam is the better than the other.

Observation 2: Due to implementation margin, the L1-RSRP measurement accuracy cannot be achieved within +/-2dB.

- If the RSRP delta between beams are less than 4dB, UE won’t be able to distinguish which beam is the better than the other.

- When Tx beam resolution in terms of RSRP delta is too high to be distinguishable, it won’t result in better L1-RSRP based beam management

Based on Fig.1-3, the resolution of beam and the corresponding L1-RSRP accuracy are summarized.

|  | Resolution of beams | Geni L1-RSRP difference at 10 percentile | L1-RSRP accuracy to achieve the resolution |
| --- | --- | --- | --- |
| Total # of tx beam is 8 | Distinguish between 1st strongest and the 2nd strongest | 0.4dB | N/A |
|  | Distinguish between 1st strongest and the 5th strongest | 9.2dB | ±2.6dB |
| Total # of tx beam is 16 | Distinguish between 1st strongest and the 2nd strongest | 0.3dB | N/A |
|  | Distinguish between 1st strongest and the 6th strongest | 6.2dB | ±1.1dB |
|  | Distinguish between 1st strongest and the 8th strongest | 9dB | ±2.5dB |
| Total # of tx beam is 32 | Distinguish between 1st strongest and the 2nd strongest | 0.2dB | N/A |
|  | Distinguish between 1st strongest and the 6th strongest | 4.5dB | ±0.25dB |
|  | Distinguish between 1st strongest and the 12th strongest | 9dB | ±2.5dB |

In this table, Resolution of beam management means the capability to distinguish the beam between the 1st strongest and Nth strongest, which give some guidelines about the likelihood the best-N beams can be identified and reported. For example, if the UE is requested to report one best beam, the reported beam can be within the best N beams in 90% cases. The beam resolution is obviously dependent on the Tx beam numbers, the more Tx beams, the more difficult to identify the best beam(s). If the L1-RSRP accuracy is defined as ±2.5dB as legacy LTE, it can only guarantee that the reported beam lies in the 11th/7th/4th best beams in 90% cases for 32/16/8 Tx beams. The corresponding angular spread is closer to 180 degrees.  If higher beam resolution is required, more strict L1-RSRP accuracy is required.

With these observations, Tx beam resolution and its corresponding L1-RSRP accuracy should be further studied. Since it can have great impact on the functionality of beam management. Only if L1-RSRP accuracy is defined, the sample number can be defined according to the accuracy requirements.

Observation 3: L1-RSRP accuracy with ±2.5dB can only guarantee that the reported beam can be within the best 5/8/12 beams for total 8/16/32 beams in 90% cases respectively.

Proposal 1: Tx beam resolution requirement and its corresponding L1-RSRP accuracy requirement should be studied first.

# Simulation results of measurement accuracy

For L1-RSRP based on CSI-RS, there is no agreed simulation assumptions. Here, we apply the agreed simulation parameter in [1] and change the SNR range to 0~12dB. The CSI-RS Periodicity is changed to 5ms from 40ms. The number of samples are changed to 1,2,4,5. The simulation results are provided in Fig.4.

Fig.4. simulation results of L1-RSRP accuracy

From the simulation results, it’s shown that the measurement accuracy is dependent on the CSI-RS density, bandwidth, channel model and subcarrier spacing. For single sample, the measurement accuracy is not good even for SNR=0 dB on some cases.  For example, the single sample accuracy is larger than 2.5 dB for ETU channel model, 24RB with D=1/3 in ETU channel and 96RB with D=1 in ETU channel model. When the SNR is lower, the measure accuracy will degrade further. According the analysis of section 2, L1-RSRP accuracy with ±2.5dB can only guarantee that the reported beam can be within the best 5/8/12 beams for total 8/16/32 beams in 90% cases respectively. Actually, the accuracy requirement will be tighter if better beam reporting quality is needed.

Proposal 2: single slot measurement can’t provide good accuracy for beam reporting.

# Measurement period of periodic CSI-RS based L1-RSRP

For periodic CSI-RS based L1-RSRP, there are two options for measurement period:

    - Option 1: No time domain averaging of L1-RSRP measurements are performed on UE side

    - Option 2: L1 averaging with X samples are assumed

For option 1, it assumes that the UE will apply the beam reporting based on single slot and it’s left to gNB to perform measurement averaging. However, there are some drawback for the report based on single slot. If there are multiple Tx beams, e.g. 16/32 beams, and UE is requested to report up to 4 beams.

UE will choose 4 beams just based on single slot measurement. Since the single slot measurement accuracy is not good enough as shown in section 3, the correct beam may not be chosen.  More seriously, the reported beams will change for different report time and it’s hard for gNB to average the measurement results. May be for some beams, they only has one reporting result and can’t be averaged. If gNB wait long time to get the result of some beams, the delay will be large either.

Observation 4: Correct beam can’t be reported based on single slot and reported beam can change during different reporting time. There maybe not enough samples to average at gNB side.

Therefore, for the UE side, it’s better to do some kind of averaging, which will help to improve the reporting quality. The sample number needs further discussion.

Proposal 3: For periodic CSI-RS beam reporting, L1 averaging with X samples are assumed at UE side and X value can be further studied.

There are no dependency of CSI-RS beam reporting interval and beam measurement period, If CSI-RS beam reporting interval and beam measurement period are identical, which is similar with RSRP measurement, there are always enough samples for averaging during measurement period. However, if CSI-RS beam reporting interval is less than beam measurement period, there maybe not enough samples for averaging. Similar with RLM, sliding window will be applied for sample averaging.

Proposal 4: For periodic CSI-RS beam reporting, if samples are not enough during beam reporting interval, sliding window will be applied for sample averaging.

# Measurement period of aperiodic CSI-RS based L1-RSRP

For aperiodic CSI-RS based L1-RSRP, if single sample measurement is applied, suppose UE receive DCI at time n, and it will send out beam reporting at n+M , where M is the measurement delay for single sample. However, since single sample can can’t provide enough accuracy, multiple samples averaging are still needed. Suppose X samples are needed for averaging.  gNB can send out candidate Tx beams for X times after it sends out DCI, then UE will send the beam reporting only when it finish the X sample averaging. Suppose UE receive DCI at time n, and it will send out beam reporting at n+P, where P is the measurement delay for multiple samples averaging. The report delay will be extended for multiple samples averaging case compared with single sample case.

Proposal 5: For aperiodic CSI-RS beam reporting, sample averaging are still needed and report delay will be extended.

# Side condition for SNR

Since the beam detection and reporting works mainly for serving cell, the channel quality should be not too bad. As a result, it is proposed to define SNR=0dB as the side condition for the measurement accuracy requirement.

Proposal 6: Define SNR=0dB as the side condition for the measurement accuracy requirement.

# Conclusion

Observation 1: it is difficult to conclude how many samples are enough for L1-RSRP measurement without knowing the corresponding measurement accuracy requirement.

Observation 2: Due to implementation margin, the L1-RSRP measurement accuracy cannot be achieved within +/-2dB.

- If the RSRP delta between beams are less than 4dB, UE won’t be able to distinguish which beam is the better than the other.

- When Tx beam resolution in terms of RSRP delta is too high to be distinguishable, it won’t result in better L1-RSRP based beam management

Observation 3: L1-RSRP accuracy with ±2.5dB can only guarantee that the reported beam can be within the best 5/8/12 beams for total 8/16/32 beams in 90% cases respectively.

Proposal 1: Tx beam resolution requirement and its corresponding L1-RSRP accuracy requirement should be studied first.

Proposal 2: single slot measurement can’t provide good accuracy for beam reporting.

Proposal 3: For periodic CSI-RS beam reporting, L1 averaging with X samples are assumed at UE side and X value can be further studied.

Proposal 4: For periodic CSI-RS beam reporting, if samples are not enough during beam reporting interval, sliding window will be applied for sample averaging.

Proposal 5: For aperiodic CSI-RS beam reporting, sample averaging are still needed and report delay will be extended.

Proposal 6: Define SNR=0dB as the side condition for the measurement accuracy requirement.

# References

[1] R4-1801301, “Simulation assumptions for CSI-RS based RLM”, Ericsson.