3GPP TSG-RAN WG4 Meeting #88                                      R4-1809935

Gothenburg, Sweden, 20th – 24th August, 2018

Agenda item:			7.11.7

Source:			Samsung, NTT DoCoMo, Inc.

Title:	Clarification on Sync/Async EN-DC scenarios

Document for:		Approval

# 1	Introduction

In last Oct. RAN4 meeting, there was WF approved for synchronous and asynchronous definitions for LTE-NR DC in Rel-15, with the following agreement for inter-band and intra-band EN-DC respectively [1]:

Furthermore, discussion on UE feature list made conclusion on the capability for Simultaneous reception and transmission for inter-band CA or EN-DC (TDD-TDD or TDD-FDD), and also Asynchronous FDD-FDD intra-band LTE-NR DC [2], and accordingly RAN4 has defined the RRC signaling.

However, in last RAN4 meeting (RAN4 AH1807), some companies propose to change to original agreement and ask RAN4 to design capability signaling to discriminate UE support of “synchronous EN-DC only” or “asynchronous EN-DC (i.e., support both synchronous and asynchronous EN-DC)” for inter-band TDD-FDD and inter-band TDD-TDD EN-DC combinations. With the above discussion history, we see the strong necessity of reconfirm the previous agreement and clarify the common understanding among companies in RAN4.

# Scenarios of EN-DC and UE Capability

With the above agreement taken into account, our understanding for EN-DC scenarios and corresponding capability can be summarized as below. Although it is a bit strange to mention both Sync and Async operations are mandatory supported by all UEs, since sync operation is naturally supported as long as UE can support async operation, here we would like to emphasize the previous agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”, in other words, there is no such group of UE which only support sync operation for TDD-TDD and TDD-FDD EN-DC.

Proposal 1: RAN4 reconfirm the previous agreement for valid EN-DC deployment scenarios and corresponding UE support/capability, summarized in below Table:

Table 1. Clarification on EN-DC deployment scenarios and UE support/capability in Rel-15

|  | FDD E-UTRA-FDD NR | TDD E-UTRA-TDD NR | TDD E-UTRA-FDD NR and FDD E-UTRA-TDD NR |
| --- | --- | --- | --- |
| Intra- band | Sync. (Mandatory supported by all UEs), Async. (Optional supported by some UEs, depends on UE capability IE asyncIntraBandENDC) Note-1: only support collocation for Rel-15 | Sync. (UE only support Sync operation for Rel-15) Note-1: only support collocation for Rel-15 | N.A. |
| Inter- band | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) (Additionally, Simultaneous TX and RX depends on UE capability RRC IE simultaneousRxTxInterBandENDC) | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) (Additionally, Simultaneous TX and RX depends on UE capability RRC IE simultaneousRxTxInterBandENDC) |

With the above understanding into account, RAN4 has reached the agreement for UE feature list [5] and ask RAN2 to consider using this feature list for UE capability definition, for which we captured as below:

Table 2. Related Features from RAN4 Approved UE Feature List [5]

| # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE | Consequences if the feature is not supported by the UE | Type (See R4-1712119) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | 1) Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) |  | Yes | UE does not support simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Type 3 (Per band combination) | No Need | No Need |  | Further discuss in RAN4 86bis whether to mandate this feature Whether to mandating this feature in certain band combinations will be FFS. | RAN4 | TBD |  |
| 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Asynchronous FDD-FDD intra-band EN-DC |  | Yes | UE does not support asynchronous FDD-FDD intra-band EN-DC | Type 3 (Per band combination) | Applicable only to FDD | Applicable only to FR1 |  |  | RAN4 | Optional |  |

And accordingly RAN4 define the capability signaling, for which we captured as below from TS38.331 v15.2.1:

MRDC-Parameters information element -- ASN1START -- TAG-MRDC-PARAMETERS-START MRDC-Parameters ::= SEQUENCE { singleUL-Transmission ENUMERATED {supported} OPTIONAL, dynamicPowerSharing ENUMERATED {supported} OPTIONAL, tdm-Pattern ENUMERATED {supported} OPTIONAL, ul-SharingEUTRA-NR ENUMERATED {tdm, fdm, both} OPTIONAL, ul-SwitchingTimeEUTRA-NR ENUMERATED {type1, type2} OPTIONAL, simultaneousRxTxInterBandENDC ENUMERATED {supported} OPTIONAL, asyncIntraBandENDC ENUMERATED {supported} OPTIONAL, ... } -- TAG-MRDC-PARAMETERS-STOP -- ASN1STOP

Observation 1: RAN2 and RAN4 have clear agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”, and no UE capability signaling is needed for this.

From Montreal meeting’s offline discussion, we found some company mixed the UE feature “simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD)” with the UE’s mandatory support of async operation of inter-band EN-DC. Specifically, some company may claim that for some band combinations in which certain UEs are allowed to not support “simultaneous reception and transmission” (by not explicitly indicating simultaneousRxTxInterBandENDC=support), it is hard to deploy asynchronous EN-DC network to let these UEs to access. Even though this is true for some band combinations; however, we don’t believe it is the reason to allow UE only support of “synchronous inter-band EN-DC” and revert the previous agreement. Here, we would like to explain as below in details:

- UE’s capability of “simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD)” is a separate issue to be considered in UE RF room, which depends on RF architecture for each band combination, while RAN4 made the agreement of “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15” from network deployment aspects.

- UE’s mandatory support of “simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD)” in some band combinations is discussed in RF room. As the consequence, for large number of band combinations, UE need to have simultaneous TXRX as mandatory feature. In other words, for these band combinations, this feature is not possible to be the limitation for UE to support “asynchronous inter-band EN-DC (i.e., support both synchronous and asynchronous inter-band EN-DC)”.

- Even with current signaling design, all UEs are needed to support “asynchronous inter-band EN-DC (i.e., support both synchronous and asynchronous inter-band EN-DC)”, for some band combinations on which UE are allowed to not support “simultaneousRxTxInterBandENDC” due to RF design, UE can still use this signaling to indicate it can’t maintain simultaneous reception and transmission for this band combination and network should consider this capability when this UE is configured with TDD configuration and scheduled.

Proposal 2: RAN4 reconfirm the agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”.

Proposal 3: No need to introduce UE capability signaling to discriminate UE support of “synchronous EN-DC only” or “asynchronous EN-DC (i.e., support both synchronous and asynchronous EN-DC)” for inter-band TDD-FDD and inter-band TDD-TDD EN-DC combinations.

# Conclusion

In this paper, we provided our clarification on the understanding for the scenarios for EN-DC and corresponding capability signaling, due to the necessity of reconfirm the previous agreement and clarify the common understanding among companies in RAN4, while the following observation and proposals are provided:

Proposal 1: RAN4 reconfirm the previous agreement for valid EN-DC deployment scenarios and corresponding UE support/capability, summarized in below Table:

Table 1. Clarification on EN-DC deployment scenarios and UE support/capability in Rel-15

|  | FDD E-UTRA-FDD NR | TDD E-UTRA-TDD NR | TDD E-UTRA-FDD NR and FDD E-UTRA-TDD NR |
| --- | --- | --- | --- |
| Intra- band | Sync. (Mandatory supported by all UEs), Async. (Optional supported by some UEs, depends on UE capability IE asyncIntraBandENDC) Note-1: only support collocation for Rel-15 | Sync. (UE only support Sync operation for Rel-15) Note-1: only support collocation for Rel-15 | N.A. |
| Inter- band | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) (Additionally, Simultaneous TX and RX depends on UE capability RRC IE simultaneousRxTxInterBandENDC) | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) (Additionally, Simultaneous TX and RX depends on UE capability RRC IE simultaneousRxTxInterBandENDC) |

Observation 1: RAN2 and RAN4 have clear agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”, and no UE capability signaling is needed for this.

Proposal 2: RAN4 reconfirm the agreement that “for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15”.

Proposal 3: No need to introduce UE capability signaling to discriminate UE support of “synchronous EN-DC only” or “asynchronous EN-DC (i.e., support both synchronous and asynchronous EN-DC)” for inter-band TDD-FDD and inter-band TDD-TDD EN-DC combinations.

# Reference

[1] R4-1711964, “WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15”, Ericsson, MediaTek.

[2] R4-1803551, “WF on UE feature list for CA/EN-DC, other capability and LTE-NR co-existence”, Intel.

[3] R4-1803849, “Clarification on Sync/Async EN-DC scenarios and Correction to Timing Requirement”, Samsung.

[4] R4-1808967, “Further discussions on synchronous/asynchronous definitions for inter-band EN-DC”, Ericsson.

[5] R4-1803564, “LS on UE feature list”, NTT DoCoMo.