3GPP TSG-RAN WG4 Meeting #88                                           R4-1809958

Gothenburg, SE, 20th–24th Aug, 2018

Agenda Item:   7.8.3.5

Source:        Samsung

Title:           Disucussion on absolute ACLR for MMW

Document for:  Discussion

# Introduction

In RAN4#87 meeting there was proposal to update absolute ACLR levels to align with corresponding OBUE requirement in range within adjacent channel up to 10% total transmission bandwidth for FR2 NR BS[1][2]. However, no agreement achieved during the discussion.

In this contribution, we elaborate on this issue further from the other angle for RAN4 consideration.

# Discussion

As we known the out of band emission requirement is specified both in terms of a OBUE（band centric way mask）and ACLR for the BS transmitter of LTE and NR. As both an ACLR-type requirement with carrier-wide reference bandwidth (relative ACLR) and OBUE mask with much narrower reference bandwidth, the ACLR limit is somewhat stricter than the integrated SEM. In this way, the relative ACLR can capture the “average” behavior over a carrier, while the OBUE mask can take into account the variations in the spectrum emissions resulting from variations in power allocations.

Absolute ACLR is defined on the same adjacent channel but with relative narrower reference bandwidth compared with relative ACLR. If compared absolute ACLR and OBUE for FR1 what we can find is for the 0-10MHz range adjacent transmission NR signal the OBUE limit is still relaxed than absolute ACLR. However, considering the reference bandwidth of OBUE is still conspicuously narrower than absolute ACLR, which is 100KHz Vs. 1MHz, somehow it still obeys the rule mentioned above for relative ACLR and OBUE.

Table 1: Comparison between absolute ACLR and OBUE for FR1 WA BS for NR operating band above 1GHz

| FR1 WA BS Cate A | Absolute ACLR | OBUE | OBUE | OBUE |
| --- | --- | --- | --- | --- |
| Emission requirement | -13dBm/MHz | [-7dBm/100KHz, -14dBm/100KHz | -14dBm/100KHz | -13dBm/1MHz |
| Applicable frequency range | Assumed adjacent channel carrier with same BW as edge CC | 0-5MHz | From 5 MHz outside the CC edge to min(10 MHz, fmax) | 10MHz to OOB boundary |

However, if we take a look at FR2 absolute ACLR and OBUE requirement, the situation is quite different compared with FR1 as shown in below table 2.

Table 2: Comparison between absolute ACLR and OBUE for FR2 WA BS

|  | Absolute ACLR | OBUE | OBUE |
| --- | --- | --- | --- |
| Emission requirement | -13dBm/MHz | [-5dBm/MHz, -12dBm/ MHz] | [-13dBm/MHz, -20dBm/ MHz] |
| Applicable frequency range | Assumed adjacent channel carrier with same BW as edge CC | Up to 0.1*BWcontiguous | From 0.1*BWcontiguous to OOB boudary |

It can be seen for FR2 the absolute ACLR emission limit is still severer than OBUE in the range up to 0.1*BWcontiguous range. However, the reference bandwidth for absolute ACLR and OBUE in adjacent channel within above range is the same, i.e., per MHz.

Hence it would be beneficial to think about the issue here, e.g., whether the reference bandwidth of absolute ACLR should be increased to obey the similar principle as FR1 and LTE.

# Conclusion

It is encouraged RAN4 to consider whether there is potential issue for current absolute ACLR definition for RF2. If it problem can be recognized among RAN4. Two potential alternatives can be discussed to resolve the issue as:

Alternative 1: update the reference BW of absolute ACLR requirement according to [respective assumed adjacent channel carrier BW or other value lager than 1MHz] and emission PSD still as -13dBm/MHz or -20dBm/MHz

Alternative 2: maintain the core requirement for absolute ACLR but update the measurement BW in conformance spec.

One example for absolute ACLR limit with reference (measurement) BW same as relative ACLR is shown as table below.

| BW of assumed adjacent channel carrier | Absolute ACLR limit for WA BS | Absolute ACLR limit for MR and LA BS |
| --- | --- | --- |
| 50MHz | 3.7dBm/50MHz | -3.3dBm/50MHz |
| 100MHz | 6.7dBm/100MHz | -0.3dBm/100MHz |
| 200MHz | 9.7dBm/200MHz | 2.7dBm/200MHz |
| 400MHz | 12.7dBm/400MHz | 5.7dBm/400MHz |

# Reference

[1] R4-1807258, On absolute ACLR limits for FR2 NR BS

[2] R4-1807267, Draft CR for TS 38.104: Absolute ACLR for FR2

[3] TS38.104-f20: NR Base Station (BS) radio transmission and reception