3GPP TSG-RAN WG4 Meeting RAN4#88                                               R4-1809966

Gotherburg ,Sweden, 20 -24 Aug, 2018

Agenda Item:   7.13.1.5

Source:        Samsung

Title:          Over views on CSI test cases

Document for:  Discussion

Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (FFS for wideband or sub-band) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, several open issues were discussed:

- Test scope

- Common parameters

- CSI-RS configuration

- CSI feedback delay

Discussion

Test Scope

Semi-persistent CSI report and semi-persistent CSI-RS resources

Based on RAN4#80 agreed UE feature list, SP CSI reporting is optional meanwhile SP CSI-RS is mandatory with capability signalling.

For test scope, it’s still FFS whether introduce test cases for SP CSI reporting type and SP CSI-RS resources. From network scheduling flexibility perspective, supporting flexible and configurable CSI reporting types and CSI-RS resource setting is important, it’s better to cover all the types in Rel-15. Furthermore, test case for aperiodic CSI-RS resource configuration can be treated as functionality test cases. Test requirements and test configurations from test cases with periodic CSI-RS resource configurations can be reused. From RAN4 work load aspect, no additional test effort and simulation work required.

Proposal1: Introduce CSI test case(s) covering SP CSI-RS resources with 2nd priority in Rel-15. For CSI reporting type only focused on aperiodic and periodic CSI reporting in Rel-15.

Reporting content

It’s still FFS whether introduce CRI/CSI test cases in Rel-15, also still FFS whether covering CRI/L1-RSRP in RRM or CSI test cases.

In NR, CSI consists of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), layer indication (LI), rank indication (RI) and/or L1-RSRP.

For CRI, it’s quite beneficial for Hybrid beam forming operation:

- Operation 1: Used for hybrid PMI reporting, combined with PMI report

- Operation 2: Used for beam management, combined with L1-RSRP report

One of key features introduced in NR is massive MIMO (Hybrid beamforming) to improve data transmission and cell coverage which applied for high frequency in FR1 i.e. above 2.6GHz and FR2.

In order to facilitate hybrid beam operation, we preferred to introduce both CRI/CSI test case and CRI/L1-RSRP test cases.

CSI reporting content

In NR, CSI consists of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), layer indication (LI), rank indication (RI) and/or L1-RSRP.

For CRI, it’s quite beneficial for Hybrid beam forming operation:

- Operation 1: Used for hybrid PMI reporting, combined with PMI report

- Operation 2: Used for beam management, combined with L1-RSRP report

In order to facilitate hybrid beam operation, we preferred to introduce both CRI/CSI test case and CRI/L1-RSRP test cases.

P2: Introduce both CRI/CSI test case and CRI/L1-RSRP test cases in Rel-15.

Common parameters

The major purpose of CSI test cases is to verify UE reporting accuracy of CSI contents, the other physical features and variable configurations can be verified in UE demodulation requirements. We propose to follow the assumption of demodulation and try to select only one configuration if several configurations used in demodulation test cases.

P3: Common parameter for CSI test given in table below

| Parameters | Configurations |
| --- | --- |
| PDSCH scheduling | - PBCH overhead: Skip scheduling PDSCH for PBCH slot i.e. slot 0 per radio frame (10ms periodicity) - Mapping type: PDSCH mapping type A - Bundling size: 2 PRB bundling - Precoding model: specific to test cases with codebook configurations |
| Control symbols assumption | - FR1: Symbols #0 and 1 in each slot - FR2: Symbol #0 in each slot - Do not schedule PDSCH in in control symbols duration |
| DMRS | - Type: Type 1 DMRS - Length: 1 additional DMRS symbol - between DMRS and data for rank 1/2: No FD |
| TRS | Slots pattern, with offset 0 slot, Bandwidth: full BWP,0 dB boosting |
| PTRS | - FR1: PTRS is not configured - FR2: port 1, per 2PRB in frequency domain, per symbol in time domain |
| Special sub-frame assumption | skip scheduling PDSCH in special sub-frames |
| RV sequence | {0,2,3,1} only valid for PMI test cases, no HARQ retransmission for CQI and RI test cases |

TDD configurations

Several configurations can be verified in demodulation test cases, for CSI test cases, using only one configuration each SCS to simplify test case design, meanwhile we also need to consider compact pattern to facilitate CSI feedback in UL sub-frames. With above considerations, below DL-UL configuration proposed:

P4: Using below TDD DL-UL configuration for CSI test cases

  - FR1 30kHz SCS: DDDSU, S=10D:2G:2U

  - FR2 120kHz SCS: DDSU, S=11D+3G

CSI computation delay and gNB emulator scheduling delay assumption

As agreed in TS38.214, for periodic CSI feedback, CSI computation delay agreed as no less than 4ms corresponding to scalable number of slot pending on numerology. For gNB emulator scheduling delay, we can assume same value as UE side 4 ms.

P5: Periodic CSI feedback delay can be assumed as below:

  - Minimum CSI computation delay 4ms

  - Minimum gNB emulator scheduling delay 4ms

Then for periodic CSI feedback, CSI delay between CSI-RS reference sub-frame and corresponding scheduling PDSCH followed CSI feedback from reference sub-frame should no less than 8ms pending on TDD DL-UL configuration.

  - For FDD, CSI delay 8ms (4ms for UE computation, 4ms for gNB emulator scheduling)

  - For TDD, pending on DL-UL configuration, 8ms ~12ms

For aperiodic CSI feedback, CSI computation time defined in TS38.214 5.4 pending on whether CSI multiplexed with ACK/PUSCH , CPUs occupied conditions, different Z and Z’ values applied as defined in table 5.4-1 and 5.4-2.

P6: Aperiodic CSI feedback delay followed the definition in TS38.214 5.4, assuming same processing delay for UE computation and gNB emulator processing delay.

CSI-RS configuration

Based on agreed test case list, 2Tx, 4Tx and 8Tx CSI-RS ports will be configured for CSI test cases. Based on Table 7.4.1.5.3-1: CSI-RS locations within a slot in TS38.211, below configuration can be used for CSI-RS ports:

| Parameters | 2 Ports | 4 ports | 8 ports |
| --- | --- | --- | --- |
| nrofPorts | 2 | 4 | 8 |
| firstOFDMSymbolInTimeDomain | 5 | 5 | 5 |
| firstOFDMSymbolInTimeDomain2 | N.A | N.A | N.A |
| cdm-Type | fd-CDM2 | fd-CDM2 | cdm4-FD2-TD2 |
| density | 1 | 1 | 1 |

For CSI-RS periodicity and slot offset, we proposed to fix with 5ms periodicity and slot offset =1:

,

Conclusion

In this contribution, preliminary views for the scope of UE CSI requirements were provided.

CSI scope

P1: Introduce CSI test case(s) covering SP CSI-RS resources with 2nd priority in Rel-15. For CSI reporting type only focused on aperiodic and periodic CSI reporting in Rel-15.

P2: Introduce both CRI/CSI test case and CRI/L1-RSRP test cases in Rel-15.

Test set-up

P3: Common parameter for CSI test given in table above.

P4: Using below TDD DL-UL configuration for CSI test cases

  - FR1 30kHz SCS: DDDSU, S=10D:2G:2U

  - FR2 120kHz SCS: DDSU, S=11D+3G

CSI feedback delay

P5: Periodic CSI feedback delay can be assumed as below:

  - Minimum CSI computation delay 4ms

  - Minimum gNB emulator scheduling delay 4ms

P6: Aperiodic CSI feedback delay followed the definition in TS38.214 5.4, assuming same processing delay for UE computation and gNB emulator processing delay.

CSI-RS configuration

P7: using below RRC configurations for CSI-RS port IE mapping:

| Parameters | 2 Ports | 4 ports | 8 ports |
| --- | --- | --- | --- |
| nrofPorts | 2 | 4 | 8 |
| firstOFDMSymbolInTimeDomain | 5 | 5 | 5 |
| firstOFDMSymbolInTimeDomain2 | N.A | N.A | N.A |
| cdm-Type | fd-CDM2 | fd-CDM2 | cdm4-FD2-TD2 |
| density | 1 | 1 | 1 |

P8: For CSI-RS periodicity and slot offset, we proposed to fix with 5ms periodicity and slot offset =1:

- ,

Reference

[1] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung