3GPP TSG-RAN WG4 Meeting RAN4#88                                               R4-1809969

Gotherburg ,Sweden, 20 -24 Aug, 2018

Agenda Item:   7.13.1.5

Source:        Samsung

Title:          Test case design for fading CQI test cases

Document for:  Discussion

Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (FFS for wideband or sub-band) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for PMI test cases provided.

Discussion

Simulation assumption

| Parameter | Unit | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | FFS | FFS |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | FFS |
| Correlation and antenna configuration |  | 8*2 8*4 [XP spatially High] | 8*2 8*4 [XP spatially High] | 2*2 FFS |
| CSI-RS resource setting (aperiodic) |  |  |  |  |
| resourceConfigType |  | aperiodic | aperiodic | aperiodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5/0 | 5/0 | 5/0 |
| NZP CSI-RS ports |  | 8 port {3000…3007} | 8 port {3000…30007} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [CDM4,density 1 (row 9)] | [CDM4,density 1 (row 9)] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (aperiodic) |  |  |  |  |
| reportConfigType |  | aperiodic | aperiodic | aperiodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| (N1,N2) |  | (2,2) | (2,2) | N.A |
| (O1,O2) |  | (4,4) | (4,4) | N.A |
| CodebookMode |  | 1 | 1 | N.A |
| Codebook Restriction |  | FFS | FFS | FFS |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Rank |  | 1/2 | 1/2 | 1/2 |
| FRC |  | MCS4/MCS13 | MCS4/MCS13 | MCS4/MCS13 |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Additional for sub-band PMI, we also evaluated with 4Tx Sub-band PMI test cases in FR1 with TDL-C 100ns and 10Hz channel model.

For MCS and rank, two options evaluated:

- Option 1: MCS 4 with rank1

- Option 2: MCS 13 with rank2

Simulation results

Simulation results for FR1 FDD

| Figure 1 MCS 4 Rank1 with 8Tx2Rx WB PMI |  |
| --- | --- |

| Figure 2 MCS 13 Rank2 with 8Tx4Rx WB PMI |  |
| --- | --- |

| Figure 3 MCS 4 Rank1 with 4Tx2Rx WB PMI |  |
| --- | --- |
| Figure 4 MCS 4 Rank1 with 4Tx4Rx WB PMI |  |

Simulation results for FR1 TDD

| Figure 5 MCS 4 Rank1 with 8Tx2Rx WB PMI |  |
| --- | --- |

| Figure 6 MCS 13 Rank2 with 8Tx4Rx WB PMI |  |
| --- | --- |

| Figure 7 MCS 4 Rank1 with 4Tx2Rx WB PMI |  |
| --- | --- |
| Figure 8 MCS 4 Rank1 with 4Tx4Rx WB PMI |  |

Reference SNR points with following PMI and relative TP summarized in table below for FR1

| Test cases | FDD | TDD |
| --- | --- | --- |
| 4Tx2Rx WB PMI with MCS4 rank1 | -3.8dB/2.2 | -1.8dB/1.68 |
| 4Tx4Rx WB PMI with MCS13 rank2 | 10.0dB/1.46 | 10.5dB/1.36 |
| 8Tx2Rx WB PMI with MCS4 rank1 | <-5dB/N.A | -4.2dB/3.5 |
| 8Tx4Rx WB PMI with MCS13 rank2 | 7.3 dB/2.1 | 9.2dB/1.7 |

Simulation results for FR2 TDD

To be updated

Analysis

For FR1 4Tx PMI test cases, it’s still FFS whether wideband or sub-band PMI test cases. In order to ensure test coverage with different Tx ports, we preferred to use sub-band PMI test cases.

P1: Using sub-band mode for 4Tx PMI test cases

For 8 TX ports: (N1,N2) = (2,2)/(4,1), (O1, O2) = (4,4)/(4,1), two configurations list in last RAN4 meeting. From UE processing aspect, configuration 1 has larger codebook size then configuration 2 (4 times compared to configuration2).

In order to verify UE processing capability and save RAN4 work load and test effort, we proposed to use configuration 1 introducing 8Tx PMI test cases.

P2: Using (N1, N2) =(2,2), (O1,O2) = (4,4) for 8ports PMI test cases

For proper MCS selection, we observed that SNR points at 70% TP with rank1 transmission for following PMI is lower than 0dB which is not favourable to introduce test cases. With above observations, we proposed to use MCS 13 Rank1 for 4Tx test cases and MCS 13 rank2 for 8Tx test cases in FR1.

P3: Using MCS 13 Rank1 for 4Tx PMI test cases, MCS 13 Rank2 for 8Tx PMI test cases

For test metric and test point, it’s feasible reusing LTE test metric and test point to differentiate different UE behaviours and PMI reporting accuracy in FR1.

P4: Reusing LTE test metric and test point at least for FR1 test PMI test cases: Relative Throughput ratio between following PMI and random PMI at SNR point corresponding to 70% TP with follow PMI.

Conclusion

In this contribution, we provided initial simulation results for PMI test cases with below observations:

- For proper MCS selection, we observed that SNR points at 70% TP with rank1 transmission for following PMI is lower than 0dB which is not favourable to introduce test cases. With above observations, we proposed to use MCS 13 Rank1 for 4Tx test cases and MCS 13 rank2 for 8Tx test cases in FR1.

- For test metric and test point, it’s feasible reusing LTE test metric and test point to differentiate different UE behaviours and PMI reporting accuracy in FR1.

P1: Using sub-band mode for 4Tx PMI test cases

P2: Using (N1, N2) =(2,2), (O1,O2) = (4,4) for 8ports PMI test cases

P3: Using MCS 13 Rank1 for 4Tx PMI test cases, MCS 13 Rank2 for 8Tx PMI test cases

P4: Reusing LTE test metric and test point at least for FR1 test PMI test cases: Relative Throughput ratio between following PMI and random PMI at SNR point corresponding to 70% TP with follow PMI.

Reference

[1] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung