3GPP TSG-RAN WG4 #88 Meeting                                     R4-1809986

Gothenburg, Sweden, August 20-24, 2018

Agenda item:	7.13.2.1

Source: 	ZTE

Title: 	On NR BS demodulation general aspects

Document for:	Discussion

Introduction

During RAN4 #AH-1807 the Way forward on General part of NR BS demodulation performance were approved [1]. Some aspects such as TDD UL-DL configurations need to be further studied. This contribution further discuss TDD UL-DL configurations.

TDD UL-DL configurations

In real network deployment, the TDD UL-DL configuration is selected by considering the factors including uplink/downlink traffic, coverage and latency performance. According to prediction of the traffic in 5G, uplink and downlink resource allocation is good to be 1:2~1:3 for NR deployment.

For FR1 SCS 30 kHz case, DDSU (S=10D:2G:2U) has good latency performance and good UL-DL balance 1:2.4. DDDSUDDSUU (S=10D:2G:2U) also has good latency performance and good UL-DL balance 1:2 meanwhile has better coverage due to more SSBs could be transmitted. We slightly prefer DDDSUDDSUU (S=10D:2G:2U). So we have the following proposal:

Proposal 1: For FR1, the following TDD UL-DL configurations will be used in BS demodulation test setup:

- For SCS 30 kHz:

  - 1st priority:   DDDSUDDSUU,  S=10D:2G:2U

  - 2nd priority:  DDSU,  S=10D:2G:2U

For FR2, DDSU (S=8D:3G:3U) has good latency performance with fast feedback meanwhile has good UL-DL balance 1:2. So we have the following proposal:

Proposal 2: For FR2, the following TDD UL-DL configurations will be used in BS demodulation test setup:

- For both of SCSs 120 and 60 kHz:

  - DDSU,  S=8D:3G:3U

Conclusions

Based on the discussions above, we have the following proposals:

Proposal 1: For FR1, the following TDD UL-DL configurations will be used in BS demodulation test setup:

- For SCS 30 kHz:

  - 1st priority:   DDDSUDDSUU,  S=10D:2G:2U

  - 2nd priority:  DDSU,  S=10D:2G:2U

Proposal 2: For FR2, the following TDD UL-DL configurations will be used in BS demodulation test setup:

- For both of SCSs 120 and 60 kHz:

  - DDSU,  S=8D:3G:3U

# References

- R4-1809370 Way forward on General part of NR BS demodulation performance, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, CATT, July 2018.