3GPP TSG-RAN WG4 Meeting #88	R4-1810128

# Gothenburg, Sweden, 20th August – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR TS 38.101-3: Single UL allowed corrections for EN-DC dual uplink operation in NR FR1 (two bands) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Skyworks Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Single UL allowed requirements for DC_1A_n77A, DC_2A_n66A, DC_2A_n78A, DC_3A_n77A, DC_3A_n78A, DC_3C_n78A, DC_5A_n66A, DC_20A_n8A EN-DC combinations are either wrong, missed or not reflected in a consistent manner in table 5.2B.4.1-1 and table 7.3B.2.3.5.1-1. Detailed analysis can be found in R4-1810113. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In table 5.2B.4.1-1, replace “No” in column “Single UL allowed” for DC_2_n66, DC_2_n78, DC_5_n66, and DC_20_n8 with “DC_2_n66”, “DC_2_n78”, “DC_5_n66” and “DC_20_n8” respectively. In table 7.3B.2.3.5.1-1, add “Yes” in column “Single UL allowed” for DC_1A_n77A (B1 UL = 1950 MHz, UL n77 = 4090 MHz), DC_2A_n66A (B2 UL = 1855 MHz, n66 UL = 1775 MHz), DC_2A_n78A (B2 UL = 1855 MHz, n78 UL = 3795 MHz), DC_3A_n77A, DC_3A_n78A, DC_3C_n78A (B3 UL= 1740 MHz, n77/n78 UL = 3575 MHz) and DC_5A_n66A (B5 UL = 838 MHz, n66 UL = 1721 MHz). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Single UL operation is not correctly captured. This may impact signalling messages and conformance test documents. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B.4.1 and 7.3B.2.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ... 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### <Start of changes>

#### 5.2B.4.1	EN-DC (two bands)

Table 5.2B.4.1-1: Band combinations for EN-DC (two bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n28 | 1 | n28 | No |
| DC_1_n40 | 1 | n40 | No |
| DC_1_n51 | 1 | n51 | No |
| DC_1_n773 | 1 | n77 | DC_1_n77 |
| DC_1_n783 | 1 | n78 | No |
| DC_1_n793 | 1 | n79 | No |
| DC_2_n5 | 2 | n5 | No |
| DC_2_n66 | 2 | n66 | DC_2_n66 |
| DC_2_n71 | 2 | n71 | No |
| DC_2_n78 | 2 | n78 | DC_2_n78 |
| DC_3_n7 | 3 | n7 | No |
| DC_3_n28 | 3 | n28 | No |
| DC_3_n40 | 3 | n40 | No |
| DC_3_n51 | 3 | n51 | No |
| DC_3_n773 | 3 | n77 | DC_3_n77 |
| DC_3_n783 | 3 | n78 | DC_3_n78 |
| DC_3_n793 | 3 | n79 | No |
| DC_5_n40 | 5 | n40 | No |
| DC_5_n66 | 5 | n66 | DC_5_n66 |
| DC_5_n783 | 5 | n78 | No |
| DC_7_n28 | 7 | n28 | No |
| DC_7_n51 | 7 | n51 | No |
| DC_7_n783 | 7 | n78 | No |
| DC_8_n40 | 8 | n40 | No |
| DC_8_n773 | 8 | n77 | No |
| DC_8_n783 | 8 | n78 | No |
| DC_8_n793 | 8 | n79 | No |
| DC_11_n773 | 11 | n77 | No |
| DC_11_n783 | 11 | n78 | No |
| DC_11_n793 | 11 | n79 | No |
| DC_12_n5 | 12 | n5 | No |
| DC_12_n66 | 12 | n66 | No |
| DC_18_n773 | 18 | n77 | No |
| DC_18_n783 | 18 | n78 | No |
| DC_18_n793 | 18 | n79 | No |
| DC_19_n773 | 19 | n77 | No |
| DC_19_n783 | 19 | n78 | No |
| DC_19_n793 | 19 | n79 | No |
| DC_20_n8 | 20 | n8 | DC_20_n8 |
| DC_20_n284 | 20 | n28 | No |
| DC_20_n51 | 20 | n51 | No |
| DC_20_n77 | 20 | n77 | No |
| DC_20_n783 | 20 | n78 | No |
| DC_21_n773 | 21 | n77 | No |
| DC_21_n783 | 21 | n78 | No |
| DC_21_n793 | 21 | n79 | No |
| DC_25_n41 | 25 | n41 | No |
| DC_26_n413 | 26 | n41 | No |
| DC_26_n773 | 26 | n77 | No |
| DC_26_n783 | 26 | n78 | No |
| DC_26_n793 | 26 | n79 | No |
| DC_28_n51 | 28 | n51 | No |
| DC_28_n773 | 28 | n77 | No |
| DC_28_n783 | 28 | n78 | No |
| DC_28_n793 | 28 | n79 | No |
| DC_30_n5 | 30 | n5 | No |
| DC_30_n66 | 30 | n66 | No |
| DC_38_n78 | 38 | n78 | No |
| DC_39_n781,3 | 39 | n78 | No |
| DC_39_n793 | 39 | n79 | No |
| DC_40_n77 | 40 | n77 | No |
| DC_41_n77 | 41 | n77 | No |
| DC_41_n78 | 41 | n78 | No |
| DC_41_n792 | 41 | n79 | No |
| DC_42_n51 | 42 | n51 | No |
| DC_42_n77 | 42 | n77 | No |
| DC_42_n78 | 42 | n78 | No |
| DC_42_n79 | 42 | n79 | No |
| DC_66_n71 | 66 | n71 | No |
| DC_66_n5 | 66 | n5 | No |
| DC_66_n78 | 66 | n78 | No |
| NOTE 1: The frequency range above 3600MHz for Band n78 is not used in this combination. NOTE 2: The frequency range below 2545MHz for Band 41 is not used in this combination. NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 4: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### <Unchanged sections omitted>

###### 7.3B.2.3.5.1	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 32.54 |  |  |  |
|  | n77 | 4090 | 10 | 25 | 4090 | N/A | TDD | N/A |  |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n77 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD |  |  |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 | Yes |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | TDD | N/A |  |
| DC_2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |  |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | TDD | IMD5 |  |
| DC_2A_n78A | 2 | 1855 | 5 | 25 | 1940 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n78 | 3795 | 10 | 25 | 3795 | N/A | TDD | N/A |  |
| DC_2A_n78A | 2 | 1885 | 5 | 25 | 1955 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3700 | 10 | 25 | 3700 | N/A | TDD | N/A |  |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |  |
|  | n7 | 2535 | 10 | 52 | 2655 | 10.25 | FDD | IMD4 |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n77, n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n77, n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_3A_n78A | 3 | 1712.5 | 5 | 25 | 1807.5 | TBD5 | FDD | IMD2 | Yes |
|  | n78 | 3515 | 10 | 50 | 3515 | N/A | TDD | N/A |  |
|  | 3 | 1762.5 | 5 | 25 | 1857.5 | N/A | FDD | N/A | No |
|  | n78 | 3465 | 10 | 50 | 3465 | N/A | TDD | N/A |  |
| DC_3A-SUL_n78A-n80A, DC_66A-SUL_n78A-n86A | 3, 66 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  | Yes |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A | Yes |
| DC_3A_SUL_n78A-n80A, DC_66A-SUL_n78A-n86A | 3, 66 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 | No |
|  |  |  |  |  |  | 10.74 |  |  | No |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A | No |
| DC_3C_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 | Yes |
|  |  |  |  |  |  | 28.75 |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_3C_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |  |
|  |  |  |  |  |  | 10.75 |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 | Yes |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |  |
|  | n77, n78 | 3635 | 10 | 52 | 3635 | N/A | TDD | H4 |  |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |  |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |  |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | FDD | IMD3 | Yes |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | FDD | IMD3 |  |
| DC_20A_n77A | 20 | 850 | 5 | 25 | 810 | 11 | FDD | IMD4 |  |
|  | n77 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
|  | 20 | 840 | 5 | 25 | 800 | 6.5 | FDD | IMD5 |  |
|  | n77 | 4160 | 10 | 50 | 4150 | N/A | TDD | N/A |  |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | 850 | 5 | 25 | 810 | 21.7 | FDD | IMD44 |  |
|  | n78 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | FDD | IMD3 |  |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | TDD | N/A |  |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD3 |  |
|  | n41 | 2562 | 10 | 52 | 2562 | N/A | TDD | N/A |  |
| DC_28A_n51A | 28 | 725.5 | 20 | 25 | 765.5 | 5 | FDD | IMD 4, 5 | Yes |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | 5 | TDD | IMD 4, 5 |  |
| DC_26A_n77A DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |  |
|  | n77, n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |  |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |  |
|  | n77, n78 | 3582.5 | 10 | 25 | 3582.5 | N/A | TDD | N/A |  |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 |  |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |  |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |  |
| DC_66A_n78A | 66 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 |  |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | 66 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A. In case Single UL is allowed and the UE only indicates support of “Single UL” the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs |  |  |  |  |  |  |  |  |  |

#### <End of changes>