3GPP TSG-RAN WG4 Meeting #88	R4-1810140

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.6.4.1

Source:	Ericsson

Title:	Simulation results of PDSCH demodulation for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the evaluation parameters for PDSCH demodulation with 1024QAM as follow [1].

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 2 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |
| 3 | TM9 | Option 1: Wideband PMI Feedback Option 2: Per-PRG Random Precoder | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 4 | TM9 | Option 1: Wideband PMI Feedback Option 2: Per-PRG Random Precoder | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |

- Test cases for demodulation fading tests for 1024QAM

  - Tx EVM: 2%

- FFS for applicability rule

In this contribution we provide the simulation results.

# 2	Simulation results

## 2.1	TM4

Figure 1 shows the ideal simulation results of PDSCH TM4 (corresponding to tests 1 and 2). Table 1 summarizes the required SNR [dB] to achieve 70% of the maximum throughput.

Figure 1	Simulation results of PDSCH TM4 EPA5 with 1024QAM.

Table 1	Required SNR [dB] to achieve 70% of maximum throughput for TM4.

|  | SNR of 70% of maximum throughput |
| --- | --- |
| Test 1 | 22.4 |
| Test 2 | 25.2 |

## 2.2	TM9

Figure 2 shows the simulation results of PDSCH TM4 with the PMI feedback (option 1) and the random precoding (option 2). Table 2 summarizes the required SNR [dB] to achieve 70% of the maximum throughput. It is observed the results with the wideband feedback requires less SNR.

In our understanding, the purpose of PDSCH demodulation requirements with TM9 is to verify the demodulation functionality. If we apply the PMI feedback, it is not clear the performance comes from demodulation part or PMI feedback part. Since it is the UE demodulation requirements, our preference is to use the random precoder as much as possible. However considering the very high SNR such as 34dB, we may need to consider PMI feedback especially for Test 4.

Proposal 1: For TM9 test, apply random precoder for test 3 (4x2 rank 1) and apply wideband PMI feedback for test 4 (4x4 rank 2).

Figure 2	Simulation results of PDSCH TM9 EPA5 with 1024QAM.

Table 2	Required SNR [dB] to achieve 70% of maximum throughput for TM9.

|  | SNR of 70% of maximum throughput |  |
| --- | --- | --- |
|  | WB PMI Feedback | Random precoder |
| Test 3 | 25.9 | 30.6 |
| Test 4 | 31.3 | 34.2 |

# 3	Conclusion

Proposal 1: For TM9 test, apply random precoder for test 3 (4x2 rank 1) and apply wideband PMI feedback for test 4 (4x4 rank 2).

# References

- R4-1807966, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon.