TSG-RAN Working Group 4 (Radio) meeting #88	R4-1810524

Gothenburg, Sweden, 20th - 24th August 2018

Source:	Ericsson

Title:	Background on receiver intermodulation uncertainty budget for FR2

Agenda item:	7.9.4.3.2.1

Document for:	Discussion

# Introduction

In RAN4 #87, the procedure, framework, and measurement uncertainty budget for FR2 EIS was discussed in detail. In addition, the need for PA for receiver in-band requirement is still under discussion in RAN4 and is further elaborated in [1].

Any possible need for PA would imply an addition of PA measurement uncertainty as well as possible impact of ACLReffect and an analysis around ACLR effect and PA ACLR is given in [2]. In this paper, the measurement uncertainty for FR2 RX IM is further elaborated.

# Discussion

According to TS 38.104 [3], the general intermodulation requirements for FR2 demands interfering signal level is 25dB higher than EISREFSENS_50M while the wanted signal is 6dB higher than EISREFSENS_50M. See the table below:

Table 10.8.3-1: General intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Mean power of interfering signals [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 25 dB | See Table 10.8.3-2 |
| NOTE: EISREFSENS and EISREFSENS_50M are given in subclause 10.3.3. |  |  |  |

Table 10.8.3-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 MHz | ±7.5 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS, 64 RB |
| 100 MHz | ±6.88 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS, 64 RB |
| 200 MHz | ±5.64 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS, 64 RB |
| 400 MHz | ±6.02 | CW |
|  | ±45 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS, 64 RB |

The interfering signals of Rx intermodulation are modulated signal and CW signal. The modulated interferer is quite similar to the one in ACS or blocking test, and the CW interferer causes no intermodulation. So if PA is needed we can get a similar curve of PA ACLR2 vs ACLR effect as showed in [2]. The effect curve is supposed between the ACS effect curve and blocking effect curve.

The measurement uncertainty for receiver intermodulation can be calculated as following. MUPA and ACLReffect are applicable if a PA is considered in the MU budget.

MURx_OTA=MUEIS_ref2+MUinterfer2+MUPA2+ACLReffect

Where MUinterfer2= 2MUCW_interfer2+MUMod_interfer2

MURxIM_OTA=MUwanted2+2MUCW_interfer2+MUMod_interfer2+MUchamberOTA2+MUmatchingOTA2+MUPA2+ACLReffect

If a PA would be needed, RAN4 should consider solutions to possibly compensate/post process of impact of ACLReffect for FR2 receiver requirements that are affected by ACLReffect. It is likely to be possible to reduce MUPA to a small number by means of careful calibration.

Pending the settlement of MUPA and ACLReffect, the receiver intermodulation uncertainty measurement calculations and tentative values are presented in Table 1 and Table 2 for anechoic chamber and CATR respectively.

The values are based on R4-1808649 [3] and R4-1808650 [4]

Table 2.1 MU of receiver intermodulation for IAC

| Frequency range (GHz) |  | 24.25<f<29.5 | 37<f<40 |
| --- | --- | --- | --- |
|  | MUMod_interfer 1.96σ | 1.1 dB | 1.1 dB |
|  | MUCW_interfer 1.96σ | 0.56 dB | 0.56 dB |
|  | MUEIS_OTA 1.96σ | 1.89 dB | 2.04 dB |
|  | MUPA | FFS | FFS |
|  | ACLR effect 1.96σ | FFS | FFS |
|  | MURxIM_OTA 1.96σ | 5.99+(MUPA2)+(ACLReffect) | 6.58+(MUPA2)+(ACLReffect) |

Table 2.1 MU of receiver intermodulation for CATR

| Frequency range(GHz) |  | 24.25<f<29.5 | 37<f<40 |
| --- | --- | --- | --- |
|  | MUinterfer 1.96σ | 1.1 dB | 1.1 dB |
|  | MUCW_interfer 1.96σ | 0.56 dB | 0.56 dB |
|  | MUEIS_OTA 1.96σ | 1.73 dB | 1.81 dB |
|  | MUPA | FFS | FFS |
|  | ACLR effect 1.96σ | FFS | FFS |
|  | MURxIM_OTA 1.96σ | 5.41+(MUPA2)+(ACLReffect) | 5.7+(MUPA2)+(ACLReffect) |

# Conclusion

In this paper, the measurement uncertainty for FR2 receiver intermodulation was elaborated.

# References

[1] 	R4-1810521, “Necessity of PA for OTA Rx directional requirements measurement for FR2”, Ericsson

[2]   R4-1810520, “Background on in-band blocking, ACS and ICS uncertainty budget for FR2”, Ericsson

[3]   R4-1808649, “Measurement uncertainties for EIS for FR2 in indoor anechoic chamber”, Ericsson

[4]   R4-1808650, “Measurement uncertainties for EIS for FR2 in CATR”, Ericsson