3GPP TSG-RAN WG4 Meeting #88              	R4-1810597

Gothenburg, Sweden, August 20 – 24 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on inter-frequency measurement requirements

Agenda item:	7.11.4.2.1

Document for:	Discussion

Introduction

RAN4 is discussing the inter-frequency measurement requirements for NR. The main open issue is how to define the requirements to account for the different SMTC (including period and offset of SMTC) on different layers.  In RAN4#86bis, WF [1] is agreed and four alternatives in deriving the scaling factor are listed. In RAN4#87 and RAN4-AH-1807, the four alternatives were discussed and some other new alternatives were also proposed. However, until now there is no conclusion on this important issue.

In this paper, we will provide our views on how to define scaling factor for gap based measurements.

Discussion

Derivation of the scaling factor

In [2] we have given an algorithm for deriving the scaling factor also considering the gap based intra-frequency measurement and measurement on E-UTRA/UTRA/GSM. The algorithm is presented below, and it is essentially very similar to Alt3 in [1].

- Denote N as the ratio between the longest SMTC period in a group and the MGRP.

- For N consecutive SMTC occasions,

  - Denote S(f,n) as the share for f-th carrier (1<= f <=F) at n-th SMTC occasion (1<= n <=N).

  - Denote Y and Z as the number of intra-frequency and inter-frequency/RAT carriers whose SMTC windows are present at n-th SMTC occasion;

  - Determine S(f,n) as:

    - If there is at least one intra-frequency layer and one inter-frequency/RAT layer whose SMTC are present, i.e. Y > 0 and Z > 0,

      - if X for MG sharing is numerical number, S(f,n) = Kintra/Y for each intra-frequency carrier whose SMTC occasion is present, and S(f,n) = Kinter/Z for each inter-frequency carrier whose SMTC occasion is present

      - if X for MG sharing is ‘equal split’, S(f,n) = 1/(Y+Z) for each carrier whose SMTC occasion is present

    - Else, S(f,n) = 1/(Y+Z) for each carrier whose SMTC occasion is present.

- For each of the F carriers, calculate the available number of gaps available for measurements within the longest SMTC period as G(f) = S(f,1) + S(f,2) + … + S(f,N)

- Denote the p(f) as the ratio between the longest SMTC period and the SMTC period of f-th carrier, carrier specific scaling factor CSFinter for the f-th carrier CSF(f) = p(f)/G(f).

Next, as a simple example, we will apply the algorithm to the configuration in Figure 1.

- It is clear that N = 2.

- For 2 consecutive SMTC occasions we get S(f,n) as

  - S(1,1) = 1/2, S(2,1) = 1/2, S(3,1) = 0

  - S(1,1) = 1/2, S(2,1) = 0, S(3,1) = 1/2

- For each of the 3 carriers, we get the available gaps within 2 SMTC periods as

  - G(1) = 1/2 + 1/2 = 1

  - G(2) = 1/2 + 0 = 1/2

  - G(3) = 0 + 1/2 = 1/2

- For each of the 3 carriers, we get the carrier scaling factor (CSF) as

  - p(1) = 2, CSF(1) = p(1)/G(1) = 2

  - p(2) = 1, CSF(2) = p(1)/G(1) = 2

  - p(3) = 1, CSF(3) = p(1)/G(1) = 2

The key of the algorithm is that

- Each MG occasion is used for measurement of layers whose SMTC are present at the occasion.

- All layers whose SMTC are present are given equal priority so they take equal share of MG occasion.

- The time window that needs to be considered corresponds to the largest SMTC period among all layers in the group, as SMTC presence would be repeated at this periodicity.

- The scaling factor for each layer is then determined based on the total measurement opportunity in all MG occasions in the period.

As can be seen, the algorithm does not force UE to measure certain layer in any MG occasion. The only assumption on the UE side is that each MG occasion is equally shared among all layers which have SMTC present that at occasion. We understand this should be a reasonable assumption, and UE can still decide how to schedule the measurement as long as the requirements are met. In RAN4-AH-1807, some companies raised the concern that in order to meet the requirement with the scaling factor derived based the on algorithm, UE scheduling of measurement could be very complex. In our understanding, the UE scheduling does not have to be specially designed, instead, as shown in [3] a simple round robin scheduling could be enough to meet the requirements in most of the case.

The proposed scaling factor derivation (Alt3) does not restrict UE implementation flexibility in terms of scheduling measurement.

The scaling factor applied to each carrier frequency will be predictable allowing the network visibility to the expected UE measurement performance and measurement delay. This is important information for the network for enabling timely configurations. The algorithm is also best in terms of MG utilization, as each MG occasion is used for measurement. Other alternatives will give more or less a relaxed requirement, which means not all MG occasions are used by UE for measurement. RAN4 should not design/define scaling requirements assuming not all measurement gaps are used efficiently. It is not a good system design principle as it causes waste of air interface resources.

The proposed scaling factor derivation (Alt3) does not cause wasted gap opportunities.

The algorithm is also best among all alternatives in that it can best inter-work with MG sharing between intra-frequency and inter-frequency/RAT measurement. As the usage is determined for each MG occasion, it is possible to define the requirement such that MG sharing only applies when it is needed, i.e. there is no need to reserve 50% of a MG occasion for intra-frequency, if there is no intra-frequency layer with SMTC present in this occasion. For other alternatives, the requirement can only be defined in the way that MG sharing always applies, and this will clearly make the requirements unnecessarily relaxed.

The proposed scaling factor derivation (Alt3) can best inter-work with MG sharing.

In RAN4-AH-1807, some companies proposed [4] that instead of assuming a carrier is measured in each MG occasion where it has SMTC present, for deriving the scaling factor, a carrier is assumed to be measured only in one MG occasion per repetition period (N MG occasions) where the number of competing carriers is at largest. In our view, the requirement based this assumption is too relaxed as it means the carrier is not measured in other MG occasions within the repetition period.

Some other companies proposed that this derivation can be modified as a compromise solution such that the scaling factor is not too tight and there is no need for additional number of samples as margins. However, based on our analysis, with the modified derivation, additional number of samples are still needed for the cases mentioned in [3]. Therefore, the relaxation in the scaling factor is not very meaningful.

Based on above analysis, our view is that Alt3 should be used to derive the scaling factor.

One issue with Alt3 is the feasibility for UE to meet the requirement derived with Alt3. As illustrated in [3], UE may not be able to get enough number of measurement samples (e.g. 5 samples as in current intra-frequency requirement) in some MO configurations. The algorithm assumes UE on average use the MG occasion equally for measuring the layers with SMTC present, but the actual number of times UE measures a particular layer during a certain number of MG occasions can be larger or smaller than the average value, and is depending on how UE schedules the measurement among layers.

There are several options to solve the problem

- UE is required to schedule the measurement properly so as to meet the requirement. It means for some specific configurations (number of MOs and SMTC on those MOs) UE needs to design the measurement scheduling instead of using a simple round-robin measurement.

- Add margin in number of samples in the delay requirement. For example, the current cell detection and measurement requirements are assuming 5 samples (SMTC periods) as measurement period. In order to ensure UE can get enough samples in worst case, the measurement period can be relaxed assuming 6 samples are needed.

- Add margin in the scaling factor. For example, if the derived scaling factor is N after ceiling, a factor N+1 will be used when defining the requirements.

In our view, it is ok to add some margin to cover the worst case as RAN4 is only defining the minimum requirements. Besides the exact margin, RAN4 should also discuss whether the margin is added generically or only for the cases where UE fails to meet the requirement with round robin measurement.

Alt3 is adopted for deriving the scaling factor for gap based measurements.

RAN4 should further discuss how to handle the issue that UE may not be able to meet the requirements based on Alt3 if following simple round robin measurement.

Conclusion

In this paper, we provided our views on how to define scaling factor for gap based measurements.

Proposal 1: Alt3 is adopted for deriving the scaling factor for gap based measurements.

Proposal 2: RAN4 should further discuss how to handle the issue that UE may not be able to meet the requirements based on Alt3 if following simple round robin measurement.

References

- R4-1805565, Way forward on SSB-based inter-frequency measurement requirements, Huawei, HiSilicon

- R4-1806641, Discussion on inter-frequency measurement requirements, Nokia, Nokia Shanghai Bell

- R4-1808926, Discussion on requirements of inter-frequency measurement, MediaTek Inc.

- R4-1809042	Discussion on open issues on SSB based inter-frequency measurement requirements in NR, Huawei, HiSilicon