3GPP TSG-RAN WG4 Meeting #88	R4-1810717

# Gothenburg, Sweden, 20th – 24th Aug 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Darft CR for Measurement Gap Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The definition of mesuarement gap sharing needs to be modified: Prioritization needs to be provided under different scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The configuration for measurement gap sharing different scenarios are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement gap sharing is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

Measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure intra-frequency cells or when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, and when UE is configured to identify and measure cells on inter-frequency carriers and inter-RAT carriers.When network signals “01”, “10” or “11”, where X is a signalled RRC parameter, namely MeasGapSharingScheme,

X is defined as in table 9.1.2-5 and table 9.1.2-6 when the UE is configured for single connectivity, and

X is defined as in table 9.1.2-7 when the UE is configured for dual connectivity (including EN-DC, NE-DC, NR-NR DC),

-	the performance of the measurements in which measurement gap sharing is applied, according to the sharing scheme as the parameter MeasGapSharingScheme indicates , shall consider the scaling factor K,

-	where K = 1 / X * 100 for the measurements not belonging to other measurement specified in the table of X,

-	where K = 1 / (100 – X) * 100 for the measurements belonging to other measurement specified in the table of X,

When network signals “00” indicating equal splitting gap sharing, X is not applied. The performance of measurements as specified in chapter 9 when “00” is indicated are FFS.

Table 9.1.2-5: Value of parameter X for Per-FR gap configured

| MeasGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ | Equal splitting |
| ‘01’ | At least [25]% MG for intra-frequency measurement The rest MG for other measurement |
| ‘10’ | At least [50]% MG for intra-frequency measurement The rest MG for other measurement |
| ‘11’ | At least [75]% MG for intra-frequency measurement The rest MG for other measurement |

Table 9.1.2-6: Value of parameter X for Per-UE gap configured

| MeasGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ | Equal splitting |
| ‘01’ | At least [25]% MG for measurement on FR1 The rest MG for other measurement |
| ‘10’ | At least [50]% MG for measurement on FR1 The rest MG for other measurement |
| ‘11’ | At least [75]% MG for measurement on FR1 The rest MG for other measurement |

Table 9.1.2-7: Value of parameter X for Dual Connectivity

| MeasGapSharingScheme | Value of X (%) |
| --- | --- |
| ‘00’ | Equal splitting |
| ‘01’ | At least [25]% MG for intra-frequency measurement corresponding to severing carrier(s) of MCG The rest MG for other measurement |
| ‘10’ | At least [50]% MG for intra-frequency measurement corresponding to severing carrier(s) of MCG The rest MG for other measurement |
| ‘11’ | At least [75]% MG for intra-frequency measurement corresponding to severing carrier(s) of MCG The rest MG for other measurement |

### >>>>>End of Change 1<<<<<