3GPP TSG-RAN WG4#88	R4-1810771

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	6.5.4.2

Source:	Ericsson

Title:	Simulation results for WUS reception for efeMTC

Document for:	Discussion

# Introduction

In this contribution, the simulation results for minimum WUS reception performance for release 15 efeMTC UEs are presented with the aim to resolve the TBDs in [1].

# Simulation assumption for WUS for efeMTC

In Table 1, simulation assumption is given.

Table 1: Simulation assumption for efeMTC

# Simulation results

Figure 1: WUS reception performance for Rmax of 1 (left) and 2 (right)

Figure 2: WUS reception performance for Rmax of 4 (left) and 8 (right)

From Figure 1, it is observed that UE does not need any repetition for receiving WUS for DRX cycle lengths up to 2.56 seconds, and repetition of 2 is needed for DRX cycle lengths 5.12 and 10.24 seconds in normal coverage assuming 99% WUS detection probability.

From Figure 2, it is observed that UE configured with DRX cycles up to 5.12 seconds needs repetitions of at least 4 and at least 8 repetitions are needed for DRX cycles of 10.24 seconds.

The observations are summarized in Table 2 and 3.

# Summary

In this contribution, the simulation results for minimum WUS reception performance for release 15 efeMTC are presented. We kindly ask the companies to consider these results when defining the minimum reception requirements for WUS. Based on the results, following observations are made:

Table 2 Conditions for WUS reception for UE in normal coverage

| DRX cycle length [s] | Required number of repetition of WUS signal |
| --- | --- |
| DRX cycle ≤ 2.56 | 1 |
| DRX cycle > 2.56 | 2 |

Table 3 Conditions for WUS reception for UE in enhanced coverage

| DRX cycle length [s] | Required number of repetition of WUS signal |
| --- | --- |
| DRX cycle ≤ 5.12 | 4 |
| DRX cycle > 5.12 | 8 |

# Reference

R4-1810772, “Introducing WUS reception requirements for efeMTC”, Ericsson