3GPP TSG-RAN WG4 Meeting #88	R4-1810791

# Gothenburg, SE, 20th – 24th Aug, 2018

Source:	Qualcomm Incorporated

Title:	SNR and signal control at reference point for RRM testing

Agenda item:	10.1.4

Document for:	Approval

# 1. Introducion

In July Ad-hoc meeting, the WF on remaining open issues for UE RRM setup was approved in [1]. And the following agreements are reached on Signal power and SNR etc.

- SNR Reference point:

  - For RRM baseline measurement setup based on DFF, the Reference Point is located at the centre of the Quiet Zone.  .

- DL Power accuracy/range

  - Feasibility is closely related to the test case definition. As such no further activity expected within the SI

- SNR accurcy/range still to be discussed

  - Capture in the TR the relationship between the SNR at the reference point and the SNR seen by the DUT at the BB.

  - Capture in the TR SNR range which can be emulated by the test system with an max difference between SNR at reference point and SNR at UE BB of [1] dB. This value is not expected to be used as MU value.

In this paper, we will provide our views on SNR and signal accuracy/range control at reference point for RRM testing.

# 2. Discussion

First, there was an agreement in [1] that AWGN can be transmitted from one/both active probes, and spatially white AWGN need and feasibility will be further discussed. In this paper, we do the calculation for both 1 probe and 2 probes scenarios based on the case that the noise transmitted with signal from TE. And spatially with noise emulation for FR2 testing is further discussed in [2].

For potential SNR range in RRM testing, when we consider RLM, the lower bound of SNR would be -10dB, and the upper bound of SNR would be 20dB.

# 2.1 SNR control at reference point with 1 probe

For 1 probe scenario, the actual SNR at the UE receiver can be calculated based on the same methodology as that in demodulation testing in [3] when considering the artificial noise is transmitted with signal. Therefore, there are also two options on how to control the SNR at reference point. While as the discussion [4], the side condition for RRM requirements should at least be based on the spherical coverage and whether side conditions for peak directions should be included can be further discussed. In such case, we provide the SNR calculation both for spherical coverage and beam peak directions.

Option 1: Fix the signal power as the maximum available Tx power from TE antenna, then control the level of artificial noise for different target SNR.

When applying the peak beam gain, the following results can be given in the Table 1.

Table 1: Signal and Noise Tx power level for Option 1 for peak directions

| Target SNR(dB) | Frequency (GHz) | Bandwidth (GHz) | Signal Tx power from TE (dBm) | Noise Tx power from TE (dBm) |
| --- | --- | --- | --- | --- |
| 20 | 39 | 1 | 12.4 | -7.6 |
| -10 | 39 | 1 | 12.4 | 22.4 |
| 20 | 39 | 0.1 | 2.4 | -17.6 |
| -10 | 39 | 0.1 | 2.4 | 12.4 |
| 20 | 28 | 1 | 5 | -15 |
| -10 | 28 | 1 | 5 | 15 |
| 20 | 28 | 0.1 | -5 | -25 |
| -10 | 28 | 0.1 | -5 | 5 |

When applying the 50%-ile spherical coverage which means 11dB below peak for 28GHz and 12.5 below peak for 39GHz for PC3, the flowing results can be given in Table 2.

Table 2: Signal and Noise Tx power level for Option 1 for 50%-ile spherical coverage

| Target SNR(dB) | Frequency (GHz) | Bandwidth (GHz) | Signal Tx power from TE (dBm) | Noise Tx power from TE (dBm) |
| --- | --- | --- | --- | --- |
| 20 | 39 | 1 | 24.9 | 4.9 |
| -10 | 39 | 1 | 24.9 | 34.9 |
| 20 | 39 | 0.1 | 14.9 | -5.1 |
| -10 | 39 | 0.1 | 14.9 | 24.9 |
| 20 | 28 | 1 | 16 | -4 |
| -10 | 28 | 1 | 16 | 26 |
| 20 | 28 | 0.1 | 6 | -14 |
| -10 | 28 | 0.1 | 6 | 16 |

Option 2: Control the SNR and artificial noise dynamically with total power constrain for different target SNR.

Reuse the assumptions in demod testing [3] for beam peak gain, the following results can be given in Table 3.

Table 3: Signal and Nosie Tx power level for Option 2 for peak directions

| Target SNR(dB) | Frequency (GHz) | Bandwidth (GHz) | Signal Tx power from TE (dBm) | Noise Tx power from TE (dBm) |
| --- | --- | --- | --- | --- |
| 20 | 39 | 1 | 12.4 | -7.6 |
| -10 | 39 | 1 | 2.0 | 12 |
| 20 | 39 | 0.1 | 2.4 | -17.6 |
| -10 | 39 | 0.l | -8 | 2 |
| 20 | 28 | 1 | 5 | -15 |
| -10 | 28 | 1 | -5.4 | 4.6 |
| 20 | 28 | 0.1 | -5 | -25 |
| -10 | 28 | 0.1 | -15.4 | -5.4 |

Table 4: Signal and Nosie Tx power level for Option 2 for 50%-ile spherical coverage

| Target SNR(dB) | Frequency (GHz) | Bandwidth (GHz) | Signal Tx power from TE (dBm) | Noise Tx power from TE (dBm) |
| --- | --- | --- | --- | --- |
| 20 | 39 | 1 | 24.9 | 4.9 |
| -10 | 39 | 1 | 14.5 | 24.5 |
| 20 | 39 | 0.1 | 14.9 | -5.1 |
| -10 | 39 | 0.l | 4.5 | 14.5 |
| 20 | 28 | 1 | 16 | -4 |
| -10 | 28 | 1 | 5.6 | 15.6 |
| 20 | 28 | 0.1 | 6 | -14 |
| -10 | 28 | 0.1 | -4.4 | 5.6 |

Proposal 1: For RRM SNR control with 1 probe, the SNR at the UE receiver can be controlled based on the same methodology as that in demod testing for artificial noise transmitted with signal and the SNR range should at least be based on the spherical coverage.

# 2.2 SNR control at reference point with 2 probes

For 2 probes scenario, the main difference between 1 probe and 2 prboes is the how to control the SNR with interference from the other probe. We can re-wirte the actual SNR with interference as:

SNR_actual = S_rx/(N_rx+Nktb*F_UE+I_rx)

where,

F_UE : Noise figure (NF) of the UE

Nktb : Thermal noise level

S_rx, N_rx :  Received signal power for the desired signal and AWGN (artificial noise) respectively, at reference point (UE baseband receiver port).

I_rx: The total received interference including interference from the other probe and AWGN (artificial noise) from the other probe or spatially white AWGN.

As a simple example with signal and AWGN can be transmitted from both two active probes which scenario is considered in the SS-RSRP accuracy measurement simulation in [4], in such case, probe 1 is to emulate the interfering cell, and probe 2 is to emulate measured cell.

Fig.1 2 probes scenario for RRM testing

Then we will have the following symbols:

SNR_target_P1 = S_rx_P1 /N_rx_P1 = S_tx_P1/N_tx_P1, which is from probe #1.

SNR_target_P2 = S_rx_P2 /N_rx_P2 = S_tx_P2/N_tx_P2, which is from probe #2.

S_tx_P1,  N_tx_P1 : Transmit signal power for the desired signal and AWGN (artificial noise) for P1, respectively.

S_tx_P2,  N_tx_P2 : Transmit signal power for the desired signal and AWGN (artificial noise) for P2, respectively.

S_rx_P1 = S_tx_P1 * PL(d)* G_UE_P1 * IL_UE

S_rx_P2 = S_tx_P2 * PL(d)* G_UE_P2 * IL_UE

N_rx_P1 = N_tx_P1 * PL(d) * G_UE_P1 * IL_UE

N_rx_P2 = N_tx_P2 * PL(d) * G_UE_P2 * IL_UE

PL(d): The free space pathloss, which depends on the distance d between the test antenna and DUT.

G_UE_P1/P2: UE receive antenna array gain for P1/P2

IL_UE: UE receiver implementation loss

Note: In the above, all symbols are represented as linear (non-dB) values.

Therefore, when considering the inference scenarios (2 probes), the expression of SNR/SNR target will be different from that in 1 probe scenario. For example, it can be expressed as

SNR_target_P2 = S_tx_P2/(N_tx_P2+ N_tx_P1+S_tx_P1)

Then, actual SINR/SNR for probe 2 can be written as for 2 probes scenario:

SNR_actual_P2 = S_rx_P2/(N_rx_P2+Nktb*F_UE+ N_rx_P1+S_rx_P1)

Then, the SNR error would be calculated by

SNR error (in dB) = SNR_target_P2 (in dB) - SNR_actual_P2 (in dB)

Based on the above equations, we can observe that when 2 probes are considered in the RRM testing, the SNR range/accuracy will depend on UE receive antenna array gains for different probes, and the receive antenna array gain for the 2 probes should also be based on spherical coverage.

Observation 1: When 2 probes are considered in the RRM testing, the SNR range/accuracy will depend on UE receive antenna array gains for different probes and the UE receive antenna array gain for the 2 probes should also be based on spherical coverage.

Proposal 2: How to control the SNR in 2 probes scenario and how to calculate the SNR range/accuracy in such case should also be clarified in RAN4 and captured in TR38810.

# 2.3 Signal level control at reference point

For DL signal level control at reference point, when a sample case in which the artificial noise is not transmitted simultaneously with signal from the TE, so the signal level would be obtained by the following assumptions.

For the assumptions with 39GHz with peak directions, UE receive antenna array gain would be 9.5dB, typical implementation loss =5dB, NF=14dB and the noise floor level would be -160dBm/Hz.  So, the range of signal level at BB would be:

Table 5: Signal level at reference point for 39GHz with peak directions

| Tx Power w/o noise (dBm) | 24.9 | 22.9 | 20.9 | 18.9 | 16.9 | 14.9 | 12.9 | 10.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum signal level at BB with 100MHz (dBm/Hz) | -111.6 | -113.6 | -115.6 | -117.6 | -119.6 | -121.6 | -123.6 | -125.6 |
| Maximum signal level at BB with 1GHz (dBm/Hz) | -121.6 | -123.6 | -125.6 | -127.6 | -129.6 | -131.6 | -133.6 | -135.6 |
| Minimal signal level at BB based on -6dB side condition (dBm/Hz) | -166 |  |  |  |  |  |  |  |  |
| Minimal signal level based on -10dB side condition (dBm/Hz) | -170 |  |  |  |  |  |  |  |  |

For the assumptions with 39GHz with 50%-ile spherical coverage, UE receive antenna array gain would be with 12.5 below peak (peak gain=9.5dB), typical implementation loss =5dB, NF=14dB and the noise floor level would be -160dBm/Hz. So, the range of signal level at BB would be:

Table 6: Signal level at reference point for 39GHz with 50%-ile spherical coverage

| Tx Power w/o noise (dBm) | 24.9 | 22.9 | 20.9 | 18.9 | 16.9 | 14.9 | 12.9 | 10.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum signal level at BB with 100MHz (dBm/Hz) | -124.1 | -126.1 | -128.1 | -130.1 | -132.1 | -134.1 | -136.1 | -138.1 |
| Maximum signal level at BB with 1GHz (dBm/Hz) | -134.1 | -136.1 | -138.1 | -140.1 | -142.1 | -144.1 | -146.1 | -148.1 |
| Minimal signal level at BB based on -6dB side condition (dBm/Hz) | -166 |  |  |  |  |  |  |  |  |
| Minimal signal level based on -10dB side condition (dBm/Hz) | -170 |  |  |  |  |  |  |  |  |

While if the artificial noise is transmitted simultaneously with signal from TE, the minimal available signal should be re-calculated with the lower bound of SNR side condition. And to make sure the SNR accuracy within 1dB, the minimal Rx noise power level with the metric of 5.9dB (around 6dB) higher than noise floor level which is derived in [3] should also be applied.

Proposal 3: With the artificial noise transmitted with signal from TE, to make sure the SNR accuracy within 1dB, the minimal Rx noise power level with the metric of 5.9dB (around 6dB) higher than noise floor level which is derived in [3] should also be applied.

# 3 Conclusion

In this paper, we provide our views on SNR control at reference point for Demodulation testing, and have the following observations and proposals:

Proposal 1: For RRM SNR control with 1 probe, the SNR at the UE receiver can be controlled based on the same methodology as that in demod testing for artificial noise transmitted with signal and the SNR range should at least be based on the spherical coverage.

Observation 1: When 2 probes are considered in the RRM testing, the SNR range/accuracy will depend on UE receive antenna array gains for different probes and the UE receive antenna array gain for the 2 probes should also be based on spherical coverage.

Proposal 2: How to control the SNR in 2 probes scenario and how to calculate the SNR range/accuracy in such case should also be clarified in RAN4 and captured in TR38810.

Proposal 3: With the artificial noise transmitted with signal from TE, to make sure the SNR accuracy within 1dB, the minimal Rx noise power level with the metric of 5.9dB (around 6dB) higher than noise floor level which is derived in [3] should also be applied.

# References

- R4-1809506 “WF on remaining open issues for UE RRM setup”, R&S, CATR, Anritsu, Intel, WG4 Ad-hoc 1807, July 2018

- R4-1810564, “Noise Emulation for FR2 Testing”, Qualcomm incorporated, WG4#88 meeting, Aug., 2018.

- R4-1810790, “SNR control at reference point for Demodulation testing”, Qualcomm incorporated, WG4#88 meeting, Aug., 2018.

- R4-1810550, “Side Conditions for FR2 RRM Requirements”, Qualcomm incorporated, WG4#88 meeting, Aug., 2018.

- R4-1806551, Link-level simulation assumptions for SSB-based measurement accuracy, Ericsson, WG4#87 meeting, May, 2018.