3GPP TSG-RAN WG4 Meeting #88                                                                  R4-1811142

Gothenburg, SE, 20 - 24 Aug 2018

Source: 	Huawei, HiSilicon

Title: 					n50 - AMPR study to protect EESS service (1400-1427MHz)

Agenda Item: 			7.1.1.2

Document for:	Approval

# 1	Introduction

As discussed in RAN4 meeting and already approved, the contribution [1] provided all technical definitions to be able to make the A-MPR studies to protect the EESService. Thus, A-MPR evaluation for n50 is conducted as defined just below:

- Proposal 1:

  - EESS protection level => -32dBm/27MHz1  in the 1400-1427MHz

- Proposal 2: A-MPR evaluation for n50 is conducted as

  - Definition of the study => Study the Tx only in the 1432-1517MHz band to protect EESS which is in 1400-1427MHz band

  - Tx CBW [MHz] => 5, 10, 15, 20, 40, 50, 60

  - Protected Band [MHz] => 1400-1427

This contribution presents the simulation results for the above scenarios.

# 2	Discussion

## 2.1	Simulation assumptions

A-MPR study for EESS protection has been defined as:

- Band range:

  - start:1432-1517MHz BW:5, 10, 15, 20, 40, 50, 60

- SCS:60M 30k, 15k for other BW

- SE limitation1:EESS: -32dBm/27MHz1  in the 1400-1427MHz

- ACLR: EUTRA limit 30dB

SEM:

MPR:

Table 6.2.2-1 Maximum power reduction (MPR) for power class [2] and 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

## 2.2 	EESS protection

# 2	AMPR results for EESS

| BW | AMPR |  |  |  |
| --- | --- | --- | --- | --- |
|  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |
|  | Outer | inner | Outer | Inner |
| 5MHz | 4.5 | 0.5 | 3 | -0.5 |
| 10MHz | 10.5 | 4.5 | 8.5 | 5.5 |
| 15MHz | 12.5 | 4.5 | 11.5 | 5.5 |
| 20MHz | 13 | 5.5 | 12 | 6 |
| 40MHz | 14.5 | 9.5 | 13.5 | 7.5 |
| 50MHz | 14.5 | 10 | 13.5 | 8 |
| 60MHz | 14.5 | 10.5 | 13.5 | 8.5 |

# 2	Figures for EESS

# Below, two A-MPR modulations are studied. At the end of each paragraph, there is a table to summary the A-MPR which need to implement to protect EESService for each modulation

## 2.3 	DFT-S-OFDM

Just below, the table 1 is the summary of the A-MPR needed to protect EESService.

Table 1: A-MPR for "NS_YY’" for DFT-S-OFDM case

| Channel bandwidth [MHz] | Carrier Centre Frequency, Fc, MHz | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- | --- |
| 5 | 1434.5 ≤ FC < 1437 | RBstart |  |  |  |
|  |  | LCRB [RBs] |  |  |  |
|  |  | A-MPR [dB] |  |  |  |
| 10 | 1437 ≤ FC < 1442 | RBstart |  |  |  |
|  |  | LCRB [RBs] |  |  |  |
|  |  | A-MPR [dB] |  |  |  |
| 15 | 1439.5 ≤ FC < 1447 | RBstart | 0 – 6 |  |  |
|  |  | LCRB [RBs] | 0 – 31 |  |  |
|  |  | A-MPR [dB] | ≤ 11 |  |  |
| 20 | 1442 ≤ FC < 1452 | RBstart | 0 - 27 |  |  |
|  |  | LCRB [RBs] | 0 - 40 |  |  |
|  |  | A-MPR [dB] | ≤ 6 |  |  |
| 40 | 1452 ≤ FC < 1472 | RBstart | 0 - 20 | 21 - 60 | 61 – 80 |
|  |  | LCRB [RBs] | 0 - 40 | 0 - 70 | 0 - 110 |
|  |  | A-MPR [dB] | ≤ 7 | ≤ 7 | ≤ 7 |
| 50 | 1457 ≤ FC < 1482 | RBstart | 0 – 30 | 31 – 60 | 61 - 115 |
|  |  | LCRB [RBs] | 0 - 40 | 0 - 80 | 0 - 100 |
|  |  | A-MPR [dB] | ≤ 7 | ≤ 7 | ≤ 7 |
| 60 | 1462 ≤ FC < 1487 | RBstart | 0 – 30 | 31 – 60 | 61 - 70 |
|  |  | LCRB [RBs] | 0 - 21 | 0 - 50 | 0 - 80 |
|  |  | A-MPR [dB] | ≤ 8 | ≤ 8 | ≤ 8 |

In the table above, it is assuming that average EIRP is 15dBm, thus, based on the ITU-R information which is “Note 1: The unwanted emission power level is to be understood here as the level measured with the mobile station transmitting at an average output power of 15 dBm.”

## 2.3 	CP-OFDM

Just below, the table 2 is the summary of the A-MPR needed to protect EESService in the case of the CP-OFDM nodulation.

Table 2: A-MPR for "NS_YY’’" for CP-OFDM case

| Channel bandwidth [MHz] | Parameters | Region A | Region B |
| --- | --- | --- | --- |
| 5 | RBstart |  |  |
|  | LCRB [RBs] |  |  |
|  | A-MPR [dB] |  |  |
| 10 | RBstart | 0 – 10 |  |
|  | LCRB [RBs] | 0 – 10 |  |
|  | A-MPR [dB] | ≤ 2 |  |
| 15 | RBstart | 0 – 20 |  |
|  | LCRB [RBs] | 0 – 40 |  |
|  | A-MPR [dB] | ≤ 4 |  |
| 20 | RBstart | 0 - 20 | 21 - 35 |
|  | LCRB [RBs] | 0 – 40 | 0 - 65 |
|  | A-MPR [dB] | ≤ 5 | ≤ 5 |
| 40 | RBstart | 0 - 40 | 41 – 80 |
|  | LCRB [RBs] | 0 - 40 | 0 - 80 |
|  | A-MPR [dB] | ≤ 6 | ≤ 6 |
| 50 | RBstart | 0 – 40 | 41 – 90 |
|  | LCRB [RBs] | 0 – 35 | 0 – 80 |
|  | A-MPR [dB] | ≤ 6 | ≤ 6 |
| 60 | RBstart | 0 – 40 | 41 – 60 |
|  | LCRB [RBs] | 0 – 20 | 0 - 60 |
|  | A-MPR [dB] | ≤ 6 | ≤ 6 |

In the table above, it is assuming that average EIRP is 15dBm, thus, based on the ITU-R information which is “Note 1: The unwanted emission power level is to be understood here as the level measured with the mobile station transmitting at an average output power of 15 dBm.”

# 3	Conclusion

In this contribution we have presented NR A-MPR back off for Band n50. As the difference between DFT-S-OFDM modulation and CP-OFDM modulation is small and that the worst case is the DFT-S-OFDM modulation, it is proposed:

Proposal 1: Take into account only the DFT-S-OFDM modulation

Proposal 2: The tables 3 & 4 below propose A-MPR levels for EESS. These tables can be the reference tables for A-MPR parameters in the TS to protect EESS in the 1400-1427MHz band:

Table 3: A-MPR for "NS_YY"

| Channel bandwidth [MHz] | Carrier Centre Frequency, Fc, MHz | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- | --- |
| 5 | 1434.5 ≤ FC < 1437 | RBstart |  |  |  |
|  |  | LCRB [RBs] |  |  |  |
|  |  | A-MPR [dB] |  |  |  |
| 10 | 1437 ≤ FC < 1442 | RBstart |  |  |  |
|  |  | LCRB [RBs] |  |  |  |
|  |  | A-MPR [dB] |  |  |  |
| 15 | 1439.5 ≤ FC < 1447 | RBstart | 0 – 6 |  |  |
|  |  | LCRB [RBs] | 0 – 31 |  |  |
|  |  | A-MPR [dB] | [≤ 11] |  |  |
| 20 | 1442 ≤ FC < 1452 | RBstart | 0 - 27 |  |  |
|  |  | LCRB [RBs] | 0 - 40 |  |  |
|  |  | A-MPR [dB] | [≤ 6] |  |  |
| 40 | 1452 ≤ FC < 1472 | RBstart | 0 - 20 | 21 - 60 | 61 – 80 |
|  |  | LCRB [RBs] | 0 - 40 | 0 - 70 | 0 - 110 |
|  |  | A-MPR [dB] | [≤ 7] | [≤ 7] | [≤ 7] |
| 50 | 1457 ≤ FC < 1482 | RBstart | 0 – 30 | 31 – 60 | 61 - 115 |
|  |  | LCRB [RBs] | 0 - 40 | 0 - 80 | 0 - 100 |
|  |  | A-MPR [dB] | [≤ 7] | [≤ 7] | [≤ 7] |
| 60 | 1462 ≤ FC < 1487 | RBstart | 0 – 30 | 31 – 60 | 61 - 70 |
|  |  | LCRB [RBs] | 0 - 21 | 0 - 50 | 0 - 80 |
|  |  | A-MPR [dB] | [≤ 8] | [≤ 8] | [≤ 8] |

Note 1: The unwanted emission power level is to be understood here as the level measured with the mobile station transmitting at an average output power of 15 dBm

Table 4: AMPR results for EESS

| BW | AMPR |  |  |  |
| --- | --- | --- | --- | --- |
|  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |
|  | Outer | inner | Outer | Inner |
| 5MHz | 4.5 | 0.5 | 3 | -0.5 |
| 10MHz | 10.5 | 4.5 | 8.5 | 5.5 |
| 15MHz | 12.5 | 4.5 | 11.5 | 5.5 |
| 20MHz | 13 | 5.5 | 12 | 6 |
| 40MHz | 14.5 | 9.5 | 13.5 | 7.5 |
| 50MHz | 14.5 | 10 | 13.5 | 8 |
| 60MHz | 14.5 | 10.5 | 13.5 | 8.5 |

# 4	Reference

[1] R4-1805727, WF on NR A-MPR evaluation scenarios for n50, n51, and n74, NTT DOCOMO, INC., Huawei, Etisalat, RAN4#86bis