3GPP TSG-RAN WG4 Meeting #88                                                                  R4-1811146

Gothenburg, SE, 20 - 24 Aug 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition parameters about n51 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Etisalat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition parameters about n51 in TS 38.101-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Paragraphs: ### 6.2.1 => Add n51 UE maximum output power in the table 6.2.3 => Update the table 6.2.3-1: Additional maximum power reduction (A-MPR) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Need to update some technical parameters about Bands n51 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1; 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n8 |  |  |  |  | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n25 |  |  |  |  | 23 | ± 2 |
| n34 |  |  |  |  | 23 | ± 2 |
| n39 |  |  |  |  | 23 | ± 2 |
| n40 |  |  |  |  | 23 | ± 2 |
| n41 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n50 |  |  |  |  | 23 | ± 2 |
| n66 |  |  |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  | 23 | +2 / - 2.5 |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | ± 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n86 |  |  |  |  | 23 | ± 2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%  (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-	[may] apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.4;

-	if the IE P-Max as defined in TS 38.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.4;

-	else (i.e the IE P-Max as defined in TS 38.331 [7] is provided and set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle as defined in TS 38.331; or the IE P-Max as defined in TS 38.331 [7] is provided and set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to 50% when maxUplinkDutyCycle is absent. The exact evaluation period is no less than one radio frame):

-	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2.4;

### 6.2.2	UE maximum output power reduction

<Unchanged sections omitted>

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3-1 specifies for UE Power Class 3 the additional requirements and allowed A-MPR with corresponding network signalling value and operating band. Unless otherwise stated, the allowed A-MPR is in addition to the allowed MPR specified in subclause 6.2.2.

Table 6.2.3-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) | Value of additionalSpectrumEmission |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 |  |  |  |  | N/A | 1 |
| NS_02 | 6.5.2.1.2 | n1, n2, n3, n4, n5, n8, n20, n25, n66, n80, n81, n82, n84, NOTE 1 |  |  | Table 6.2.3-2 | 1 |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70 |  |  | Table 6.2.3.7-1 | 3 |
| NS_04 | 6.5.2.3.1 | n41 | 10, 15, 20, 40, 50, 60 80, 100 |  | Subclause 6.2.3.2 | 4 |
| NS_06 | 6.5.2.3.3 | n12 | 5, 10, 15 | 5.3.5 | N/A | 2 |
| NS_10 |  | n20, n82 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 | NS_xx |
| NS_07 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.3-1 | [1]3,4 |  |
| NS_05 | 6.5.3.3.3 | n28, n83 | 5 | ≥ 2 | ≤ 24 |  |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 54 |  |
| NS_08 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 205 |  | Subclause 6.2.3.4- |  |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A | 2 |
| NS_40 | 6.5.2.3.7 | n51 | 5 |  | Table 6.2.3.5-1 |  |
| NS_09 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Subclause 6.2.3.6 |  |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: The total maximum output power reduction for NS_xx and NS_yy is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-1 in TS 36.101 and A-MPR specified in Table 6.2.3-1. NOTE 3: The A-MPR is 0dB for inner RB allocations for DFT-s-OFDM PI/2 BPSK and QPSK. NOTE 4: The A-MPR for CP-OFDM shall also add the corresponding MPR specified in Table 6.2.2-1. NOTE 5: No A-MPR is applied for 5MHz CBW where the lower channel edge is ≥1930MHz,10MHz CBW where the lower channel edge is ≥1950MHz and 15MHz CBW where the lower channel edge is ≥1955MHz. |  |  |  |  |  |  |

Table 6.2.3-2: A-MPR for UTRA protections

| Modulation | A-MPR |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 1.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 0.5 | 0 |
| DFT-s-OFDM 64 QAM | ≤ 0.5 | 0 |
| DFT-s-OFDM 256 QAM | 0 | 0 |
| CP-OFDM QPSK | ≤ 1 | 0 |
| CP-OFDM 16 QAM | ≤ 1 | 0 |
| CP-OFDM 64 QAM | ≤ 0.5 | 0 |
| CP-OFDM 256 QAM | 0 | 0 |
| NOTE 1: A-MPR defined in this Table is additive to MPR defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |

#### 6.2.3.2	A-MPR for NS_04

<End of the change>