3GPP TSG-RAN WG4 #88	R4- 1811194

Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 OTA EIRP accuracy under extreme temperature conditions in a Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no text capturing detailed procedures or MU for EIRP accuracy under extreme temp conditions in a Near Field Test Range |  |  |  |  |  |  |  |  |
| Summary of change: |  | EIRP accuracy under extreme temp conditions, test procedure and MU budget are included for Near Field Test Range. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text is needed for EIRP accuracy under extreme temp conditions in a Near Field test range |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 10.2.8 	EIRP accuracy – extreme conditions

#### 10.2.8.1	General

For the direct far field method (see sub-clause 5.2.3.4) the MU budget is very similar to the existing MU budget for the EIRP accuracy requirement in 3GPP TR 37.842. However, there are numbers of additional sources of uncertainty due to the environmental enclosure that need to be added to the budget.

#### 10.2.8.4	Near Field

##### 10.2.8.4.1	General

This method measures the EIRP of the AAS BS under test inside a RF transparent environmentally sealed enclosure so that the DUT temperature can be controlled whilst the temperature of the OTA chamber facility is at nominal temperature.

The Near field setup for EIRP type of measurements is depicted in section 10.x.x.x.x of TR37.842. In figure 10.2.8.4.1-1, an example of a Near Field setup is reported:

Figure 10.2.8.4.1-1: Example of a Near Field system setup for EIRP measurement under extreme temperature conditions

##### 10.2.8.4.2	Calibration

Calibration shall be done with the same procedure shown in 10.2.2.4.2 to ensure that the SNR at the measurement equipment input is appropriate for the measurement of the requirement and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.8.4.3 	Procedure

Reference procedure in subclause 10.2.2.4.3 where the parameter is the EIRP defined in subcaluse 10.3.1.1.5 of TR37.842[x]

##### 10.2.8.4.4 	MU assessment

##### 10.2.8.4.4.1 	MU Budget

Table 10.2.8.4.4.1-1: Near Field Test Range uncertainty contributions
for extreme EIRP accuracy measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2:, EIRP near field radiation pattern measurement and EIRP near field DUT power measurement |  |
| 1 | Axes Intersection | B4-1 of 3GPP TR37.842 [XX] |
| 2 | Axes Orthogonality | B4-2 of 3GPP TR37.842 [XX] |
| 3 | Horizontal Pointing | B4-3 of 3GPP TR37.842 [XX] |
| 4 | Probe Vertical Position | B4-4 of 3GPP TR37.842 [XX] |
| 5 | Probe H/V pointing | B4-5 of 3GPP TR37.842 [XX] |
| 6 | Measurement Distance | B4-6 of 3GPP TR37.842 [XX] |
| 7 | Amplitude and Phase Drift | B4-7 of 3GPP TR37.842 [XX] |
| 8 | Amplitude and Phase Noise | B4-8 of 3GPP TR37.842 [XX] |
| 9 | Leakage and Crosstalk | B4-9 of 3GPP TR37.842 [XX] |
| 10 | Amplitude Non-Linearity | B4-10 of 3GPP TR37.842 [XX] |
| 11 | Amplitude and Phase Shift in rotary joints | B4-11 of 3GPP TR37.842 [XX] |
| 12 | Channel Balance Amplitude and Phase | B4-12 of 3GPP TR37.842 [XX] |
| 13 | Probe Polarization Amplitude and Phase | B4-13 of 3GPP TR37.842 [XX] |
| 14 | Probe Pattern Knowledge | B4-14 of 3GPP TR37.842 [XX] |
| 15 | Multiple Reflections (extreme) | B4-15 of 3GPP TR37.842 [XX] |
| 16 | Room Scattering (extreme) | B4-16 of 3GPP TR37.842 [XX] |
| 17 | DUT support Scattering (extreme) | B4-17 of 3GPP TR37.842 [XX] |
| 18 | Scan Area Truncation | B4-18 of 3GPP TR37.842 [XX] |
| 19 | Sampling Point Offset | B4-19 of 3GPP TR37.842 [XX] |
| 20 | Spherical Mode Truncation | B4-20 of 3GPP TR37.842 [XX] |
| 21 | Positioning | B4-21 of 3GPP TR37.842 [XX] |
| 22 | Probe Array Uniformity | B4-22 of 3GPP TR37.842 [XX] |
| 23 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | B4-23 of 3GPP TR37.842 [XX] |
| 24 | Insertion loss of receiver chain | B4-24 of 3GPP TR37.842 [XX] |
| 25 | Uncertainty of the absolute gain of the probe antenna | B4-25 of 3GPP TR37.842 [XX] |
| 26 | Measurement Receiver | E of 3GPP TR37.842 [XX] |
| 27 | Measurement repeatability - Positioning Repeatability (extreme) | B4-27 of 3GPP TR37.842 [XX] |
| 28 | Radome loss variation |  |
| 29 | Wet radome loss variation |  |
|  | Stage 1: Calibration measurement |  |
| 30 | Uncertainty of network analyser | E of 3GPP TR37.842 [XX] |
| 31 | Mismatch of receiver chain | B4-29 of 3GPP TR37.842 [XX] |
| 32 | Insertion loss of receiver chain | B4-30 of 3GPP TR37.842 [XX] |
| 33 | Mismatch in the connection of the calibration antenna | B4-31 of 3GPP TR37.842 [XX] |
| 34 | Influence of the calibration antenna feed cable | B4-32 of 3GPP TR37.842 [XX] |
| 35 | Influence of the probe antenna cable | B4-33 of 3GPP TR37.842 [XX] |
| 36 | Uncertainty of the absolute gain of the calibration antenna | E of 3GPP TR37.842 [XX] |
| 37 | Short term repeatability | B4-35 of 3GPP TR37.842 [XX] |

##### 10.2.8.4.4.2 	MU Value

Table 10.2.8.4.4.2-1: Near Field Test Range uncertainty assessment for extreme EIRP accuracy measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections (extreme) | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 16 | Room Scattering (extreme) | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 17 | DUT support Scattering (extreme) | FFS | FFS | Gaussian | 1.00 | 1 | FFS | FFS |
| 18 | Scan Area Truncation | 0.003 | 0.003 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 19 | Sampling Point Offset | 0.0058 | 0.0058 | Gaussian | 1.00 | 1 | 0.01 | 0.01 |
| 20 | Spherical Mode Truncation | 0.015 | 0.015 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 21 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 22 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 23 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 24 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 25 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 26 | Measurement Receiver | 0.14 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 |
| 27 | Measurement repeatability - positioning repeatability (extreme) | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 28 | Radome loss variation | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 29 | Wet radome loss variation | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 30 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 31 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 32 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 33 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 34 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 35 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 36 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 37 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | FFS | FFS |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | FFS | FFS |

##### 10.2.8.x	Summary

The different OTA test chamber MU are summarised below:

Table 10.2.8.x-1: Test system specific measurement uncertainty values for the EIRP test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | FFS | FFS |
| Compact Antenna Test Range | FFS | FFS |
| Near Field Test Range | FFS | FFS |
| … |  |  |
| Common maximum accepted test system uncertainty | FFS | FFS |