3GPPRAN4#88	R4-1811288

Gothenburg, Sweden, Aug 20th – 24th

Title: 	RRM Test Cases for UE Transmit Timing

Source: 	Qualcomm Incorporated

Agenda item:	7.12.3.7 - Timing and Signal characteristics (RLM/Interruption/(de)activation) [NR_newRAT-Perf]

Document for:	Discussion

# Introduction

In this paper we provide a test description for testing UE UL timing. The purpose of the test is to ensure that the UE can follow frame timing change when connected to gNodeB.

# Test Description

The UE UL timing requirements for NR are captured in Section 7 of 38.133. There are requirements for initial transmit timing accuracy, autonomous adjustment rates and values, and TA command application.

In this contribution we provide a test description for initial transmit timing accuracy, the maximum amount of timing change in one adjustment, and the minimum and the maximum adjustment rate.

Proposal 1: The SSB SCS and DL/UL SCS should be the highest supported by UE capability in each FR.

For these tests, a single cell is used. Table 1 defines the test parameters to be used for this test and Table 2 defines the relative strength of signals transmitted

Table 1: Test Parameters

| Parameter | Unit | Test1 | Test2 |
| --- | --- | --- | --- |
| Band |  | TBD | TBD |
| Channel Number |  | TBD | TBD |
| SS Raster Number |  | TBD | TBD |
| Reference Measurement Channel |  | TBD | TBD |
| OCNG Pattern |  | TBD | TBD |
| UL/DL Pattern |  | TBD | TBD |
| DL/UL BW | MHz | 40 (FR1)/100 (FR2) | 40 (FR1)/100 (FR2) |
| Number of BWP’s |  | 1 | 1 |
| BWP BW |  | Same as CC BW | Same as CC BW |
| SSB periodicity | ms | 20 | 20 |
| DL/UL BW | MHz | 40/100 | 40/100 |
| DRX | ms | None | 320 |
| Noc (per RE) | dbm/RE | -98 | -98 |
| Es/Iot | db | 3 | 3 |
| Es/Noc | db | 3 | 3 |
| Propagation Condition |  | AWGN | AWGN |

Table 2: Relative strength of transmitted signals

| Parameter | Unit | Value |
| --- | --- | --- |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of OCNG DMRS to SSS | dB | 0 |
| EPRE ratio of OCNG to OCNG DMRS | dB | 0 |

The transmit timing is measured by UE transmitting SRS.

The test sequence shall be carried out in RRC_CONNECTED for both test cases.

Following will be the test sequence for this test

- After connection set up with the cell, the test equipment will verify that the timing is within (NTA + NTA_offset) ± Te of the first detected path of DL SSB.

  - The NTA offset value (in Tc units) is 25600 for FR1 and 13792

  - The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 1.

Table 3: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | [12]*64*Tc |
|  |  | 30 | [10]*64*Tc |
|  |  | 60 | [10]*64*Tc |
|  | 30 | 15 | [8]*64*Tc |
|  |  | 30 | [8]*64*Tc |
|  |  | 60 | [7]*64*Tc |
| 2 | 120 | 60 | [3.5]*64*Tc |
|  |  | 120 | [3.5]*64*Tc |
|  | 240 | 60 | [3]*64*Tc |
|  |  | 120 | [3]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 Editor’s note: The final values of Te for 120KHz SSB SCS are subject to further discussions in further meeting, and may not be outside 3*64*Tc to 3.5*64*Tc. |  |  |  |

- The test system shall adjust the timing of the DL path by values given in Table

Table 4: Adjustment value for DL timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |
| 120 | +16*64Tc | +8*64Tc |
| 240 | +8*64Tc | +4*64Tc |

- The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Section 7.1.2 Table 7.1.2-3. This will only be done for Test1.

- The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ± Te of the first detected path of DL SSB. For test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment