3GPP TSG-RAN WG4 Meeting #88	R4-1811433

Gothenburg, Sweden, 20 - 24 August, 2018

Agenda item:	9.1.2

Source:	Dish Network

Title:	TP for TR38.716-01-01: Requirements for CA_n66(2A) and CA_n66B

Document for:	Approval

# 1	Introduction

This TP captures the requirements for CA_n66(2A) and CA_n66B.

# 2	Discussion

Both of these band combinations are defined in LTE CA. Thus it is straightforward to re-use those requirements in NR CA as well. Note that CA_n66B equals to CA_66C in LTE slang.

In this TP we have assumed that all 15, 30, and 60kHz SCS are supported, except that 5MHz is not supported by 30/660kHz SCS.

# 3	TP for TR38.716-01-01

## 5.X	CA_2DL_n66B_1UL_n66A

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported bandwidth combinations for CA_n66B

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n66B | - | 51 | 20 | 25 | 0 |
|  |  | 10 | 15 |  |  |
|  |  | 15 | 10 |  |  |
|  |  | 20 | 51 |  |  |
|  |  | 10 | 20 | 30 |  |
|  |  | 15 | 15 |  |  |
|  |  | 20 | 10 |  |  |
|  |  | 15 | 20 | 35 |  |
|  |  | 20 | 15 |  |  |
|  |  | 51 | 40 | 45 |  |
|  |  | 40 | 51 |  |  |
|  |  | 10 | 40 | 50 |  |
|  |  | 40 | 10 |  |  |
| Note 1: 5MHz is not applicable for 30/60kHz SCS |  |  |  |  |  |

### 5.1.2	UE co-existence studies

There are no co-existence issues for this combination.

## 6.X	CA_2DL_n66(2A)_1UL_n66A

### 6.X.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_n66(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n66(2A) | - | 51 | 20 | 25 | 0 |
|  |  | 10 | 15 |  |  |
|  |  | 20 | 51 |  |  |
|  |  | 15 | 10 |  |  |
|  |  | 10 | 20 | 30 |  |
|  |  | 15 | 15 |  |  |
|  |  | 20 | 10 |  |  |
|  |  | 15 | 20 | 35 |  |
|  |  | 20 | 15 |  |  |
|  |  | 20 | 20 | 40 |  |
|  |  | 51 | 40 | 45 |  |
|  |  | 40 | 51 |  |  |
|  |  | 10 | 40 | 50 |  |
|  |  | 40 | 10 |  |  |
|  |  | 15 | 40 | 55 |  |
|  |  | 40 | 15 |  |  |
|  |  | 20 | 40 | 60 |  |
|  |  |  |  | 40 |  | 20 |  |  |  |  |  |
| Note 1: 5MHz is not applicable for 30/60kHz SCS |  |  |  |  |  |  |  |  |  |  |  |

### 6.X.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.X.3	REFSENS

There are no REFSENS exceptions for this combination. However, UL configuration for REFSENS needs to be captured after general principles for RX requirements have been agreed.

# 4	References