3GPP TSG-RAN WG4 Meeting #88bis	R4-1812049

Chengdu, CN, 8th – 12th Oct 2018

Source: 	Nokia

Title: 					NS_03 A-MPR for NR

Agenda Item: 			7.6.10.2

Document for:	Approval

# 1	Introduction

In this contribution we present simulation results for NR NS_03 A-MPR.

# 2	Discussion

NS_03 A-MPR is already specified in current TS 38.101-1, see below tables.

Table 6.2.3-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) | Value of additionalSpectrumEmission |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 |  |  |  |  | N/A | 1 |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, |  |  | Table 6.2.3.7-1 | 3 |
|  |  | n70 |  |  |  |  |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66 |  |  | FFS |  |

Table 6.2.3.7-1 A-MPR for NS_03

| Modulation | Channel BW / Transmission BW in MHz |  |  |  |  | A-MPR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 40 | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM | 1.44 | 1.44 – 2.16 | 1.44 – 2.88 | 2.16 – 3.24 | 2.88 – 4.32 | N/A | 1 |
|  | > 1.44 | > 2.16 | > 2.88 | > 3.24 | > 4.32 | 1 | 2 |
| DFT-s-OFDM 64 QAM | 1.44 | 1.44 – 2.16 | 1.44 – 2.88 | 2.16 – 3.24 | 2.88 – 4.32 | 0.5 | 0.5 |
|  | > 1.44 | > 2.16 | > 2.88 | > 3.24 | > 4.32 | 1.5 | 1.5 |
| DFT-s-OFDM 256 QAM | <1.44 |  |  |  |  | 0.5 | 0.5 |
|  | 1.44 |  |  |  |  | 1.5 | 1.5 |
| CP-OFDM QPSK | 1.44 | 1.44 – 2.16 | 1.44 – 2.88 | 2.16 – 3.24 | 2.88 – 4.32 | N/A | 1 |
|  | > 1.44 | > 2.16 | > 2.88 | > 3.24 | > 4.32 | 1 | 2 |
| CP-OFDM 16 QAM | 1.44 | 1.44 – 2.16 | 1.44 – 2.88 | 2.16 – 3.24 | 2.88 – 4.32 | N/A | 2 |
|  | > 1.44 | > 2.16 | > 2.88 | > 3.24 | > 4.32 | 1 | 2 |
| CP-OFDM 64 QAM | > 1.08 | > 1.08 | > 1.44 | > 1.8 | > 2.88 | 1 | 1 |
| CP-OFDM 256 QAM | > 1.08 | > 1.08 | > 1.44 | > 1.8 | > 2.88 | 1 | 1 |
| NOTE 1: A-MPR defined in this Table is additive to MPR defined in Table 6.2.2-1 NOTE 2: Inner and outer allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

But it is not studied properly for NR as the above A-MPR table is based directly on LTE NS_03 A-MPR. LTE NS_03 was one of the first if not the first A-MPR that was studied and specified for REL-8 and was done around early 2008. At that time there were no RF simulators used in RAN4 work as we are used to nowadays hence A-MPR work was more creative art form. NS_03 impacts many UE bands n2, n25, n66 and n70 so it worth checking.

What we did was that we simulated necessary A-MPR that UE need to meet the NS_03 SEM. Also NR ACLR and general spurious emissions requirement were part of simulations but UE can meet those requirements with MPR.

Table 6.5.2.3.3-1: Additional requirements for “NS_03”

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | Measurement bandwidth |  |
| 0-1 | -13 | -13 | -13 | -13 | -13 | 1 % of channel BW |  |
| 1-6 | -13 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 6-10 | -25 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 10-15 |  | -25 | -13 | -13 | -13 | 1 MHz |  |
| 15-20 |  |  | -25 | -13 | -13 | 1 MHz |  |
| 20-25 |  |  |  | -25 | -13 | 1 MHz |  |
| 25-40 |  |  |  |  | -13 | 1 MHz |  |
| 40-45 |  |  |  |  | -25 | 1 MHz |  |

What we found out was that UE can meet the NS_03 SEM with current NR MPR thus it seems that the A-MPR currently specified is not necessary. Detailed results are presented in Table 1 which presents the margins in decibels that UE emission have to the SEM when UE uses specified MPR.

Table 1: Emission margin in dB to NS_03 SEM with NR MPR applied

What are the possible reasons why we get this difference to LTE? One reason is that LTE A-MPR is not very large hence the difference is not large. Secondly as explained already NS_03 was probably first A-MPR that was defined and no emission simulations took place as we are now used to. Thirdly when LTE NS_03 was done it only considered worst case position of allocation i.e. in the channel edge not like in NR where we have also inner allocations. In fact current NS_03 gives biggest A-MPR to inner allocations which does not make much sense as inner allocation have larger guard band against NS_03 SEM but this was the outcome of trying to match LTE A-MPR always to NR. Lastly some improvement in PA’s or at least PA modelling seem to have happened in 10 years’ time.

Thus we are proposing that NS_03 A-MPR is removed from specifications.

Proposal 1: NS_03 A-MPR is removed from specifications

If proposal 1 is accepted, then NS_03U is same as NS_100 i.e. Table 6.2.3-2.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal 1: NS_03 A-MPR is removed from specifications