3GPP TSG-RAN4 Meeting #88bis 	R4-1812156

# Chengdu, China, 8 – 12 Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UE Timing Advance Adjustment Accuracy Test for SA (section A.7.3.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-09-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To add the UE Timing Advance Adjustment Accuracy Test for SA |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the UE Timing Advance Adjustment Accuracy Test for SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE Timing Advance Adjustment Accuracy Test for SA is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### A.7.3.2	UE Timing Advance Adjustment Accuracy Test for SA

#### A.7.3.2.1	UE Timing Advance Adjustment Accuracy Test with NR FDD PCell in FR1

##### A.7.3.2.1.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment accuracy requirements for NR FDD PCell in FR1, defined in clause 7.3.2.2, in an AWGN model.

The test parameters for NR PCell are given in tables A.7.3.2.1.1-1, A.7.3.2.1.1-2 and A.7.3.2.1.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.3.2.1.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.3.2.1.1-3. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+6 for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.3.2.1.1-1: General Test Parameters for Timing Advance Accuracy Test for FDD FR1 PCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SMTC configuration |  | SMTC.1 FR1 | As specified in clause A.3.2.1.1.1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA_new = NTA_old + 8192*Tc (based on equation in TS38.213 section 4.2) |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.3.2.1.1-2: Cell specific Test Parameters for Timing Advance Accuracy Test for FDD FR1 PCell

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number |  | 2 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters defined in clause A.3.1.1.1 |  | SR.1.1 FDD |  |
| RMSI CORESET defined in clause A.3.1.2.1 |  | CR.1.1 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cell(s) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.3.2.1.1-3: Sounding Reference Symbol Configuration for Timing Advance Accuracy Test for FDD FR1 PCell

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 12 | SRS BW= 48 PRBs, and frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | nonCodebook | Non-codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |

##### A.7.3.2.1.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 slots after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

#### A.7.3.2.2	UE Timing Advance Adjustment Accuracy Test with NR TDD PCell in FR1

##### A.7.3.2.2.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment accuracy requirements for NR TDD FR1 PCell, defined in clause 7.3.2.2, in an AWGN model.

The test parameters for NR PCell are given in tables A.7.3.2.2.1-1, A.7.3.2.2.1-2 and A.7.3.2.2.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.3.2.2.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.3.2.2.1-3. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+6 for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.3.2.2.1-1: General Test Parameters for Timing Advance Accuracy Test for TDD FR1 PCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SMTC configuration |  | SMTC.2 FR1 | As specified in clause A.3.2.1.1.2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA_new = NTA_old + 8192*Tc (based on equation in TS38.213 section 4.2) |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.3.2.2.1-2: Cell specific Test Parameters for Timing Advance Accuracy Test for TDD FR1 PCell

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number |  | 2 |  |
| BWchannel | MHz | 40 |  |
| PDSCH parameters defined in clause A.3.1.1.2 |  | SR.2.1 TDD |  |
| RMSI CORESET defined in clause A.3.1.2.2 |  | CR.2.1 TDD |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/30 KHz | -98 |  |
|  | dB | 3 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cell(s) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.3.2.2.1-3: Sounding Reference Symbol Configuration for Timing Advance Accuracy Test for TDD FR1 PCell

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 24 | SRS BW= 96 PRBs, and frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | nonCodebook | Non-codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |

##### A.7.3.2.2.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 slots after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

#### A.7.3.2.3	UE Timing Advance Adjustment Accuracy Test with NR TDD PCell in FR2

##### A.7.3.2.3.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment accuracy requirements for NR TDD FR2 PCell defined in clause 7.3.2.2, in an AWGN model.

The test parameters for NR PCell are given in tables A.7.3.2.3.1-1, A.7.3.2.3.1-2 and A.7.3.2.3.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.3.2.3.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.3.2.3.1-3. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+6 for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.3.2.3.1-1: General Test Parameters for Timing Advance Accuracy Test for TDD FR2 PCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SMTC configuration |  | SMTC.1 FR2 | As specified in clause A.3.2.1.2.1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA_new = NTA_old + 8192*Tc (based on equation in TS38.213 section 4.2) |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.3.2.3.1-2: Cell specific Test Parameters for Timing Advance Accuracy Test for TDD FR2 PCell

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number |  | 2 |  |
| BWchannel | MHz | 100 |  |
| PDSCH parameters defined in clause A.3.1.1.2 |  | SR.3.1 TDD |  |
| RMSI CORESET defined in clause A.3.1.2.2 |  | CR.3.1 TDD |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/120 KHz | -98 |  |
|  | dB | 3 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cell(s) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.3.2.3.1-3: Sounding Reference Symbol Configuration for Timing Advance Accuracy Test for TDD FR2 PCell

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 16 | SRS BW= 60 PRBs, and frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | nonCodebook | Non-codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |

##### A.7.3.2.3.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 slots after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

End of Change 1