3GPP TSG-RAN WG4 Meeting #88bis	R4-1812178

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	7.13.1.5

Source: 		Intel Corporation

Title: 			Discussion on channel models for FR2

Document for:	Discussion

# Introduction

In RAN4#88 the WF for channels models was approved [1]. The following were agreed for FR2 channel models:

Confirm Working assumption: Channel Model Option 1 will be used for FR2. TDL Model - TDL-A: 30 ns RMS delay spread - FFS if additional models are needed - Option 1: TDL-C: {60-80} ns RMS delay spread - Option 2: TDL-D: [10-30] ns RMS delay spread - Other models are not precluded Doppler spread - Low speed: 75 Hz for simulation assumptions - High speed: 300 Hz for simulation assumptions Channel models of simulation alignment - Model 1: TDL-A: 30 ns 75 Hz for CSI / PDCCH/ PDSCH/ PBCH - Model 2: TDL-A: 30 ns 300 Hz for PDCCH/ PDSCH / PBCH - Note: use channel models without simplifications for simulation alignment in RAN4 88bis Channel models simplification - The maximum number of taps: [12] - 5 ns quantization grid (200MHz sampling frequency) - Follow same procedure as for FR1 to generate the final channel models

In this contribution we provide the channel models for FR2.

# Discussion

## TDL Models

TDL-A with 30ns was agreed to be used for FR2. It was FFS if additional channel models were needed the options for additional channels were:

  - Option 1: TDL-C: {60-80} ns RMS delay spread

  - Option 2: TDL-D: [10-30] ns RMS delay spread

In RAN4#88, in [2] we show that the delay spread in FR2 considering Tx-Rx beamforming is limited to 30ns. The delay spread range of 60-80ns is very large for FR2 scenarios. Hence we propose not to use TDL-C with [60-80] ns to define requirements in FR2.

The LOS channel model TDL-D doesn’t stress the UE performance. There might not be any aspect tested in the UE by defining requirements with TDL-D/ LOS channel model.

Observation #1: TDL-C with 60-80ns are not suitable for modelling FR2 propagation conditions

Observation #2: TDL-D/ LOS channel model doesn’t stress UE performance and hence not suitable to define requirements in FR2

Proposal#1: Define UE demodulation requirements in FR2 with TDL-A-30ns.  TDL-C-[60-80]ns and TDL-D-[10-30]ns shall not be used for defining UE demodulation requirements in FR2

## Simplified TDL Models

The way forward for the channel model simplification for FR2 in [1] was:

- The maximum number of taps: [12]

- 5 ns quantization grid (200MHz sampling frequency)

- Follow same procedure as for FR1 to generate the final channel models

In our companion paper [3], we analyse different simplification methods and propose to use the method of choosing strongest paths contributing to 90% of total power for TDL-A.

Figure 1: FCF of TDL-A Channel

Based on visual inspection of the FCF of the modified TDL-A channel with strongest paths contributing to 90% of total power, it is suitable for channel BWs up to 100MHz, with 5ns delay grid quantization. Also, in Rel-15 the NR UE demodulation requirements in FR2 are defined with maximum CBW of 100MHz.

Proposal #2: Use simplification method of choosing strongest paths contributing to 90% of total power for TDL-A

For TDL-A the re-normalized channel model and the channel model with the delay spread scaling of 30ns and 5ns delay quantization are provided in table below.

Table 1: TDL Channel Model for FR2

| TDL-A-Mod Tap #; Normalized delay; Power in [dB]. 1; 0.624161; 0. 2; 0.657829; -2.2. 4; 0.753439; -6. 5; 0.878467; -8.2. 7; 0.939755; -10.5. 3; 0.959041; -4. 6; 1.096327; -9.9. 8; 1.245053; -7.5. 9; 3.101683; -6.6. 10; 4.105341; -10.8. 11; 4.99819; -11.3. | TDL-A-30 Tap #; Path delay [ns]; Power in [dB]. 1; 20; 0.0. 2; 25; -6.0. 3; 30; -5.2. 4; 35; -7.6. 5; 95; -8.6. 6; 125; -12.8. 7; 150; -13.3. |
| --- | --- |

# Conclusion

In this paper we provide our views on additional channel models and simplified TDL channel models for FR2. Our observations and proposals are summarized below:

Observation #1: TDL-C with 60-80ns are not suitable for modelling FR2 propagation conditions

Observation #2: TDL-D/ LOS channel model doesn’t stress UE performance and hence not suitable to define requirements in FR2

Proposal#1: Define UE demodulation requirements in FR2 with TDL-A-30ns.  TDL-C-[60-80]ns and TDL-D-[10-30]ns shall not be used for defining UE demodulation requirements in FR2

Proposal #2: Use simplification method of choosing strongest paths contributing to 90% of total power for TDL-A

Proposal #3: Define the following channel models for demodulation and CSI requirements in FR2:

| TDL-A-30 Tap #; Path delay [ns]; Power in [dB]. 1; 20; 0.0. 2; 25; -6.0. 3; 30; -5.2. 4; 35; -7.6. 5; 95; -8.6. 6; 125; -12.8. 7; 150; -13.3. | TDL-A-Mod Tap #; Normalized delay; Power in [dB]. 1; 0.624161; 0. 2; 0.657829; -2.2. 4; 0.753439; -6. 5; 0.878467; -8.2. 7; 0.939755; -10.5. 3; 0.959041; -4. 6; 1.096327; -9.9. 8; 1.245053; -7.5. 9; 3.101683; -6.6. 10; 4.105341; -10.8. 11; 4.99819; -11.3. |
| --- | --- |

# References

- R4-1811725, “Way forward on channel model for NR performance requirements”, Huawei, Spirent

- R4-1809859, “Channel Models for FR2 demodulation requirements”, Intel,corporation

- R4-1812177, “Discussion on simplified TDL channel models ”, Intel Corporation