3GPP TSG-RAN WG4#88Bis Meeting	R4-1812180

Chengdu, China, 8 – 12 October 2018

Agenda item:	7.13.1.4

Title: 	Simulation results and discussion on NR CQI reporting under fading conditions

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In RAN4#88 Meeting (August 2018), in [5], we provided our initial results for FR1 FDD and FR2 TDD CQI reporting tests. Also, in RAN4#88 Meeting, a WF [6] was agreed and companies were encouraged to bring CQI simulation results (wideband and sub-band CQI) under fading channel conditions. In this contribution, we present our initial WB and SB CQI results under fading channel conditions for alignment purpose.

# Wideband CQI reporting results in fading channels

In this section, we provide our initial simulation results for FR1 FDD wideband CQI reporting in TDL-A fading channel conditions. Table 1 lists the common parameters for the CQI reporting test case specified in this section.

Table 1 Test parameters

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| SCS | kHz | 15 |
| Transmission Scheme |  | 1 |
| Duplex Mode |  | FDD |
| TDD DL/UL pattern |  | N/A |
| Propagation channel |  | TDL-A, 30ns, 5Hz |
| Correlation and antenna configuration |  | [ULA 2x2 High] |
| CSI-RS resourceConfigType |  | Periodic |
| CSI-RS periodicity/slot offset |  | 5/1 |
| CSI-RS number of ports (nrofPorts) |  | 2 ports {3000,3001} |
| CSI-RS cdm-Type |  | fd-CDM2 |
| CSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [5] |
| CSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000001] |
| CSI-RS density |  | 1 |
| CSI-IM periodicity/slot offset |  | 5/1 |
| CSI-IM-RE-Pattern |  | Pattern0 |
| ZPCSI-RS number of ports (nrofPorts) |  | 4 ports {3000, …, 3003} |
| ZPCSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [5] |
| ZPCSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000100] |
| ZPCSI-RS density |  | 1 |
| ZP CSI-RS reportConfigType |  | Periodic |
| Reporting periodicity | ms | 5 |
| Reporting offset | ms | 4 |
| CQI delay | ms | 8 |
| Reporting granularity |  | Wideband |
| Physical channel for reporting |  | [PUCCH] |
| reportQuantity |  | [cri-RI-CQI] |
| Codebook type |  | Type I single-panel |
| Codebook subset restriction |  | 000001 |
| CQI table |  | Table 5.2.2.1-3 [38.214] |

It was also agreed in [6] to reuse LTE test metric for WB CQI tests under fading channel conditions:

- a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α % of the time;

- the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ;

- when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [TBD].

Table 2 shows our initial WB CQI results with assumptions specified in Table 1 for alignment purpose.

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/random) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 20% | 1.166 | 0.2 |
| 7 | 24% | 1.0657 | 0.2529 |

# Sub-band CQI reporting results in fading channels

In this section, we provide our initial simulation results for FR1 FDD sub-band CQI reporting in fading channel conditions. Table 3 lists the common parameters for the CQI reporting test case specified in this section.

Table 3 Test parameters

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| SCS | kHz | 15 |
| Transmission Scheme |  | 1 |
| Duplex Mode |  | FDD |
| TDD DL/UL pattern |  | N/A |
| Sub-band size | PRB | 8 |
| Propagation channel |  | Clause B.2.4 in TS36.101 |
| Correlation and antenna configuration |  | [ULA 1x2] |
| CSI-RS resourceConfigType |  | Periodic |
| CSI-RS periodicity/slot offset |  | 5/1 |
| CSI-RS number of ports (nrofPorts) |  | 2 ports {3000,3001} |
| CSI-RS cdm-Type |  | fd-CDM2 |
| CSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [5] |
| CSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000001] |
| CSI-RS density |  | 1 |
| CSI-IM periodicity/slot offset |  | 5/1 |
| CSI-IM-RE-Pattern |  | Pattern0 |
| ZPCSI-RS number of ports (nrofPorts) |  | 4 ports {3000, …, 3003} |
| ZPCSI-RS firstOFDMSymbolInTimeDomain (CSI-RS-ResourceMapping, l0) |  | [5] |
| ZPCSI-RS frequencyDomainAllocation (CSI-RS-ResourceMapping, k0) |  | [000100] |
| ZPCSI-RS density |  | 1 |
| ZP CSI-RS reportConfigType |  | Periodic |
| Reporting periodicity | ms | 5 |
| Reporting offset | ms | 4 |
| CQI delay | ms | 8 |
| Reporting granularity |  | Wideband |
| Physical channel for reporting |  | [PUCCH] |
| reportQuantity |  | [cri-RI-CQI] |
| Codebook type |  | Type I single-panel |
| Codebook subset restriction |  | 000001 |
| CQI table |  | Table 5.2.2.1-3 [38.214] |

It was also agreed in [6] to reuse LTE test metric for SB CQI tests under fading channel conditions:

- sub-band differential CQI offset level of 0 shall be reported at least α % of the time but less than β % for each sub-band;

- the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ γ;

- when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to [TBD].

Table 4 shows our initial SB CQI results with assumptions specified in Table 3 for alignment purpose.

| SNR | Probability of 0 CQI offset | Throughput ratio (maxSB/medWB) | BLER for follow max SB CQI |
| --- | --- | --- | --- |
| 11 | 18.32% | 1.6367 | 0.3608 |
| 12 | 18.32% | 1.4909 | 0.3588 |

Proposal 1: Apply 2x2 antenna configuration for WB CQI test cases, and 1x2 antenna configuration for SB CQI test cases.

# Conclusion

In this contribution, we present our initial WB and SB CQI results under fading channel conditions for alignment purpose.

Proposal 1: Apply 2x2 antenna configuration for WB CQI test cases, and 1x2 antenna configuration for SB CQI test cases.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809798, “Initial simulation results and discussion on NR CQI tests”, Intel Corporation, August 2018

[6] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018