3GPP TSG-RAN WG4 Meeting #88bis 	R4-1812192

Chengdu, China, October 8 – 12, 2018

Agenda item:	7.12.1

Source:	Intel Corporation

Title:	On remaining issues for SFTD accuracy requirement

Document for:	Discussion

# Introduction

After RAN4 #88 meeting, the SFTD measurement accuracy for FR1 in EN-DC is approved and captured in the following table

Table 1: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| [40] | [-3] dB | FR1 |
| [TBD] |  | FR2 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211. |  |  |

Some remaining issues on SFTD accuracy requirement are

- SFTD measurement accuracy for FR2 in EN-DC

- SFTD measurement accuracy in NR-NR DC

In this contribution, we propose the SFTD measurement accuracy requirement for above two scenarios.

# Discussion

Since SFTD is an extension of E-UTRA SSTD measurement reporting to allow reporting of time difference between E-UTRA and NR cells, it is reasonable to follow the same mechanism as SSTD accuracy requirement to design the accuracy requirement for SFTD measurement.

## SFTD Accuracy for EN-DC

The SSTD accuracy requirement specified in TS36.133 section 9.1.20 is derived from the requirements on UE accuracy for uplink transmit timing are captured in TS 36.133 section 7.1[1]. The argument is that since the transmit timing is depending on the first detected path of the corresponding radio frame in the downlink, UE derives the transmit timing based on the DL reception timing. Hence the accuracy of the transmit timing is directly dependent on the accuracy of the downlink timing as determined by the UE.

Following the design mechanism of SSTD accuracy requirement, one can express the accuracy requirement for SFTD measure as follows:

SFTD accuracy = TX timing error in E-UTRA PCell + TX timing error in NR PSCell + margin

For the transmit timing error for LTE, it is captured in TS36.133 section 7.1.2, which is copied in Table 1 below

Table 2: Timing Error Limit in LTE

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 1.4 | 24*TS |
| ≥3 | 12*TS |
| Note: TS is the basic timing unit defined in TS 36.211 |  |

It can be seen from Table 2, that the timing error for LTE cell is 24TS.

For NR PSCell, the TX timing error is captured in TS38.133 section 7.1.2, which is shown in Table 3 below.

Table 3: Timing Error Limit in NR

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | [12]*64*Tc |
|  |  | 30 | [10]*64*Tc |
|  |  | 60 | [10]*64*Tc |
|  | 30 | 15 | [8]*64*Tc |
|  |  | 30 | [8]*64*Tc |
|  |  | 60 | [7]*64*Tc |
| 2 | 120 | 60 | [3.5]*64*Tc |
|  |  | 120 | [3.5]*64*Tc |
|  | 240 | 60 | [3]*64*Tc |
|  |  | 120 | [3]*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 Editor’s note: The final values of Te for 120KHz SSB SCS are subject to further discussions in further meeting, and may not be outside 3*64*Tc to 3.5*64*Tc. |  |  |  |

In RAN4 #88 meeting, it is approved to separate the SFTD accuracy requirement into two categories by different frequency ranges, i.e., FR1 and FR2. For FR1, the TX timing error is 12TS in the case of 15 kHz SCS for both DL and UL. Likewise, the worst case for TX timing error in FR2 is 3.5TS.

As for the implementation margin, in SSTD measurement accuracy, it is agreed to be 4TS [2]. For SFTD accuracy in EN-DC, the implementation margin in FR1 should be same as LTE DC, given the fact that NR UE implement should at least no worse than LTE. For FR2, the implementation margin maybe scaled according to TX timing error, i.e., 2TS for TX timing error margin from UE to LTE cell and 1TS for TX timing error margin from UE to NR cell. Thus, we propose 3TS for the implementation margin in FR2.

Thus, it is proposed the SFTD accuracy = 24TS + 3.5TS + 3TS = 30.5TS.

Proposal 1: The SFTD accuracy requirement for EN-DC is 30.5Ts when NR PSCell is operated in FR2.

## SFTD Accuracy for NR-NR DC

Following the same design principle as EN-DC, the SFTD accuracy for NR-NR DC can be expressed as

SFTD accuracy = TX timing error in NR PCell + TX timing error in NR PSCell + margin

It is shown in Table 3, the timing error is 12TS for FR1 and 3.5 TS for FR2. The implementation margin used for NR cell is applied to NR DC case as well. It is assumed that the implementation margin is 4TS for FR1, 2TS for FR2, and 3TS for mixed FR1 and FR2. Thus, we propose that

Proposal 2: The SFTD accuracy requirement for NR-NR DC is

- 28Ts if both two NR cells are operated in FR1;

- 18.5Ts if one NR cell is operated in FR1 and the other NR cell is operated in FR2;

- 9Ts if both two NR cells are operated in FR2.

# Summary and Conclusions

This contribution discusses the remaining issues on SFTD measurement accuracy for EN-DC and NR-NR DC. The conclusions are drawn as follows:

Proposal 1: The SFTD accuracy requirement for EN-DC is 30.5Ts when NR PSCell is operated in FR2.

Proposal 2: The SFTD accuracy requirement for NR-NR DC is

- 28Ts if both two NR cells are operated in FR1;

- 18.5Ts if one NR cell is operated in FR1 and the other NR cell is operated in FR2;

- 9Ts if both two NR cells are operated in FR2.

# References

- R4-156288, “Measurement accuracy of subframe offset reporting for Dual Connectivity”, Ericsson, RAN4 meeting #76bits

- R4-1809879, “On SFTD Accuracy Requirement”, Intel, RAN4 meeting #88