3GPP TSG-RAN4 Meeting #88bis	R4-1812258

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for SA BWP switching interruption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-6 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for SA BWP switching interruption |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases: 1. SA FDD DL BWP switching interruption of SCell in non-DRX in FR1 with 15KHz SCS 2. SA TDD DL BWP switching interruption of SCell in non-DRX in FR1 with 30KHz SCS 3. SA TDD DL BWP switching interruption of SCell in non-DRX in FR2 with 120KHz SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for SA BWP switching interruptions will be missed in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.6.5.2.1.5, A.6.5.2.1.6 and A.7.5.2.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.6.5.2.1.5	SA FDD DL BWP switching interruption of SCell in non-DRX in FR1 with 15KHz SCS

A.6.5.2.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the SA interruptions on SCell due to PCell DL BWP switching are within the requirements stated in section 8.2. This test will verify the missed ACK/NACK rate for NR SCell in SA specified in TS38.133 section 8.2.

The test parameters are given in Tables A.6.5.2.1.5.1-1 and cell-specific parameters in A.6.5.2.1.5.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 2 (SCell) on radio channel 2 (SCC). The UE shall be continuously scheduled in the PCell and SCell throughout the whole test.

After reading RMSI in SA, UE will camp on the initial bandwidth-BWP0. The BWP1 can be provided by RRC configuration defaultDownlinkBWP-1 as the default BWP. At the beginning of T1, the UE receives RRC signalling including firstActiveDownlinkBWP-1 command in time slot #n. During the time period T1, UE shall be able to receive PDSCH on the new BWP1 of the serving cell on which BWP switch occurs no later than at slot # (n+X), X is TBD.

The point in time at which the DCI format 1_1 is received in a slot #m, defines the start of time period T2. The bandwidth part indicator field triggers the change of active DL BWP during the time duration T2. UE shall be able to receive PDSCH on the new BWP2 of the serving cell on which BWP switch occurs no later than at slot # (m+Y), Y is TBD.

Upon the expiry of bwp-InactivityTimer, defines the start of time period T3 in a slat #i. The UE shall be able to switch its bandwidth back to the default bandwidth part – BWP1 and receive PDSCH on the BWP1 of serving cell no later than at slot #(i+Z), Z is TBD.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in SCell during PCell DL BWP switching, respectively.

PDCCHs indicating new transmissions shall be sent continuously on PCell/SCell to ensure that the UE would have ACK/NACK sending during identifying new BWP switching of PCell/PSCell.

Table A.6.5.2.1.5.1-1: General test parameters for SA FDD DL BWP switching interruption of SCell in non-DRX in FR1 with 15KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured activated SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| bwp-InactivityTimer |  | ms5 | The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.331, section 6.3.2) |
| firstActiveDownlinkBWP-Id |  | BWP-Id |  |
| T1 | s | 1 |  |
| T2 | s | 1 |  |
| T3 | s | 1 |  |

Table A.6.5.2.1.1.1-2: Cell specific test parameters for SA FDD DL BWP switching interruption of SCell in non-DRX in FR1 with 15KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | FDD |  |  | FDD |  |  |
| BWchannel | MHz | 10MHz |  |  | 10MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| CORESET parametersNote1: |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.1 FR1 |  |  | SMTC.1 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.6.5.2.1.5.2	Test Requirements

During T1, the UE shall be able to receive PDSCH on the new BWP of the serving cell on which BWP switch occurs no later than at slot # (n+X), X is TBD.

During T2, UE shall be able to receive PDSCH on the new BWP of the serving cell on which BWP switch occurs no later than at slot # (m+Y), Y is TBD.

During T3 the UE shall be able to switch its bandwidth back to the default bandwidth part and receive PDSCH on the BWP1 of serving cell no later than at slot #(i+Z), Z is TBD..

The interruption of SCell shall not be more than the values specified in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

A.6.5.2.1.6	SA TDD DL BWP switching interruption of SCell in non-DRX in FR1 with 30KHz SCS

A.6.5.2.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the SA interruptions on SCell due to PCell DL BWP switching are within the requirements stated in section 8.2. This test will verify the missed ACK/NACK rate for NR SCell in SA specified in TS38.133 section 8.2.

The test parameters are given in Tables A.6.5.2.1.6.1-1 and cell-specific parameters in A.6.5.2.1.6.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 2 on radio channel 2. The UE shall be continuously scheduled in the PCell and SCell throughout the whole test.

After reading RMSI in SA, UE will camp on the initial bandwidth-BWP0. The BWP1 can be provided by RRC configuration defaultDownlinkBWP-1 as the default BWP. At the beginning of T1, the UE receives RRC signalling including firstActiveDownlinkBWP-1 command in time slot #n. During the time period T1, UE shall be able to receive PDSCH on the new BWP1 of the serving cell on which BWP switch occurs no later than at slot # (n+X), X is TBD.

The point in time at which the DCI format 1_1 is received in a slot #m, defines the start of time period T2. The bandwidth part indicator field triggers the change of active DL BWP during the time duration T2. UE shall be able to receive PDSCH on the new BWP2 of the serving cell on which BWP switch occurs no later than at slot # (m+Y), Y is TBD.

Upon the expiry of bwp-InactivityTimer, defines the start of time period T3 in a slat #i. The UE shall be able to switch its bandwidth back to the default bandwidth part – BWP1 and receive PDSCH on the BWP1 of serving cell no later than at slot #(i+Z), Z is TBD.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in SCell during PCell DL BWP switching, respectively.

PDCCHs indicating new transmissions shall be sent continuously on PCell/SCell to ensure that the UE would have ACK/NACK sending during identifying new BWP switching of PCell/PSCell.

Table A.6.5.2.1.6.1-1: General test parameters for SA TDD DL BWP switching interruption of SCell in non-DRX in FR1 with 30KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured activated SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| bwp-InactivityTimer |  | ms5 | The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.331, section 6.3.2) |
| firstActiveDownlinkBWP-Id |  | BWP-Id |  |
| T1 | s | 1 |  |
| T2 | s | 1 |  |
| T3 | s | 1 |  |

Table A.6.5.2.1.6.1-2: Cell specific test parameters for SA TDD DL BWP switching interruption of SCell in non-DRX in FR1 with 30KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | TDD |  |  | TDD |  |  |
| BWchannel | MHz | 10MHz |  |  | 10MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| CORESET parametersNote1: |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.1 FR1 |  |  | SMTC.1 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.6.5.2.1.6.2	Test Requirements

During T1, the UE shall be able to receive PDSCH on the new BWP of the serving cell on which BWP switch occurs no later than at slot # (n+X), X is TBD.

During T2, UE shall be able to receive PDSCH on the new BWP of the serving cell on which BWP switch occurs no later than at slot # (m+Y), Y is TBD.

During T3 the UE shall be able to switch its bandwidth back to the default bandwidth part and receive PDSCH on the BWP1 of serving cell no later than at slot #(i+Z), Z is TBD..

The interruption of SCell shall not be more than the values specified in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

<< End of Change #1>>

<< Start of Change #2>>

A.7.5.2.1.x	SA TDD DL BWP switching interruption of SCell in non-DRX in FR2 with 120KHz SCS

A.7.5.2.1.x.1	Test Purpose and Environment

The purpose of this test is to verify that the SA interruptions on SCell due to PCell DL BWP switching are within the requirements stated in section 8.2. This test will verify the missed ACK/NACK rate for NR SCell in SA specified in TS38.133 section 8.2.

The test parameters are given in Tables A.7.5.2.1.x.1-1 and cell-specific parameters in A.7.5.2.1.x.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 2 on radio channel 2. The UE shall be continuously scheduled in the PCell and SCell throughout the whole test.

After reading RMSI in SA, UE will camp on the initial bandwidth-BWP0. The BWP1 can be provided by RRC configuration defaultDownlinkBWP-1 as the default BWP. At the beginning of T1, the UE receives RRC signalling including firstActiveDownlinkBWP-1 command in time slot #n. During the time period T1, UE shall be able to receive PDSCH on the new BWP1 of the serving cell on which BWP switch occurs no later than at slot # (n+X), X is TBD.

The point in time at which the DCI format 1_1 is received in a slot #m, defines the start of time period T2. The bandwidth part indicator field triggers the change of active DL BWP during the time duration T2. UE shall be able to receive PDSCH on the new BWP2 of the serving cell on which BWP switch occurs no later than at slot # (m+Y), Y is TBD.

Upon the expiry of bwp-InactivityTimer, defines the start of time period T3 in a slat #i. The UE shall be able to switch its bandwidth back to the default bandwidth part – BWP1 and receive PDSCH on the BWP1 of serving cell no later than at slot #(i+Z), Z is TBD.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in SCell during PCell DL BWP switching, respectively.

PDCCHs indicating new transmissions shall be sent continuously on PCell/SCell to ensure that the UE would have ACK/NACK sending during identifying new BWP switching of PCell/PSCell.

Table A.7.5.2.1.x.1-1: General test parameters for SA TDD DL BWP switching interruption of SCell in non-DRX in FR2 with 120KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured activated SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| AoA number |  | 1 | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| bwp-InactivityTimer |  | ms5 | The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.331, section 6.3.2) |
| firstActiveDownlinkBWP-Id |  | BWP-Id |  |
| T1 | s | 1 |  |
| T2 | s | 1 |  |
| T3 | s | 1 |  |

Table A.7.5.2.1.x.1-2: Cell specific test parameters for SA TDD DL BWP switching interruption of SCell in non-DRX in FR2 with 120KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | TDD |  |  | TDD |  |  |
| BWchannel | MHz | 10MHz |  |  | 10MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| CORESET parametersNote1: |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.1 FR1 |  |  | SMTC.1 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.7.5.2.1.x.2	Test Requirements

During T1, the UE shall be able to receive PDSCH on the new BWP of the serving cell on which BWP switch occurs no later than at slot # (n+X), X is TBD.

During T2, UE shall be able to receive PDSCH on the new BWP of the serving cell on which BWP switch occurs no later than at slot # (m+Y), Y is TBD.

During T3 the UE shall be able to switch its bandwidth back to the default bandwidth part and receive PDSCH on the BWP1 of serving cell no later than at slot #(i+Z), Z is TBD..

The interruption of SCell shall not be more than the values specified in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

<< End of Change #2>>