3GPP TSG-RAN WG4 Meeting #88bis	R4-1812284

Chengdu, China, 8th - 12th Oct, 2018

Agenda Item:	9.3.2

Source:	CATT, CMCC

Title:	TP for 37.716-11-11: DC_3A-n41A

Document for:	Approval

- Introduction

This contribution provides a text proposal for inter-band DC_3A-n41A for TR 37.716-11-11.

- Discussion

Interference analysis for DC_3A-n41A is shown in Table 2-1.

Table 2-1: Harmonics and IMD products for DC_3A-n41A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5130 | 5355 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
|  | 6840 | 7140 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
|  | 8550 | 8925 | 12480 | 13450 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| Two-tone 3rd order IMD products | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
|  | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
|  | 1960 | 1422 | 8412 | 8950 |
| Two-tone 4th order IMD products*1 | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 711 | 980 | 4206 | 4475 |
| Two-tone 4th order IMD products*1 | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3420 | 3570 | 4992 | 5380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 10908 | 11640 | 10122 | 10735 |
| Two-tone 5th order IMD products*2 | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5130 | 5355 | 7488 | 8070 |
| Two-tone 5th order IMD products*2 | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 730 | 1074 | 3207 | 3670 |
| Two-tone 5th order IMD products*2 | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 5196 | 6260 | 6702 | 7165 |
| NOTE1: The center frequency of this IMD is the same as that of 2nd harmonics and IMD. It is depending on the proponents of each band combination to include the impact of this IMD in MSD analysis. NOTE2: The center frequency of this IMD is the same as that of 3rd harmonics and IMD. It is depending on the proponents of each band combination to include the impact of this IMD in MSD analysis. |  |  |  |  |

Based on above table, 4th order IMD may fall into Rx frequencies of band 3 and n41. But IMDs will not cause any problem for n41 Rx since n41 is a TDD band. MSD due to IMD4 needs to be considered for Band 3. And according to the experience in LTE CA_3-41, there will be cross band isolation issue for this band combination. It can be expected that the same RF architecture will be used in NR for this combination, corresponding MSD requirements for LTE CA_3-41 shall be used to specify DC_3A-n41A.Table 2-2 and table 2-3 show the required sensitivity degradation due to those two issues based on the section 7.3.1A in TS36.101.

Table 1: MSD due to cross band isolation for CA_3-41

| MSD due to cross band isolation for the DL band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | NR UL band | NR DL band | Channel bandwidth |  |  |  |
|  |  |  | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| CA_3A-41A | 3 | n41 | [4.7] | [4.3] | [4.0] | [3.9] |
|  | n41 | 3 | [3.0] | [3.0] | [3.0] | [3.1] |
| NOTE 1: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. |  |  |  |  |  |  |

Table 2: MSD due to IMD4 for CA_3-41

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_3A-41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 52 | 2657.5 | N/A | TDD | N/A |
| NOTE 1: RBSTART = 0 NOTE 2: 15 kHz SCS is assumed. |  |  |  |  |  |  |  |  |

In LTE 2 different delta values are specified for Band 41 due to different IL between 2496-2545MHz and 2545-2690MHz. According to the data provided in [xx], we propose to specify a single delta value for the frequency range 2515-2690MHz.

A text proposal is given in the annex.

- Conclusion

This paper discussed the requirement for DC_3A-n41A. It is proposed to approve the text proposal on DC_3A-n41A for TR 37.716-11-11 in the annex.

- Annex

----- Start of TP -----

## 6.1	Inter-band EN-DC within FR1

## 6.x.1	DC_3A-n41A

### 6.x.1.1	Operating bands for DC

Table 6.x.1.1-1: Band combinations for EN-DC (two bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_n41 | 3 | n41 | Yes |

### 6.x.1.2	Configuration for DC

Table 6.x.1.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_n41A | DC_3A_n41A | 3A | n41A |

### 6.x.1.3	Maximum output power for DC

Table 6.1.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3A_n41A | 23 | +/-2 |

### 6.x.1.4	Spurious emission band UE co-existence for DC

Table 6.1.1.4-1: Co-existence requirements for DC_3A-n41A

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n41A | E-UTRA Band 1, 5, 8, 20, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 4, 5 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5, 6 |
| NOTE 1: To simplify Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 4: This requirement applies for 5, 10, 15 and 20 MHz NR channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 5: This requirement applies when the NR carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 6: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

### 6.x.1.5	MSD for DC

Reference sensitivity exceptions due to cross band isolation are specified for DC_3A-n41A in Table 6.x.1.5-1 with uplink configuration specified in Table 6.x.1.5-2.

Table 6.x.1.5-1: Reference sensitivity exceptions due to cross band isolation

| EN-DC configuration | UL band | DL band | MSD |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| DC_3A-n41A | 3 | n41 |  | [4.3] | [4.0] | [3.9] |  |  | [3.5] | [3.3] | [3.2] | [3.1] | [3.0] | [3.0] |
|  | n41 | 3 | [3.0] | [3.0] | [3.0] | [3.1] | [3.0] | [3.0] |  |  |  |  |  |  |
| NOTE 1: Band n41 MSDs are modified by 0.5 dB when carrier frequency of the assigned NR channel bandwidth is within 2515-2690 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15kHz | 25 | 52 | 521 | 521 | 521 | 521 |  |  |  |  |  |  |
|  | 30kHz |  | 24 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |
| n41 | 15kHz |  | 52 | 79 | 106 |  |  | 216 | 270 |  |  |  |  |
|  | 30kHz |  | 24 | 38 | 51 |  |  | 106 | 133 | 162 | 217 | 245 | 273 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-3 lists the required MSD due to IMD4 for the dual uplink configuration.

Table 6.x.1.5-3: Reference sensitivity exceptions due to IMD4

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 | Yes |
|  | n41 | 2657.5 | 10 | 52 | 2657.5 | N/A | TDD | IMD4 |  |
| NOTE 1: RBSTART = 0 |  |  |  |  |  |  |  |  |  |

### 6.1.1.6	∆TIB and ∆RIB values

Table 6.1.1.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-n41 | 3 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.1.1.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-n41 | 3 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

----- Start of TP -----